EPC YARD OPERATIONS

Method Statement for

Al Laith Yard

Ref: ALS-MS-SS.YARD OPERATIONS-1000-MC

Name of Project: Water Tank Support Structure

ERECTION, MODIFICATION, & DISMANTLING OF SCAFFOLD STRUCTURES

JOB NUMBER: 1000

# CONTENTS

1.0 TASK DETAILS
 1.1 Customer
 1.2 Project Name
 1.3 Site Address/Location
 1.4 Description of the Task/Activity

2.0 REFERENCES
 2.1 Specification
 2.2 Reference (Drawings)

3.0 SCOPE & OBJECTIVES

4.0 PROGRAM

5.0 KEY RESOURCES
 5.1 Persons / Responsibility
 5.2 Accidents & Incidents
 5.3 CAPA
 5.4 Materials
 5.5 Other Essential Equipment
 5.6 Mandatory PPE
 5.7 Task Specific PPE
 5.8 Plant Equipment
 5.9 Purpose
 5.10 Identified Risks
 5.11 Site Specific Rules & Restrictions

6.0 SEQUENCE OF OPERATION

6.0 WORKING AT HEIGHT

8.0 SUPPORT WORK & STORAGE ARRANGEMENTS
 8.1 Temporary Supports and Props needed to facilitate the works
 8.2 Storage Arrangements
 8.3 Details of Permits to Work
 8.4 SWL’s

9.0 SAFETY REQUIREMENTS
 9.1 Specific Identified Residual Hazards
 9.2 Required Personal Protective Equipment Applicable
 9.3 Hazardous Substances Applicable

10.0 QUALITY MONITORING

11.0 EMERGENCY PROCEDURES
 11.1 Welfare Requirement
 11.2 Services to be supplied by others
 11.3 Other information and comments
 11.4 Emergency procedures

12.0 ACCESS & EGRESS POINTS

13.0 ENVIRONMENTAL IMPACT

14.0 THIRD PARTY INTERACTION

15.0 PERMITS KEY RESOURCE
 15.1 Sketches / Date sheets
 15.2 Certification of Plant/operator competency
 15.3 Program of Works
 15.4 Risk Assessment
 15.5 Excavation permit
 15.6 Daily briefing form
 15.7 Emergency contact details
 15.8 Emergency arrangements
 15.9 Rescue plan
 15.10 Method Statement Amendments and Review

16.0 EMERGENCY CONTACT DETAILS

17.0 REVIEW AND AMENDMENTS
 17.1 Rescue Arrangements

18.0 EMERGENCY PROCEDURES

19.0 METHOD STATEMENT BRIEFING RECORD

| # 1.0 Task Details |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1.2 | Project Name | Water Tank Support Structure |  |  |  |
| 1.3 | Site Address/Location | Al Laith Yard, Dubai Industrial City |  | Date: |  |
|  |  |  |  | Time: |  |
| 1.4 | Description of the Task/Activity | The scope includes Installation of Water Tank Support Structure |  |  |  |
| # 2.0 References |  |  |  |  |  |
| 2.1 | Specification | All scaffold activities shall be planned and executed in accordance with: ### Legal & Regulatory Requirements - UAE Federal Laws - Dubai Municipality & Trakhees EHS Requirements - ADOSH CoP 26 – Construction Safety - OSHAD CoP 49 – Fall Prevention (Work at Height) ### International Standards - ISO 45001:2018 – Occupational Health & Safety Management System - BS EN 12811-1 – Scaffolding Performance Requirements - NASC TG20 – Good Practice Guidance ### Temporary Structure Requirements - Temporary structures shall comply with temporary Structure Procedure Ref: 001-002-014, approved design and drawings - No scaffold shall be: - Erected - Modified - Used Without: - Approved MSRA - Approved drawings/design (where applicable) - Valid Permit to Work (PTW) All structures shall be installed only within designated yard areas ### Work at Height Requirements (OSHAD CoP 49) Where applicable, the Fall Prevention Plan shall include: - Fully decked working platforms - Guardrails and toe boards (collective protection) - Safe access (ladders/stair towers) - Controlled access and supervision - Harness use only as last control measure - Rescue plan and equipment availability ### Traffic & Yard Safety Requirements - Segregated pedestrian and vehicle routes - Defined speed limits within yard - Banksman control for reversing and lifting operations - Designated loading/unloading zones - No lifting over personnel ### Environmental Requirements - Environmental Waste Management Procedure - Waste Disposal Procedure Ref: 001-002-013 - UAE Environmental Regulations - UAE Environmental Regulations All waste shall be: - Segregated - Stored safely - Disposed through approved channels ### Document Control The following shall be available at the work location: - Approved MSRA - Permits to Work (PTW) - Approved drawings (ALS-FAB-0009-00-REV00) - Scaffold inspection and tagging records - Training and competency records - Equipment certificates ### Management of Change (MoC) Work shall STOP immediately if any of the following occur: - Change in scaffold design, layout, or sequence - Change in work location or yard conditions - Adverse weather (high wind, rain, heat) - Structural instability or ground condition concerns - Incident or near miss - Change in manpower or competency Work shall only restart after: - MSRA review and update - Required approvals - Workforce re-briefing (Toolbox Talk) - Attendance recorded |  |  |  |
| 2.2 | Reference (Drawings) | ALS-FAB-0095-01-REV01, ALS-FA-6020 – Scaffold Structure Calculations, |  |  |  |
| 2.3 | This Method Statement shall be read in conjunction with the approved Risk Assessment (RA Ref: 1000 - Water Tank Support Structure ). - Control measures defined in RA shall be implemented through Section 6.0 (Sequence of Work). - Any deviation or unsafe condition shall result in STOP WORK and review of MSRA. |  |  |  |  |
| # 3.0 Scope & Objectives |  |  |  |  |  |
| This Method Statement applies to Water Tank Support Structure within Al Laith Yard (DIC). These structures shall be executed strictly in accordance with the approved drawings, design calculations, specifications, and applicable standards. ### Scope of Work The scope includes: - Handling, delivery, and controlled storage of scaffold materials - Erection, modification, and dismantling of heavy-duty scaffold structures - Installation of primary structural members, beams, bracing systems, and load-bearing components - Installation of working platforms, access systems, and edge protection - Controlled application and management of loads on scaffold structures - Inspection, tagging (Scafftag), and formal handover - Work at height activities associated with scaffold erection and use - Housekeeping and waste management |  |  |  |  |  |
| # 4.0 Program |  |  |  |  |  |
| Scaffold activities shall be planned on a daily and weekly basis Supervisors shall conduct: - Daily task planning - Toolbox Talks (TBT) - Work allocation based on competency Work shall consider: - Yard layout and congestion - Traffic movement - Weather conditions (wind / heat) - Interface between activities (lifting, fabrication, scaffolding) Safe sequencing shall be maintained to avoid: - Overlapping hazards - Congestion - Conflicting operations |  |  |  |  |  |

5.0 KEY RESOURCES

## 5.1 Roles, Responsibilities and Authority

Roles and responsibilities shall follow the Al Laith organisational structure and this Method Statement. All personnel shall comply with yard rules, company procedures, and the approved MS/RA.

### Project / Yard Manager

The Project/Yard Manager is responsible for overall implementation of this Method Statement, including:

- Ensure adequate resources (manpower, plant, equipment, and materials) are available

- Ensure scaffold activities are planned and executed as per approved MSRA and drawings

- Ensure coordination of scaffold works with yard operations and traffic

- Ensure Management of Change (MoC) is implemented when required

- Ensure compliance with ISO 45001, ADOSH CoP 26, and Dubai Municipality requirements

- Ensure incidents, near misses, and non-conformances are reported and investigated

### Site Supervisor

The Supervisor is responsible for day-to-day scaffold operations:

- Execute works safely as per MSRA and approved drawings

- Allocate tasks and verify worker competency

- Ensure work area segregation, barriers, and signage

- Conduct daily Toolbox Talks (TBT) and briefings

- Monitor weather conditions (wind / heat) and stop work if unsafe

- Ensure scaffold stability at all stages

- Coordinate inspections, hold points, and tagging

- Stop work immediately if unsafe conditions are identified

### Scaffold Foreman / Competent Scaffold Inspector

Responsible for all scaffold-related activities:

- Ensure erection, modification, and dismantling as per approved drawings and method statement

- Maintain scaffold stability throughout all stages of work

- Conduct inspections at key stages (erection, modification, completion)

- Implement and maintain Scafftag system

- Maintain scaffold register and inspection records

- Ensure correct duty class and SWL signage is displayed

- Stop work if any unsafe scaffold condition is identified

### Plant Operators (Forklift / Telehandler)

- Operate plant only if trained, certified, and authorised

- Conduct pre-use inspections and report defects

- Follow designated yard routes and speed limits

- Ensure safe handling and movement of scaffold materials

- Operate only under banksman control where required

- Do not operate equipment under unsafe conditions

### Banksman / Signalman

Responsible for safe movement of plant and materials:

- Control vehicle movement and reversing operations

- Guide forklift/telehandler during loading and unloading

- Ensure exclusion zones are maintained

- Prevent personnel from entering hazardous areas

- Maintain clear communication with operators

### HSE Manager / HSE Officers

Responsible for safety compliance and monitoring:

- Ensure implementation of control measures as per Risk Assessment

- Verify permits (PTW and WAH where applicable)

- Conduct inspections and audits

- Ensure worker competency and training compliance

- Monitor work at height activities in line with OSHAD CoP 49

- Support incident reporting, investigation, and corrective actions

- Ensure emergency preparedness and rescue arrangements are in place

### Scaffolders / General Workers

- Carry out scaffold erection, modification, and dismantling as instructed

- Follow supervisor instructions and MSRA requirements

- Use PPE as required

- Report unsafe conditions immediately

- Do not perform tasks without proper training

- Maintain good housekeeping at all times

## 5.2 Selection of People

Al Laith shall ensure that all personnel are:

- Competent, trained, Medically fit for work and experienced for assigned scaffold tasks

Only certified personnel shall perform:

- Scaffolding works

- Work at Height (WAH)

- Forklift / Telehandler operation

- Banksman duties

All competency certificates shall be:

- Valid and verified

- Recorded in the Training Matrix

## 5.3 Worker Participation and Consultation

Worker participation shall be encouraged to improve safety:

- Daily Toolbox Talks (TBT) shall include: Task hazards, Control measures, Worker feedback

- Feedback shall be: Recorded in Daily Briefing Forms, Reviewed by Supervisor and HSE

Where required:

- MSRA shall be updated (Management of Change)

- Workforce shall be re-briefed

Any safety concern raised shall be:

- Addressed immediately

- Escalated where necessary

| # 5.4 Operational Controls |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.4.1 | Controls |  |  | Control measures shall be selected and implemented in accordance with ISO 45001 (Clause 8.1 – Operational Planning and Control) using the hierarchy of controls: Elimination → Substitution → Engineering Controls → Administrative Controls → PPE (last line of defence) - Preference shall always be given to eliminating hazards where reasonably practicable - Where elimination is not possible, risks shall be reduced through engineered and administrative controls before relying on PPE - Controls shall be monitored for effectiveness and reviewed through Management of Change (MoC) # Competence - The Project Manager / Supervisor shall verify that all personnel are trained, competent, and medically fit before task allocation - Valid and in-date competency certificates/licences shall be verified for: - Scaffolders - Forklift / telehandler / crane operators - Records shall be maintained in the Training Matrix / Training Register ### Refresher Training Refresher training shall be conducted at least every 3 years for: - Manual Handling - Work at Height (WAH) - Hand and Power Tools - Lifting and rigging awareness - Fire safety and emergency response Personnel without valid competence shall not be assigned to the task |  |  |  |  |  |  |  |  |  |  |
| 5.4.2 | Accidents & Incidents |  |  | All incidents, near misses, unsafe acts/conditions, and property damage shall be: - Reported immediately to Supervisor and HSE - Made safe without delay Initial notification shall be done immediately Formal reporting shall be completed within 24 hours (or as per requirement) Where applicable, notification to authority shall be made |  |  |  |  |  |  |  |  |  |  |
| 5.4.3 | CAPA |  |  | The Project Manager, Supervisor, and HSE team are responsible for: Assigning actions, Tracking progress, Ensuring closure. All findings from: Inspections, Audits, Toolbox talks, Incidents / near misses Shall be recorded in the CAR / PAR Register within 24 hours ### Each record shall include: ID / reference, Source of finding, Description, Risk level, Root cause / contributing factors, Corrective action, Responsible person, Target date, Status and closure evidence, Weekly review shall be conducted, Overdue actions shall be escalated |  |  |  |  |  |  |  |  |  |  |
| 5.4.4 | Materials |  |  | All materials used for scaffold works shall be: - Suitable for the intended scaffold application - Compatible with the selected scaffold system - Inspected prior to delivery, during use, and before installation ### General Requirements - All materials shall be inspected on delivery and prior to use - Damaged, defective, or non-compliant materials shall be: - Segregated immediately - Tagged “NOT FOR USE” - Removed from service - Only approved and compliant scaffold materials shall be used ### Scaffold Materials #### Tubes / Steel Components: Scaffold grade material compliant with relevant standards, Free from: Dents, Cracks, Corrosion, Deformation, Unauthorised welding or repairs #### Fittings / Couplers: Free from cracks, distortion, or excessive wear, Threads in good working condition, All moving parts functional and properly secured #### Timber Scaffold Boards: Free from: (Splits, Cracks, Rot, Warping, Contamination), Compliant with approved standards (e.g. BS 2482), No unauthorised cutting or modification #### System Scaffold Components (e.g. Cuplock / Ringlock): Manufacturer-compliant components only, Free from deformation, cracks, or corrosion, Locking mechanisms fully functional and properly engaged, No mixing of incompatible systems unless approved #### Beams / Ladder Beams: Free from cracks, distortion, or structural damage, Installed only as per approved design and method ### Storage and Handling of Materials: Materials shall be stored in designated laydown areas, Stacked in a stable and secure manner to prevent collapse or movement, Segregated by type and size, Loose fittings stored in bins or stillages, Materials shall not obstruct access or emergency routes Any defective material identified during erection or use shall result in: - Immediate STOP of affected work - Removal and replacement of the material - Re-inspection before continuation |  |  |  |  |  |  |  |  |  |  |
| 5.4.5 | Other Essential Equipment: |  |  | Spanners, ratchets, levels, hammers, podger tools as required; shackles and lifting slings where applicable. Tools used at height shall be secured where practicable to reduce dropped-object risk. |  |  |  |  |  |  |  |  |  |  |
| 5.4.6 | Plant, Equipment |  |  | Only authorised and competent operators shall operate plant. All operators shall comply with Al Laith Alcohol & Drug Policy ### General Controls - No operation under: Fatigue, Illness, Influence of drugs/alcohol ### Traffic & Driving Controls - Only licensed and authorised drivers permitted - No mobile phone use while driving - Follow yard speed limits and traffic routes - Pedestrian and vehicle segregation shall be maintained ### Loading / Unloading - Controlled by trained personnel - Exclusion zones established - No personnel allowed in line of fire ### Plant & Lifting Equipment - Valid certification required - Daily pre-use inspections shall be recorded - Equipment shall not be used if defective - SWL shall not be exceeded |  |  |  |  |  |  |  |  |  |  |
| 5.4.7 | Identified Risks |  |  | Key yard risks include: Work at height, Falling objects, Scaffold instability or collapse, Manual handling injuries, Slips, trips, and falls, Vehicle and plant movement, Lifting operations, Fabrication hazards (hot work, sparks, fumes), Fire hazards, Heat stress and weather conditions, Poor housekeeping, Untrained personnel. All risks are controlled through this Method Statement and the approved Risk Assessment. |  |  |  |  |  |  |  |  |  |  |
| 5.4.8 | Yard Rules & Restrictions |  |  | - PPE shall be worn at all times - Only authorised personnel allowed in work areas - No unauthorised modification of structures or equipment - Speed limits shall be followed - Smoking only in designated areas - No work without permit where required - No access under suspended loads |  |  |  |  |  |  |  |  |  |  |
| 5.4.9 | Housekeeping Inspections |  |  | Housekeeping shall be maintained at all times. Requirements: Work areas shall be kept clean and organised, Debris shall be removed regularly, Materials shall be stacked safely, Access/egress routes shall be kept clear ### Inspections: Housekeeping inspections shall be conducted each shift, Recorded in Daily Housekeeping Log, Supervisor responsible for closure of actions, Housekeeping shall be rechecked after; Material movement, Work area changes, Weather interruptions |  |  |  |  |  |  |  |  |  |  |

## 6.0 SEQUENCE OF OPERATION

### This Method Statement defines the sequence of operations for the erection, modification, inspection, loading, and dismantling of Water Tank Support Structure within Al Laith Yard (DIC).

### These structures are engineered load-bearing systems and shall be executed strictly in accordance with approved drawings, design calculations, and specifications.

### Permits are shift-based unless stated otherwise on the PTW. If a permit expires mid-task, work shall STOP and be revalidated before continuation.

### If any change occurs in load, geometry, sequence, materials, or ground conditions → STOP WORK, apply MoC, update MSRA, and re-brief before resuming.

Control measures defined in this section follow the hierarchy of controls (elimination, substitution, engineering controls, administrative controls, and PPE) in accordance with ISO 45001:2018.

## 6.1 REQUIRED PERMITS, EXCLUSION ZONES, INTERFACES & WEATHER CONTROLS

### 6.1.1 Required Permit

### Permit: A General Work Permit (PTW) shall be obtained as per Client / Main Contractor procedure before work starts. Where additional permits are required, they shall be identified and obtained before starting the relevant activity. For this MSRA, additional permits may include lifting for material handling and Work at Height. All permits are controlled through the Client PTW system. The Supervisor is the designated PTW holder and responsible for permit validation, display, suspension, and handback.

Work at Height activities shall only proceed under a valid Work at Height Permit and after verification of fall protection controls.

### 6.1.2 Display and Control

### The live PTW shall be displayed at the workface or access point. The Supervisor shall keep a copy in the Supervisor pack.

### 6.1.3 Permit Change / Suspension (HOLD POINT)

### If the permitted conditions change, including scope, location, method, weather, interfaces, or supervision, the task shall STOP, the PTW shall be suspended or returned as required, and works shall restart only after re-authorisation and re-brief.

### 6.1.4 Exclusion Zones, Segregation and Protection

### Exclusion Zones:Physical barriers and warning signage shall be installed below and around structures during erection, modification, and dismantling. Exclusion zones shall cover lifting areas, working at height zones, and areas where falling objects or structural instability risks may occur. The extent of the exclusion zone shall be defined based on structure height, footprint, and activity.

Toe boards, debris netting, or other suitable measures shall be installed where there is a risk of falling objects. Tools shall be secured using tool lanyards where applicable.

### 6.1.5 Coordination and Interface Management

### Before work starts each shift, the Supervisor shall confirm:

### access and delivery routes;

### pedestrian and vehicle segregation and any marshalled crossings;

### interfaces with material handling operations and nearby site activities;

### emergency arrangements and assembly point access;

### that the work area is suitable and ready, including space, bearing, clearances, and any wind-sensitive shade structure areas.

### Any interface conflict or unsafe overlap shall result in STOP WORK and re-planning.

### 6.1.6 Weather Controls (External Works)

### Monitoring: Weather conditions shall be checked before starting and monitored during the shift using a handheld anemometer, or site system where provided.

### Wind (HOLD POINT): Stop external works when sustained wind speed reaches 9.72 m/s (approximately 35 km/h) or exceeds the design limit of the structure, manufacturer recommendations, or project-specific requirements, whichever is lower. This is particularly critical for stair components, ramp components, loose boards, shade coverings, wire rope bracing, and lightweight roof or cover elements. Resume only when wind remains below the applicable limit and the Supervisor confirms conditions are safe.

### Rain: Stop during heavy rainfall or when slip risk becomes unacceptable. Reassess footing, visibility, structure stability, and access before restarting.

### Lightning: Stop work immediately and seek shelter. Resume only after the all-clear and re-brief where conditions have changed.

### Rain: Stop during heavy rainfall or when slip risk becomes unacceptable. Reassess footing, visibility, structure stability, and access before restarting.

### Lightning: Stop work immediately and seek shelter. Resume only after the all-clear and re-brief where conditions have changed.

### 6.1.7 Wind Log (Evidence and Governance)

### Wind readings shall be recorded hourly as a minimum, and more frequently during gusting, storms, or adverse forecasts. The Wind Log shall be kept in the Supervisor pack and available at the workface. Wind conditions and any holds or restarts shall be confirmed during the start-of-shift TBT and at permit hand-back or end of shift.

6.1.8 Ground Bearing Pressure Verification

Before erection of any temporary structure, the ground condition shall be assessed to ensure adequate bearing capacity. Where required, sole boards, base plates, or load distribution measures shall be provided. Soft ground, backfilled areas, or uneven surfaces shall be stabilised before work proceeds. If ground conditions are uncertain, work shall STOP and be reviewed by the Site Engineer.

## 6.2 PRE-JOB PLANNING

### Competence and records: Only trained and experienced personnel shall be used. The Supervisor verifies valid cards and licences for scaffolders, forklift / telehandler operators, and banksman, and updates the training matrix.

### Briefing and documents: All personnel complete site induction and daily TBT covering MS/RA, drawings, layout, sequence, interfaces, emergency arrangements, stop-work rules, and wind-related controls. Latest approved MS/RA and drawings shall be available at the workface. Any changes shall be managed in accordance with the project MoC process defined in Section 6.0.

### Registers and inspections: Supervisor maintains Scafftag register, scaffold inspection records, plant and tool inspections, material inspection records, and briefing attendance logs.

### Risk Assessment suitability: PM / QHSE confirms the RA is suitable and sufficient. Where conditions change, the RA is updated and re-briefed before work continues.

### Segregation and protection: Plan and install exclusion zones, barriers, pedestrian routes, warning signage, and controlled crossings. Coordinate deliveries and access with site security and logistics.

### Materials control: Inspect materials for correct type, grade, and condition. Non-conforming items shall be tagged NOT FOR USE, quarantined, and reported.

### Traffic interface: Confirm pedestrian and vehicle segregation and appoint a named banksman. Agree radio channel where applicable.

### Daily checks: Complete Daily Residual Risk Checklist at start of shift and after significant changes such as weather, handling method, layout, access arrangement, or canopy / cover installation. Daily housekeeping inspection shall be completed at active workfaces.

### Start Criteria – HOLD POINT: Work may start only when personnel are competent and briefed, documents are current, registers are in order, segregation is in place, and materials have passed receipt checks.

## 6.3 DRIVING TO SITE, ON-SITE VEHICLE MOVEMENT & DELIVERIES

### Drivers: UAE-licensed, medically fit, authorised drivers only. Daily Driving-for-Work and vehicle pre-use checks shall be recorded.

### Access and routing: Follow the site Traffic Management Plan, including one-way routes where set, speed limits, pedestrian walkways and crossings, lighting, mirrors, and segregation controls.

### Banksman: A banksman is mandatory for reversing, tight manoeuvres, congested areas, or poor ground / limited visibility.

### Ground conditions: Reduce speed on soft, sandy, or uneven roads. STOP if the route becomes unsafe, including standing water, washouts, deep ruts, or unstable shoulders. Escalate to Supervisor for alternative route or controls.

### Deliveries and laydown: All delivery vehicles report to security or logistics. Offload only in the designated laydown or drop zone agreed with the Client.

### Public interface: If third parties are present, increase segregation using barriers and marshals until materials are fully controlled in the laydown area.

### Environmental controls: No vehicle maintenance outside approved areas. Use drip trays for minor leaks. Spill kits shall be available at laydown areas.

## 6.4 MATERIALS HANDLING & OFFLOADING (FORKLIFT / TELEHANDLER)

### Competence: Certified operators and banksman only. Daily pre-use checks for machines and attachments; defects mean do not use.

### Set-up: Operate on firm, level ground. Chock trailers where required. Confirm SWL and centre of gravity.

### Exclusion zone: Barriers and signage shall be installed. A named Exclusion-Zone Controller maintains zone integrity during offloading.

### Operation: Controlled movements only; loads kept low while travelling; no lifting over people. Tag lines may be used for tall or awkward loads. No persons on trailer beds during lifting.

### Weather – HOLD POINT: Apply Section 6.1.6 wind, rain, and lightning rules. Stop lifting or offloading when wind is 9.72 m/s or more, or where handling becomes unsafe.

### After use: Park safely, lower attachments, secure keys, clear debris, and log defects or near misses.

## 6.6 MANUAL HANDLING

### Objective: Prevent strains, crush injuries, and loss of control of loads.

### Sequence / Controls: Plan the task by assessing weight, shape, grip points, route, landing area, trip hazards, pinch points, and communication method. Use mechanical aids first, including forklift, telehandler, pallet truck, dolly, or skates.

### If manual handling is required, use team lifts for awkward, long, or heavy loads. Keep the load close to the body, maintain neutral back posture, avoid twisting, turn with feet, and keep movements controlled. Push rather than pull where practicable and safe.

### STOP and re-plan if the load is unstable, too heavy, awkward, the route is not clear, or control is lost.

### Supervisor verification: Confirm controls and record on the Daily Residual Risk Checklist.

### Records: Daily Residual Risk Checklist and toolbox talk attendance.

## 6.7 POWER AND HAND TOOLS

### Objective: Prevent cuts, abrasions, flying particles, electrocution, noise or dust exposure, and fire.

### Sequence / Controls: Competent users only; tools issued and returned via tool register; pre-use inspection; tagging as required.

### Electrical controls: Use 110V CTE or battery tools. 230V only via transformer and RCD, with Supervisor approval and suitable conditions.

### Safe use: Guards and safety devices shall remain fitted and functional. Control dust and noise where required. Keep the work area tidy and manage leads to prevent trip hazards.

### Defects / faults: STOP, isolate, tag “Do Not Use”, and remove from service.

### Records: Tool register, inspection/tag log, and PAT/inspection records if applicable.

## 6.9 SCAFFOLD, ACCESS & SHADE STRUCTURES – MATERIALS, SAFETY, QUALITY & SEQUENCES

### 6.9.1 Scaffolding Materials, Procedures, Safety, and Quality Controls

### A. Foundation and bearing surface
 Scaffolds, access structures, buggy store supports, stair towers, ramp supports, and shade structures shall be erected only on firm, stable, and level ground, or on existing structures verified as capable of supporting imposed loads. Foundation shall remain safe for the full life of the structure. Any uncertainty regarding ground or structure capacity shall result in STOP WORK and review by a competent person or engineer.
 Hold Point: Foundation and bearing suitability verified and recorded before erection continues above base lift.

### B. Sole boards, base plates and base jacks
 Each standard shall be founded on a base plate and sole board as required by surface condition. Base plates shall be positioned centrally on sole boards. Sole board size and thickness shall suit ground bearing and design requirements.

### C. Standards
 Standards shall be plumb and vertical, erected at correct bay spacing for the scaffold type or design. Joints in standards shall be staggered and connected using suitable joining methods.

### D. Lift heights
 Lift heights shall suit scaffold type, access use, and work requirements and comply with design, manufacturer guidance, and applicable standards.

### E. Ledgers
 Ledgers shall be fixed horizontally to standards using load-bearing right-angle couplers and shall be level. Ledger joints shall use sleeve couplers or approved joint pins, staggered correctly.

### F. Transoms
 Transoms shall support platforms and prevent excessive board deflection under load. Support boards at ends and provide intermediate transoms as required by board span and duty rating.

### G. Decking
 Platforms shall be fully decked using sound boards, clean and free from defects. Boards shall be retained against uplift or movement where required. Gaps shall be minimised and controlled.

### H. Platform width and loading
 Scaffold duty or load class shall be defined and communicated. Minimum general platform duty shall follow design. Access-only platforms shall meet the required minimum width. Working platforms with tools or materials shall be wider as required by task and design.
 Hold Point: SWL and duty class displayed on scaffold tag and briefed to users.

### I. Toe boards
 Toe boards shall be installed on working platforms where there is risk of tools or materials falling, or where required by design or PTW/site rules.

### J. Guardrails / edge protection
 Guardrails shall be installed wherever there is a risk of falling. Top guardrail height, intermediate protection, and gaps shall comply with approved standards and design.

### K. Bracing
 Bracing shall be installed as required for stability. Ledger, façade, and plan bracing shall follow design and manufacturer requirements.

### L. System scaffolds (Ringlock / Cuplock)
 Proprietary system scaffolds shall be erected, altered, and dismantled to manufacturer instructions and any project design.

### M. Ties and stabilisation
 Façade, access, and shade-related scaffold supports shall be tied or stabilised to prevent overturning, sliding, or collapse. Tie frequency and layout shall follow design or guidance.
 Hold Point: No handover until tie pattern and stability checks are completed and signed off.

### N. Beams and special members
 Scaffold beams and special arrangements shall be installed as per design, including any knee bracing or special support framing.

### O. Access and egress (stairs and ladders)
 Stair towers shall be preferred where practicable. Where ladders are used, they shall be secured on stable ground, correctly angled, and extend above landing. Staircases, ramps, and access walks shall follow the issued drawings, provide safe landings, and include required handrails, mid-rails, toe boards, or kerbs. Tube-and-fitting staircases over 1.5 m in height shall be designed. The Staircase drawing and access arrangement for Buggy Store and Structure #5 shall be followed exactly.

### P. Material handling and storage
 Materials shall be unloaded, moved, and stored in a controlled area. Stillages and fitting bins shall be used for safe storage.

### Q. Ancillary equipment
 Rope bags, lifting bags, and accessories shall be visually inspected pre-use and fit for purpose.

### R. Safe system of work (WAH controls)
 Scaffolders shall follow safe systems including continuous attachment where required, minimum platform requirements, edge protection, and controlled erection and dismantle methods.

### S. Health, Safety and Environmental Compliance
 Company QHSE system and project requirements shall apply, including compliance with applicable laws, regulations, and client standards.

### T. Quality controls and inspection
 Works shall comply with project quality and inspection requirements. After any incident, near miss, or event that could affect integrity, STOP WORK, inspect, and release only by competent person.

### 6.9.2 Structure erection, installation, alteration & dismantling – general requirements

### Before the start of erection, installation, alteration, or dismantling of any structure, the Project Manager or Supervisor shall conduct a planning brief covering the approved erection sequence as per drawings, key hazards, exclusion zones, wind and weather limits, interfaces, communication arrangements, emergency procedures, and STOP / HOLD points.

### Task-specific toolbox talks shall be completed before work starts.

### Design and compliance: All structures under this MSRA shall strictly follow approved drawings (Ref: 2.1), design calculations, manufacturer recommendations, and this Method Statement. No unauthorised modification or change of sequence is permitted. No unauthorised removal of bracing, restraints, locking devices, or stability measures is permitted.

### Access control, tagging and inspection: Incomplete or unsafe structures shall be blocked and RED TAGGED – DO NOT USE. Supervisor inspections shall be completed at base stage, intermediate erection stages, final completion, and after adverse weather, impact, or alteration. No structure shall be used until formal inspection is completed, GREEN TAG / Scafftag is issued, and handover is confirmed.

### Work at height: Collective protection shall be prioritised at all times in accordance with OSHAD CoP 49. Full decking and guardrails (top rail, mid rail, and toe board) shall be installed before access is permitted. Harness shall only be used where required and must be attached to certified anchor points or lifelines. Rescue arrangements shall be briefed and available at all times.

### Management of Change (MoC): Any changes shall be managed in accordance with the project MoC process defined in Section 6.0.

### Wind / weather: Apply Section 6.1.6. Work shall be stopped during unsafe wind conditions, especially during erection activities and handling of plywood decking.

### Waste Management: Waste management shall be carried out in accordance with Section 13.0.

### 6.9.3 Erection Procedure (Water Tank Support Structure)

### Pre-start: Approved drawings and calculation  (ALS-FA-6020 and ALS-FAB-0095-01-REV01) shall be verified and available at site. Ground bearing capacity shall be confirmed as adequate for imposed loads. All materials (tubes, beams, base plates, plywood, fittings) shall be inspected and confirmed free from damage, corrosion, or deformation. Only approved materials shall be used.

### Setting out: The platform footprint, tower positions, and support locations shall be marked strictly as per approved drawings. No deviation in spacing or layout is permitted without engineering approval.

### Base installation: Sole boards and base plates shall be installed on firm, level ground ensuring full load distribution. Base levels shall be checked and adjusted to prevent differential settlement. Any soft or uneven ground shall be rectified before proceeding.

### Tower erection: Vertical members (standards) shall be erected in sequence as per design. Bracing (longitudinal, transverse, and plan bracing where required) shall be installed immediately during erection to maintain stability. No incomplete, unbraced, or free-standing frames shall be left unattended at any time.

### Beam installation: Primary and secondary RHS/SHS beams shall be installed exactly as per approved design. All members shall be properly seated, aligned, and securely connected. No substitution, relocation, or omission of members is allowed without design approval. Load transfer points shall be checked to ensure correct bearing.

### Deck installation: Approved 20 mm plywood sheets shall be installed to achieve full and uniform coverage. Decking shall be securely fixed to prevent movement, uplift, or displacement. Gaps, uneven surfaces, or loose boards are not permitted. Access to partially completed or partially decked platforms is strictly prohibited.

### Edge protection (Work at Height – OSHAD CoP 49): Guardrails (top rail and mid rail) and toe boards shall be installed immediately after decking of each section. No work at height shall proceed without full edge protection in place. Where temporary exposure exists, fall prevention measures (such as harness with lifeline) shall be implemented.

### Access provision: Safe access shall be provided using secured ladders or approved access systems. Ladders shall be tied, extend above landing level, and positioned at correct angle. Access systems shall be inspected prior to use.

### Inspection and tagging: A competent person shall carry out a full inspection upon completion. The structure shall only be released for use after issuance of a valid Green Scafftag. The structure shall be re-inspected after any modification, adverse weather, or load impact.

### Load control (Critical – as per calculations): The structure shall only be loaded in accordance with the approved calculation pack. No overloading, point loading, or material stacking is permitted unless specifically included in the design. Load shall be uniformly distributed as per design intent.

### Stop work criteria: Work shall be stopped immediately if any instability, settlement, misalignment, loose connections, damaged members, or deviation from design is observed. The structure shall be made safe and re-assessed by a competent person.

### 6.9.4 Structure Dismantling

### Dismantling shall be carried out by competent personnel in the reverse order of erection using an approved safe system of work.

### MoC applies: Any changes shall be managed in accordance with the project Management of Change process defined in Section 6.0.

### Before dismantling: All materials and debris shall be removed. The Green Scafftag shall be removed and replaced with a Red “DO NOT USE” tag at access points. A pre-dismantling inspection shall be carried out to confirm stability. PTW requirements shall be completed where applicable. Exclusion zones shall be established and maintained.

### During dismantling: Safe working platforms and edge protection shall be maintained as far as reasonably practicable. Materials shall be lowered in a controlled manner; no dropping or throwing is permitted. Components shall be secured, and materials stacked neatly in designated areas.

### If paused: Incomplete structures shall remain Red tagged and access shall be physically prevented.

### Emergency: STOP WORK immediately, make the area safe, notify site supervision, implement the site emergency and rescue plan, provide first aid, and do not resume work until inspection and formal approval by a competent person.

## 7.0 WORK AT HEIGHT

### All work at height activities shall be planned and executed in accordance with OSHAD CoP 49 and project requirements. Collective protection shall be prioritised over personal protection. Safe access shall be maintained at all times. Harness systems shall only be used where required by the task and only with suitable anchor arrangements. Rescue arrangements shall be confirmed before work starts, especially where work involves stair installation, external edges, or shade structure erection in exposed areas.

All work at height shall comply with the project Fall Protection Plan.

- Full body harness shall be worn above 1.8m

- Double lanyard with shock absorber

- 100% tie-off at all times

- Certified anchor points shall be used

- Guardrails and edge protection shall be installed wherever possible

| # 8.0 Support Work & Storage Arrangement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.1 | Temporary Supports and Props needed to facilitate the works |  |  | Details of any support work to be added as per the individual job requirements |  |  |  |  |  |  |  |  |  |  |
| 8.2 | Storage Arrangements |  |  | Materials shall be stored in designated zones within the yard to prevent instability and obstruction of vehicle routes. Storage requirements include: - scaffold materials stacked safely - hazardous materials stored in approved containers - segregation between pedestrian routes and storage areas |  |  |  |  |  |  |  |  |  |  |
| 8.3 | Permit to Work |  |  | PTWs are issued by the HSE/Authoriser and accepted by the Al Laith Supervisor. Validity and any extensions are stated on the permit. Signed PTWs are displayed at the workface, and originals are filed in the PTW Register. |  |  |  |  |  |  |  |  |  |  |
| 8.4 | SWL’s |  |  | All structures and equipment shall operate within defined Safe Working Load (SWL) limits. |  |  |  |  |  |  |  |  |  |  |

| Permit to Work (PTW) Workflow & Control Sheet |  |  |  |
| --- | --- | --- | --- |
| Step | Activity | Responsible | Evidence / Record |
| 1. Request | Supervisor/PM submits PTW request with MS/RA, latest drawings, and area plan. Confirms required associated permits (e.g., lifting, MEWP/WAH, hot work, electrical isolation, work near/over water) and identifies interfaces with other trades. | Supervisor / PM | PTW request pack (MS/RA, drawings, area plan); interface checklist (if used) |
| 2. Review & Issue | Authorised Issuer reviews controls, area readiness, and interfaces; issues PTW with conditions, validity period, and any hold points. | Authorised Person (Issuer) | Issued PTW with conditions & validity |
| 3. Acceptance | Al Laith Supervisor accepts PTW, briefs crew (TBT), confirms competence, and posts the live PTW at the workface/PTW board. | Al Laith Supervisor | Signed/accepted PTW; TBT record; PTW displayed |
| 4. Display & Monitor | Supervisor monitors compliance with PTW conditions and MS/RA, maintains exclusion zones and records required checks (e.g., wind log, inspections, housekeeping). | Al Laith Supervisor | Daily PTW check / Supervisor log; wind log; inspection records |
| 5. Stop / Suspend (if conditions change) | If scope / method / location / interfaces / weather / conditions change, or a permit condition cannot be met: STOP, make safe, and suspend/hand back PTW as per site procedure. Restart only after re-authorisation and re-brief. | Al Laith Supervisor + Issuer | Suspension note / re-issue record; re-brief/TBT record |
| 6. Extension (if needed) | Before expiry, Supervisor requests extension; Issuer confirms conditions remain valid and extends/reissues PTW with updated validity. | Issuer + Al Laith Supervisor | Extension authorisation; updated PTW displayed |
| 7. Close-out / Hand-back | On completion or end of shift (as required), area is made safe, barriers adjusted/removed as authorised, and PTW is closed/handed back. Outstanding hazards are communicated to the Issuer/site team. | Al Laith Supervisor + Issuer | PTW hand-back/close-out record; end-of-shift brief note |

| SWL & Load (Note: Enter platform SWL as kN/m² (preferred) or kg per bay/component as applicable. Cross-check against design drawing references before sign-off.) |  |  |  |
| --- | --- | --- | --- |
| Structure Name | Drawing Ref | Description | SWL |
| SS Storage Tank Scaff | ALS-FA-6020 Scaffold Structure Calculations.pdf | First draftALS-FAB-0095-01-REV00 SCAFFOLDING STRUCTURE.pdf Final drawing: ALS-FAB-0095-01-REV01 SCAFFOLDING STRUCTURE.pdf | Final check after calculations: ALS-FAB-0095-01-REV01 SCAFFOLDING STRUCTURE.pdf |

| # 9.0 Safety Requirements |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Safety Gears Required | Yes | No | Safety Equipment Required | Yes | No |
| Safety Helmet (Hard Hat) | √ |  | Safety Gloves – Cloth or leather | √ |  |
| Hearing Protection | AR |  | Safety Footwear – Boots | √ |  |
| Safety Glasses | √ |  | Dust Mask or Face Mask | √ |  |
| Coveralls | √ |  | Full Body Harness (Double Hook) > 1.8 meters | √ |  |
| High Visibility Vest – Orange for banksman and/or signallers | √ |  | Inertia Reel/Retractable | √ |  |
| Life Jackets | N/A |  | Life Line | AR |  |

√ = Required, R = Recommended, AR = As Required, N/A = Not Applicable; hearing protection selection will follow measured/expected noise levels and posted area signage.

| 9.1 | Specific Identified Residual Hazards |  |  | Refer to Risk assessment |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9.2 Required Personal Protective Equipment (select all applicable) |  |  |  | Safety Boot |  |  |  |  | Yes | Hard Hats |  |  |  |  | Yes |
|  |  |  |  |  |  |  |  |  | No |  |  |  |  |  | No |
|  |  |  |  | Eye Protection |  |  |  |  | Yes | Respiratory Protection |  |  |  |  | Yes |
|  |  |  |  |  |  |  |  |  | No |  |  |  |  |  | No |
|  |  |  |  | Safety Gloves |  |  |  |  | Yes | Hearing Protection |  |  |  |  | AR |
|  |  |  |  |  |  |  |  |  | No |  |  |  |  |  | No |
|  |  |  |  | His-Vis Clothings |  |  |  |  | Yes | Harness / lanyard |  |  |  |  | AR |
|  |  |  |  |  |  |  |  |  | No |  |  |  |  |  | No |
|  |  |  |  | Coveralls |  |  |  |  | Yes |  |  |  |  |  | AR |
|  |  |  |  |  |  |  |  |  | No |  |  |  |  |  | No |
| 9.3 | Hazardous Substances (select all applicable) |  |  | Very Toxic |  |  |  |  | Yes | Harmful / Irritant |  |  |  |  | Yes |
|  |  |  |  |  |  |  |  |  | No |  |  |  |  |  | No |
|  |  |  |  | Corrosive |  |  |  |  | Yes | Dangerous for the environment |  |  |  |  | Yes |
|  |  |  |  |  |  |  |  |  | No |  |  |  |  |  | No |
|  |  |  |  | Oxidizing |  |  |  |  | Yes | Highly flammable |  |  |  |  | Yes |
|  |  |  |  |  |  |  |  |  | No |  |  |  |  |  | No |
|  |  |  |  | Explosives |  |  |  |  | Yes | Anything else not listed above |  |  |  |  | Yes |
|  |  |  |  |  |  |  |  |  | No |  |  |  |  |  | No |
| 9.4 | Hot Weather and Heat Stress Controls - Follow the mandatory midday break and any authority heat-stress rules. Plan heavy work for cooler periods where practicable and limit non-essential outdoor tasks during peak heat. - Provide cool potable drinking water at the workface and encourage frequent drinking. Provide shaded or air-conditioned rest areas. - Supervisors to monitor heat stress indicators during pre-start and mid-shift checks and record in the Daily Brief/TBT (and Heat Stress log where used). - Acclimatisation: new/returning workers shall be gradually introduced to hot work and monitored closely. Use buddy checks. - Sun protection: long sleeves where practicable, hat/helmet shade where permitted, and SPF-rated sun protection on exposed skin. - Heat illness trigger: cramps, dizziness, confusion, fainting, or hot dry skin → STOP, move to shade/cool area, cool and hydrate, call First Aider/medical support as required, and record the event. Records: Midday-break compliance, water/rest provision, and any heat-related interventions recorded in Daily Brief/TBT and Heat Stress log (if used). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.5 | Residual Risk Monitoring - The PM/Supervisor shall complete the Daily Residual Risk Checklist before work starts and after major changes (weather, layout, method, interfaces). Any new controls or pauses shall be recorded and briefed. Minimum check items: - Working at height: risk of falls during erection/alteration/dismantle; maintain edge protection, harness systems where required, and rescue readiness. - Manual handling: strain from heavy/awkward loads; use mechanical aids first, team lifts, and planned routes. - Traffic interface: plant–pedestrian conflict during deliveries/offloading; banksman, exclusion zones, and TMP controls in place. - Adverse weather: wind/rain/heat affecting stability and access; apply wind limits, stop during lightning/heavy rain/unsafe visibility, enforce heat controls. Drift control: If residual risk increases (controls not effective, near miss, unsafe condition), take immediate corrective action and update/re-brief at TBT. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.6 | # Night Work # Night work shall only proceed with adequate supervision, lighting, segregation, and communications. # Chargehand/Supervisor conducts regular rounds and records checks in the Night Work Supervision Log (lighting, exclusion zones, housekeeping, worker alertness). # Provide task and route lighting to maintain safe visibility; check emergency lighting at shift start where required. # Reflective PPE required. Maintain barriers/signage and apply site speed limits and banksman control for any vehicle movement. # Fit-for-duty checks at start and mid-shift. Fatigue indicators → reassign task, increase breaks, or stand-down and record. # No lone working for high-risk tasks (WAH, lifting, work near/over water, confined access areas). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.7 | # Tie Testing Plan # Purpose: Verify tie/anchor capacity where the scaffold design requires pull testing or where substrate conditions are uncertain. # Test a sample of ties in each zone/elevation and each tie type as specified by the design / engineer / project ITP (use the design requirement as the governing rule). # Method: pull-test using calibrated equipment to the required proof/design load; hold as required; record pass/fail. # Records to include: tie/anchor ID, location/grid, substrate, date, tester, test load and result. # Failures: replace the anchor and re-test as required. Increase testing in the surrounding area and escalate to PM/QHSE and the designer if repeated failures occur. # When: initial installation (where required), after significant alteration affecting ties, and after events that could affect anchors (impact, storm, suspected overload). # Records: File results in the scaffold inspection/handover pack (inspection register / tie register / test certificates) and make available for audits. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.8 | # Simultaneous Operations Where multiple activities occur simultaneously within the yard, supervisors shall coordinate work to prevent conflicts between lifting operations, scaffold erection, vehicle movement and fabrication activities. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.9 | # Dropped Object Prevention # Tools secured with lanyards where practicable # No throwing of materials # Toe boards installed where working platforms exist # Exclusion zone maintained below workface # Materials stored away from edges |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.10 | # Electrical Earthing and Grounding # All metal temporary structures and any steel temporary installations, shall be electrically grounded using suitable earthing cables and approved grounding points. # Earthing shall be installed prior to use and verified by a competent person. Continuity of grounding shall be maintained at all times and rechecked after any modification. # This control is required to reduce the risk of electrocution and lightning-related hazards within the yard. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| # 10.0 Quality Monitoring |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Quality monitoring shall be carried out to ensure all works comply with approved procedures, drawings, and specifications. - Inspections shall be conducted at key stages of work - Non-conforming work shall be identified and corrected - Records shall be maintained for verification |  |  |  |  |  |

| Activity | Who checks | Verification |
| --- | --- | --- |
| Scaffold & Temporary Structure Erection / Alteration / Dismantling | Site Supervisor / Competent Scaffold Inspector | Scaffold Inspection Register, Scafftag, Alteration Log |
| Vehicle & Plant Operations (Deliveries / Offloading) | Supervisor / Operator | Daily Pre-Use Checklist, Traffic Control Record |
| Lifting Operations (Crane / HIAB / Telehandler) | Appointed Person / Supervisor | Lift Plan, Permit to Lift, Lifting Checklist, Gear Register |
| Fuel Storage & Environmental Controls | Supervisor / HSE | Weekly Storage Check, Spill Log (if applicable) |
| Housekeeping & Workface Controls (Daily) | Supervisor | Daily Housekeeping Log / Residual Risk Checklist |

| # 10.1 Daily Monitoring Checklist |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The Site Supervisor will complete a Residual Risk Daily Checklist for the following: manual handling, scaffold modification/dismantling, hot weather, night work, live fueling/plant, and vehicle/pedestrian interface. Any “No” triggers immediate corrective action and re-brief. File with the Daily Briefing Form. |  |  |  |  |  |

| # 11.0 Emergency Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11.1 |  |  |  | Name of On-Site First Aider: |  |  |  |  |  | (Complete at prestart ) Name: Phone/radio: |  |  |  |  |
|  |  |  |  | First Aid Room Location: |  |  |  |  |  | Provided within the Yard- (confirmed & Completed at prestart) |  |  |  |  |
|  |  |  |  | Location of Nearest Hospital: |  |  |  |  |  | NOOM MEDICAL CLINIC LLC - 04-5693338 |  |  |  |  |
| 11.2 | Muster Point: |  |  | Location (Complete at prestart ) |  |  |  |  |  |  |  |  |  |  |
| 11.3 | Welfare Requirements |  |  | Provide potable water, shade/rest (hot-weather), sanitary facilities, and lighting for night work as applicable. |  |  |  |  |  |  |  |  |  |  |
| 11.4 | Emergency Arrangements |  |  | Emergency arrangements within the yard include: - first aid facilities - fire extinguishers in workshop areas - emergency evacuation routes - emergency contact numbers displayed All personnel must report incidents immediately to the supervisor. |  |  |  |  |  |  |  |  |  |  |
| 11.5 | Emergency Numbers |  |  | 998 (Ambulance), 997 (Fire) |  |  |  |  |  |  |  |  |  |  |
| 11.6 | Emergency Procedures |  |  | In case of emergency Al Laith staff operatives & Sub contractors follow the action plan as mentioned in the site Health & Safety plan. |  |  |  |  |  |  |  |  |  |  |

| # 12.0 Access and Egress point |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| - Access and egress routes shall follow the approved site layout / traffic management plan (Ref. 2.2, where applicable). - The Project Manager / Supervisor shall verify routes at the start of each shift and after adverse weather or any site layout change. - Routes shall be kept clear of obstructions, stored materials, waste, and trailing cables. Barriers, signage, and pedestrian/vehicle segregation shall be maintained. - Access routes shall be adequately lit, have safe, stable walking surfaces, and include edge protection on ramps, stairs, and elevated walkways where required. - Emergency exits, fire points, and muster routes shall remain clear and unobstructed at all times. - Trenches, services, and cables crossing routes shall be covered, protected, and secured, and re-checked after rain or wind. - Any change to access/egress routes shall be briefed at pre-start, updated on site signage/plans where used, and old or conflicting signage removed. HOLD POINT: If safe access or egress cannot be maintained, STOP work and re-plan before continuing. |  |  |  |  |  |
| # 13.0 Environmental and Waste Management Arrangements |  |  |  |  |  |
| # Waste management & Pest Control # Manage waste to Al Laith standards, the project Environmental Management Plan (EMP), and the approved Waste Disposal Procedure. # Identify and classify all waste prior to disposal (general / recyclable / hazardous). # Segregate waste where practicable using labelled containers/skips (e.g., metal / timber / general / hazardous). # Maintain records of waste type, source, quantity, and storage location. # Hazardous waste/COSHH: store sealed and labelled; SDS available; use secondary containment; dispose via approved/licensed contractor and retain transfer notes, manifests, and Waste Disposal Certificates. # Only approved and licensed waste contractors shall be used. Waste carrier licenses, permits, and approvals shall be verified prior to collection. # Remove waste to the designated refuse area daily (or as scheduled). Do not obstruct access/egress. # Pest control: keep food waste in sealed bins and remove daily; follow Al Laith pest control arrangements (where applicable) and report infestations to Facilities/QHSE. |  |  |  |  |  |
| Noise # Use quieter methods/plant where practicable; position equipment to reduce impact; rotate tasks and use barriers/screens where practical. # Hearing protection requirements shall follow the Risk Assessment and site rules. # Where monitoring or site conditions indicate elevated noise, implement signage and additional controls as required. # Maintain any noise monitoring records in the work pack where required. |  |  |  |  |  |
| # Spill Prevention and Response (Fuel, Oils, Paint) Prevention - Refuel only at designated attended points; engines OFF; use drip trays and approved hoses/nozzles. - Store fuels/oils in bunded areas (≥ 110% of the largest container). - Spill kits available and checked routinely (including vehicles where applicable). - Protect drains where relevant and keep SDS available where substances are used. Response (record in Spill/Environmental Log) STOP → CONTAIN → PROTECT DRAINS → NOTIFY Supervisor/QHSE → CLEAN UP → DISPOSE via approved contractor → RECORD / CAPA If soil/water contamination is suspected: stop, contain, notify QHSE, and arrange approved clean-up and disposal; retain disposal records. |  |  |  |  |  |
| Dust - Use low-dust methods where practicable; use on-tool extraction/LEV where available, otherwise water spray/sheeting as suitable (without creating slip hazards). - Use RPE where required by the Risk Assessment. - Stop work if dust affects visibility or creates unacceptable exposure. - Clean using vacuum or damp methods; no dry sweeping. - Controls briefed at daily TBT and recorded where required. |  |  |  |  |  |
| # 15.0 Third party Interactions |  |  |  |  |  |
| - Al Laith shall coordinate works to protect third parties, adjacent trades, facility users, and the public. - Daily coordination via TBT and/or coordination meeting to confirm work zones, interfaces, communication method, and hold points. - Maintain segregation using barriers, signage, and banksman/marshal where required. - Avoid conflicting work at height: no work over others/public unless protected (e.g., overhead protection/debris netting) or re-sequenced. - Visitor control: sign-in, escort, required PPE, and brief on exclusion zones and emergency arrangements. - Any changes affecting the public interface (route closures, access changes, noisy work, lifting near routes) shall be communicated to the in advance where practicable. Attachments / Records (as applicable) - Site TMP extract / drawings reference - Daily Briefing & Residual Risk Checklist - Housekeeping / Environmental Log (incl. spill log) - Rescue kit inspection sheet (where WAH or water interface applies) |  |  |  |  |  |

| # 16.0 Permits Key Resource |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Items Attached: (Please indicate in the box below) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| No | Accidents & Incidents |  |  | Yes |  |  |  |  |  | No |  |  |  |  |
| 16.1 | Sketches/ Date sheets |  |  | x |  |  |  |  |  |  |  |  |  |  |
| 16.2 | Certification of Plant/operator competency |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16.3 | Program of Works |  |  | x |  |  |  |  |  |  |  |  |  |  |
| 16.4 | Risk Assessments |  |  | x |  |  |  |  |  |  |  |  |  |  |
| 16.5 | Excavation permit |  |  |  |  |  |  |  |  | x |  |  |  |  |
| 16.6 | Daily briefing form |  |  | x |  |  |  |  |  |  |  |  |  |  |
| 16.7 | Emergency contact details |  |  | x |  |  |  |  |  |  |  |  |  |  |
| 16.8 | Emergency arrangements |  |  | x |  |  |  |  |  |  |  |  |  |  |
| 16.9 | Rescue plan |  |  | x |  |  |  |  |  |  |  |  |  |  |
| 16.10 | Method Statement Amendments and Review |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16.11 | Wind Log, Spill Kit Checklist, Residual Risk Checklist, Housekeeping Log, and Scafftag Inspection Register to be kept at the workface or site office for audit. |  |  |  |  |  |  |  |  |  |  |  |  |  |

| # 17.0 Emergency Contact Details |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Michael Clark Operation Director - EPC M: 050 152 5411 E: michael.clark@allaith.com Reinhart Swart Yard Manager - EPC M: 050 653 9375 E: Reinhart.Swart@allaith.com Johan Tredoux QHSE Manager M: 056 179 3189 E: johan.tredoux@allaith.com |  |  |  |  |  |

| # 17.0 Review and Amendments |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| REVIEW DATES: This Method Statement will be reviewed every 3 months, and after any incident, near miss, or significant change in scope or conditions. LAST REVIEW DATE: 05/04/2026 NEXT REVIEW DATE: 05/07/2026 AMENDMENTS AUTHORIZED BY: Michael Clark Johan Tredoux AMENDMENTS COMMUNICATED TO: All operatives involved |  |  |  |  |  |

## 18.0 Emergency Procedures / Emergency Response Plan

18.01 Reference to Emergency Information

Emergency numbers, muster point(s), first aid locations/coverage, and emergency access routes are defined in Section [11.0/16.0 Emergency Contacts & Site Emergency Information] and shall be briefed at induction and available to the workface.

18.0.2 Daily confirmation (TBT / pre-start, as applicable)

Supervisor confirms and briefs for the day’s activities:

- communication method (radio channel / backup)

- rescue readiness requirements (WAH / lifting / work near or over water where applicable)

- any site-specific restrictions or hold points

### 18.0.3 Immediate Actions (Any Emergency / Incident)

- STOP work and make the area safe (isolate energy sources / stop plant movements where applicable).

- Raise the alarm as per site procedure (site emergency number / Site Control / HSE).

- Provide first aid by trained First Aider and request medical support as required.

- Rescue only if safe and only by competent persons using suitable equipment.

- Keep routes clear for emergency responders and guide them to the location.

NB: Casualty shall be treated as a medical emergency. Follow trained First Aider guidance and emergency services instructions. Do not attempt improvised medical positioning unless trained and competent.

### 18.0.4 Reporting / Scene Preservation

- Report all incidents and near misses as per company and site procedure.

- Preserve the scene and equipment where safe and instructed (do not disturb unless required for rescue or to prevent further harm).

## 18.1 Work at Height Rescue (When Fall Arrest is Used)

HOLD POINT: Where fall arrest is used, a task-specific rescue arrangement shall be in place before work starts.

Pre-start (confirm and record where required):

- agreed recovery approach suitable for the work location (nearest safe platform/ground)

- rescue kit available, accessible and checked

- suitable rescue anchor point(s) identified (no handrails/guardrails/uncertified fixtures)

- clear access/egress maintained for recovery and medical response

- communications confirmed (radio/phone and emergency contact method)

If a fall / suspension occurs:

- STOP work, make the area safe and establish an exclusion zone.

- Raise the alarm and request an emergency response / ambulance as required.

- Recover the casualty only using the agreed rescue method from the nearest safe position.

- Do not use plants to lift people unless a purpose-designed, approved man-riding/rescue system is authorised under site rules.

After recovery:

- Treat as a medical emergency and follow First Aider and emergency services instructions.

- Record incident/near miss; quarantine any loaded rescue equipment and do not re-use until checked/replaced as required.

- Affected scaffold/structure shall be inspected and released by a competent person before re-use.

Records (where required):

- rescue briefing record (TBT)

- rescue kit inspection record

- drill / exercise record (if required)

## 18.2 Rescue Without Specialist Remote Kit (Contingency)

- Notify Site Control/HSE and request emergency services as required.

- Do not expose rescuers to additional fall risk. Use the safest available access method permitted (e.g., MEWP or adjacent protected platform).

- Recover casualty to a safe area and provide first aid/medical response as required.

- Brief this contingency at TBT and ensure competent supervision and first aid arrangements are in place.

| # 19.0 Method Statement Briefing Record |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Briefing delivered by………………………………………………………….…… Position…………Supervisor or Project Manager………………….. Date…………………………………………………………………………………. We (the undersigned) have read and understood the attached method statement and will comply with the specified requirements and control measures. If the work activity changes or deviates from that originally envisaged, we will seek further advice and request an amended method statement. |  |  |  |  |  |

| Name (Print) | Signature | Date |
| --- | --- | --- |