REQUEST FOR QUOTATION — TECHNICAL SPECIFICATION
AL LAITH PROJECT SERVICES LLC

Automated Scaffold Coupler Sorting & Decoupling Cell

| Field | Value |
| --- | --- |
| Document No. | ALPS-URS-001 |
| Revision | B — Issued for Quotation (RFQ). Adds Variant D (deferred decoupling), 4K/OBB compute requirements, 24/7 duty items, mandatory Supplier Engineered Recommendation. |
| Date | 24 July 2026 |
| Site | Al Laith Project Services LLC, Dubai Industrial City, Dubai, UAE |
| Purpose | Procurement specification for a turnkey robotic sorting and decoupling cell. Suppliers shall quote against one or more of Configurations A–C (Sections 6–8), with or without Variant D (Section 9), return a clause-by-clause compliance matrix, and submit their own Engineered Recommendation (Section 12). |
| Confidentiality | Commercial in confidence. Do not distribute outside the quoting organisation. |

# Contents

Contents	2

1. Purpose and Scope	3

2. Task Definition and Process Logic	3

2.1 Process narrative	3

2.2 Pair / link decision matrix (FR-01)	3

2.3 Operating modes (FR-02)	4

2.4 Definitions	4

3. Input Material Specification	4

3.1 Part classes	4

3.2 Part condition (design basis)	4

3.3 Presentation and mix	5

4. Performance Requirements (contractual)	5

5. Common Core Stack (applies to all configurations)	6

5.1 Vision system	6

5.2 Compute and Model Integration Interface (MII)	6

5.3 Vision responsibility split (contractual)	7

5.4 End-of-arm tooling (EOAT)	7

5.5 Decouple station (fixed, robot-tended) — mandatory in base A–C; deleted under Variant D (Section 9)	8

5.6 Software and HMI	8

5.7 Safety	8

5.8 Environment and utilities (Dubai outdoor duty)	9

6. Configuration A — Single-Robot Long-Reach Cell	10

6.1 Concept and layout	10

6.2 Configuration-specific equipment	10

6.3 Cycle analysis (design basis, to be confirmed by Supplier study)	10

6.4 Best suited for	10

6.5 Known limitations (accepted by design)	10

7. Configuration B — Dual-Robot Sorting Line	11

7.1 Concept and layout	11

7.2 Configuration-specific equipment	11

7.3 Cycle analysis (design basis)	11

7.4 Best suited for	11

7.5 Known limitations (accepted by design)	11

8. Configuration C — Gantry Portal Cell	12

8.1 Concept and layout	12

8.2 Configuration-specific equipment	12

8.3 Cycle analysis (design basis)	12

8.4 Best suited for	12

8.5 Known limitations (accepted by design)	12

9. Variant D — Deferred Decoupling (applies to base Configuration A, B or C)	13

9.1 Concept	13

9.2 Variant requirements	13

9.3 Best suited for	13

9.4 Known limitations (accepted by design)	13

10. Acceptance Testing	14

10.1 Factory Acceptance Test (FAT)	14

10.2 Site Acceptance Test (SAT)	14

11. Deliverables and Documentation	14

12. Quotation Requirements (what your offer must contain)	14

Appendix A — Reference Component Classes	16

Appendix B — Glossary	16

# 1. Purpose and Scope

1.1 This document specifies a fully automated robotic cell that receives mixed scaffold tube-and-fitting couplers ("clips") returned from construction sites in bulk stillages, identifies each part, sorts parts into type-segregated bins, and automatically separates ("decouples") pairs of couplers that are clamped to one another when — and only when — the two members are of different types. Same-type linked pairs are sorted as linked pairs without separation.

1.2 Division of responsibility (fundamental to this contract):

- SUPPLIER SCOPE — all mechanical, structural, electrical, pneumatic, motion, safety, environmental hardening, vision hardware, vision runtime integration, HMI/software, installation, commissioning and acceptance testing. The Supplier must deliver a working cell that solves every problem in this document EXCEPT model training.

- CLIENT SCOPE — object-detection and classification model development only. The Client will train Ultralytics YOLO26 models — primary detector yolo26x-obb (oriented bounding boxes) exploiting native 4K resolution per VS-07, plus a crop classifier — and deliver them as TensorRT .engine files built on the cell compute device itself (MI-01). The Supplier integrates these via the Model Integration Interface (Section 5.2/5.3) and is NOT responsible for classification accuracy, but IS responsible for everything around the model: acquisition, calibration, 3D lifting, grasp planning, motion, placement, logging and reject mechanics.

1.3 Three base cell configurations (A–C) plus a Deferred-Decoupling variant (D, applicable to any base) are defined for the identical task. They share one Common Core stack (Section 5). Suppliers may quote any or all; each numbered requirement applies to all configurations unless a configuration or variant section states otherwise. Suppliers are additionally REQUIRED to submit their own engineered recommendation per Section 12.

1.4 One (1) cell is to be supplied initially. The design and all documentation shall permit replication of further cells; all documentation becomes Client property (Section 10).

# 2. Task Definition and Process Logic

## 2.1 Process narrative

Stillages of mixed, entangled couplers arrive by forklift. The cell shall: (a) present parts individually (singulation); (b) identify class of every part in the identification zone using the Client model; (c) verify every pick is exactly one object; (d) route singles to their class bin; (e) for linked pairs, classify BOTH members, then apply the decision matrix of 2.2; (f) route mixed-type pairs through the decouple station and sort both resulting parts; (g) route unresolvable items to a manual reject chute with image and reason code; (h) run day and night with no change of behaviour.

## 2.2 Pair / link decision matrix (FR-01)

The matrix below is the contractual behaviour. The HMI shall expose it as a Client-editable table per class-pair (Section 5.6), because sorting policy may change.

| Condition detected | Link state | Action | Destination |
| --- | --- | --- | --- |
| Single part, class known | n/a | Sort | Class bin |
| Two parts, SAME class | Linked | Do NOT decouple | Class bin, or linked-pair offtake — HMI policy (DL-02 under Variant D) |
| Two parts, DIFFERENT classes | Linked | Decouple at station (Variant D: no decoupling) | Each part to its own class bin (Variant D: linked-pair offtake) |
| Any member class unknown / below confidence threshold | Any | Re-present for up to 2 additional views | If still unknown: manual reject chute |
| Chain of 3 or more linked parts | Linked | No automatic action (out of auto scope, Rev A) | Manual reject chute |
| Foreign object (tube offcut, timber, debris, other hardware) | n/a | Reject | Debris chute |

Under Variant D (Section 9) the default policy routes mixed-type pairs — optionally all pairs — to the linked-pair offtake instead of the decouple station; the matrix above remains Client-editable in all cases.

## 2.3 Operating modes (FR-02)

- AUTO — continuous unattended operation, 24/7 capable.

- MANUAL — full jog (Cartesian and joint), speed override, single-cycle step, macro buttons (home, spread cycle, decouple dry-run, purge conveyor). A hardware teach pendant shall be retained in addition to the HMI.

- TEACH — zone drawing and bin teaching per Section 5.6, without safety compromise.

- MODEL-UPDATE — Client loads a new .engine per Section 5.2; total line stop ≤ 10 minutes.

- MAINTENANCE — LOTO-supported access with safe-torque-off.

## 2.4 Definitions

Identification zone: camera field where parts are classified. Grab zone: region from which the robot may pick. Sort location: taught bin pose mapped to one or more classes. Pair: exactly two couplers clamped together. Chain: three or more linked parts. Stillage: steel box/pallet cage of bulk parts.

# 3. Input Material Specification

## 3.1 Part classes

Parts are 48.3 mm tube-and-fitting scaffold couplers to BS EN 74-1 / BS 1139 patterns, hot-dip galvanised or zinc-plated forged/pressed steel. Nominal data below; the Client will confirm exact dimensions and may add classes. The system shall support at least sixteen (16) classes and unlimited class-to-bin mappings; class definitions live entirely in the Client model and class-map file (FR-03).

| Class (nominal) | Mass (kg) | Envelope (mm) | Look-alike risk / distinguishing feature |
| --- | --- | --- | --- |
| Drop-forged double (right-angle) coupler | 1.15–1.40 | ~150 × 115 × 80 | vs swivel: fixed forged spine, no centre pivot |
| Drop-forged swivel coupler | 1.30–1.55 | ~165 × 115 × 85 | Centre swivel rivet/pivot between jaws |
| Pressed double coupler | 0.90–1.10 | ~150 × 115 × 75 | Folded sheet body vs forged ribs — near-identical silhouette to forged |
| Pressed swivel coupler | 1.00–1.20 | ~165 × 115 × 80 | As above, with pivot |
| Sleeve coupler | 1.00–1.20 | ~170 × 95 × 95 | Two half-shells inline, two bolts, no 90° jaw |
| Single / putlog coupler | 0.70–0.90 | ~140 × 100 × 60 | One jaw plus flat hook plate |
| Board-retaining coupler | 0.75–0.95 | ~150 × 100 × 70 | Board hook geometry |
| Girder / beam coupler (each) | 1.60–1.90 | ~200 × 120 × 90 | Hook body with captive bolt |

## 3.2 Part condition (design basis)

- Surface: galvanised, weathered; surface rust up to ~30% of area; dust, dried concrete/paint splash possible; parts may be wet.

- Thermal: sun-soaked steel surface temperature up to 70 °C. All grippers, magnets and jaws shall tolerate sustained contact at 70 °C (EN-07).

- Fasteners: every coupler carries a captive EN 74 T-bolt with 22 mm A/F flange nut. Specified assembly torque ≈ 50 N·m; corroded breakaway torque up to 150 N·m shall be assumed. The nut shall NEVER be fully removed by the cell — the T-bolt and nut remain captive (FR-04).

## 3.3 Presentation and mix

- Stillage: nominally 1.2 × 1.0 × 0.8 m, gross ≤ 1,500 kg, ~800–1,200 parts, heavily entangled.

- Linked pairs: design basis 5–15% of incoming parts (Client survey in progress; Supplier shall state throughput sensitivity to pair rate in the quotation).

- Chains (≥3 linked): design basis 1–3% — auto-rejected per 2.2.

- Foreign objects: occasional; must be detected and rejected, never sorted or jammed.

# 4. Performance Requirements (contractual)

Values below are minimums measured at FAT and SAT (Section 9). PR-01 has per-configuration values stated in Sections 6–8.

| ID | Requirement | Value |
| --- | --- | --- |
| PR-01 | Sustained sorting throughput (singles equivalent), averaged over any 60 min of AUTO operation | A ≥ 480/h · B ≥ 720/h · C ≥ 420/h. With Variant D: A-D ≥ 600/h · B-D ≥ 720/h · C-D ≥ 480/h |
| PR-02 | Decoupling capacity (mixed-type pairs) | ≥ 60 pairs/h (Config B: ≥ 100 pairs/h, parallel to sorting). N/A under Variant D |
| PR-03 | Automatic decouple success rate on FAT batch incl. corroded worst cases | ≥ 90% (failures to reject chute with reason code). N/A under Variant D |
| PR-04 | Mechanical placement accuracy: part deposited in the bin commanded by the model output | ≥ 99.9% |
| PR-05 | Pick verification: every pick confirmed as exactly one object before sorting; multi-grabs automatically recovered (shake/re-drop/recirculate) | 100% of cycles |
| PR-06 | Availability during 3-day SAT | ≥ 95% |
| PR-07 | Stillage exchange time (forklift swap, cell paused or scanner-muted) | ≤ 90 s |
| PR-08 | Client model hot-swap, validation to resumed AUTO | ≤ 10 min |
| PR-09 | Jams requiring manual intervention | ≤ 1 per 1,000 parts |
| PR-10 | Day/night invariance: identical performance 24/7 under cell lighting | Mandatory |
| PR-11 | Continuous 24/7 duty: all mechanical and electrical equipment rated for continuous (S1) operation; preventive-maintenance intervals stated in run-hours assuming 24/7 duty | Mandatory |

# 5. Common Core Stack (applies to all configurations)

## 5.1 Vision system

| ID | Requirement |
| --- | --- |
| VS-01 | Primary 3D camera over the identification/grab zone: colour ≥ 3840 × 2160 (4K UHD); registered depth ≥ 1.2 MP; Z accuracy ≤ 0.5 mm at working distance; full acquisition (trigger to point cloud) ≤ 300 ms; HDR/multi-exposure capability proven on shiny galvanised steel. Reference class: Mech-Mind Mech-Eye PRO M / LSR L, Photoneo MotionCam-3D M+, Zivid 2+ M60, or equivalent. |
| VS-02 | Close-range 2D inspection camera (wrist-mounted, or station-mounted at the pair-presentation position): ≥ 5 MP global-shutter colour with ring light, used for pair member classification and nut localisation. Reference class: Hikrobot MV-CS050 / Daheng 5 MP GS. |
| VS-03 | Lighting: identification and grab zones fully enclosed in a shroud/hood with matte interior; high-CRI (≥ 90) flicker-free LED illumination ≥ 2,000 lx at the pick plane; identical illumination day and night. Direct sunlight shall never reach the imaging volume. |
| VS-04 | All optical windows fitted with positive-pressure purge or air-knife to prevent dust film; camera housings ≥ IP65. |
| VS-05 | Calibration: Supplier provides hand-eye calibration routines and fixtures; combined hand-eye residual ≤ 0.5 mm; TCP calibration procedure documented; recalibration executable by trained Client staff in ≤ 30 min. |
| VS-06 | Nut-pose localisation at the decouple station: ± 1 mm position, ± 3° rotation. |
| VS-07 | Native-resolution utilisation: detection shall exploit the FULL 4K frame. Full-frame downscaling that discards pixel density is not acceptable for the look-alike classes of Section 3.1. The runtime shall provide batched TILED inference (configurable tile size 1280–1536 px, overlap ≥ 15%, cross-tile merging compatible with NMS-free / oriented-box outputs) AND shall accept Client full-frame engines (3840 × 2176 letterboxed) wherever the MI-07 latency budget is still met. |

## 5.2 Compute and Model Integration Interface (MII)

One computer performs both inference and Client model training (training is a Client activity, run off-shift on the same machine so TensorRT engines are built on the deployment GPU architecture).

| ID | Requirement |
| --- | --- |
| MI-01 | ONE on-cell compute device performs BOTH inference and Client model training. Reference class: industrial GPU workstation, GPU ≥ 32 GB VRAM (RTX 5090 class), 48 GB (RTX 6000 Ada class) preferred; CPU ≥ 16 cores; ≥ 128 GB RAM; ≥ 4 TB NVMe (RAID-1); dual 10 GbE; Ubuntu 22.04/24.04 LTS; CUDA ≥ 12.4; TensorRT ≥ 10; housed per EN-03 with ≥ 10 min UPS; chassis shall accommodate an optional second GPU dedicated to training. TRAINING CAPABILITY FLOOR (demonstrated at FAT on the delivered device): fine-tune of yolo26x-obb, 10,000-image dataset, 100 epochs at imgsz 1536, completed in ≤ 12 h with thermals sustained inside the cabinet. Embedded modules (e.g., NVIDIA Jetson Thor class) are acceptable ONLY where this benchmark is met. |
| MI-02 | Runtime shall load Client-supplied TensorRT .engine files. PRIMARY detector: yolo26x-obb, FP16, end-to-end NMS-free oriented-box output — the box angle is consumed directly as grasp yaw; tile inputs 1×3×1280×1280 up to 1×3×1536×1536 per VS-07, plus optional full-frame 1×3×2176×3840. SECONDARY classifier: 1×3×224×224 crops taken from the NATIVE 4K frame. All engines are built by the Client on the MI-01 device (TensorRT engines are GPU-architecture-locked; single-device train-and-deploy is by design). |
| MI-03 | Pre/post-processing (letterboxing, normalisation, output tensor parsing) driven by a Client-editable JSON/YAML configuration; class map and per-class confidence thresholds Client-editable without Supplier involvement. |
| MI-04 | Golden-set validator: tool that dry-runs any candidate engine against ≥ 100 stored reference frames and reports per-class deltas BEFORE activation. Activation with one-click rollback to the previous engine. |
| MI-05 | Detection results (2D boxes, classes, confidences, lifted 3D poses and grasp yaw) published on a documented gRPC or ROS 2 interface; full interface documentation is a contract deliverable. |
| MI-06 | Data pipeline: rolling ≥ 30-day storage of raw frames + system annotations + outcomes; one-click dataset export (images + JSON) for Client retraining. All data remains on-premises and is Client property; the cell shall have zero cloud dependency for operation. |
| MI-07 | Latency budget on the proposed device: oriented-box detection over the FULL 4K frame (batched tiles per VS-07) ≤ 150 ms; end-to-end vision decision (trigger to grasp command) ≤ 500 ms. Budgets hold 24/7, including during any concurrent training per MI-08. |
| MI-08 | 24/7 service duty: the inference service runs continuously day and night with watchdog auto-recovery; ≤ 1 unplanned vision-service restart per 7 days; zero GPU-memory growth over a 72 h soak (verified at FAT). Client training jobs shall be schedulable with resource isolation or preemption such that MI-07 is never violated — otherwise jobs queue automatically to idle windows. |

## 5.3 Vision responsibility split (contractual)

| CLIENT provides / owns | SUPPLIER provides / owns |
| --- | --- |
| Training datasets and labels (captured via MI-06 pipeline) | Cameras, optics, lighting, shroud, mounts |
| Model training; .engine files (detection, classification, optional OBB) | Calibration (VS-05) and its long-term stability |
| Class map, thresholds, pair decision-matrix policy values | Acquisition latency, runtime integration, MII (5.2) |
| Classification accuracy of the model | 2D→3D lifting, grasp pose generation, collision-free motion |
| Model updates over the cell lifetime | Pick verification mechanics, placement ≥ 99.9%, reject mechanics, logging/export pipeline, vision-chain uptime |

## 5.4 End-of-arm tooling (EOAT)

| ID | Requirement |
| --- | --- |
| ET-01 | Dual-mounted EOAT on one bracket (no tool changer): electro-permanent magnet + servo parallel gripper + wrist 2D camera (VS-02). Total EOAT mass ≤ 10 kg. |
| ET-02 | Electro-permanent magnet: pole footprint ≤ 60 mm sized for single-coupler acquisition from a cluttered surface; verified hold ≥ 3× safety factor on a 2.0 kg part at worst-case contact; programmable partial-release and shake routine to shed hitchhiker parts. Reference class: Magswitch AR / SMC MHM / Schmalz. |
| ET-03 | Servo parallel gripper: stroke ≥ 80 mm; programmable force 100–600 N with position feedback; hardened jaws profiled for (a) coupler body grip and (b) the pair-extraction pull at the decouple station. Reference class: DH-Robotics PGI/AG series, Schunk EGU. |
| ET-04 | All EOAT elements rated for sustained contact with 70 °C parts and ambient per Section 5.8; collision protection (sensor or software) fitted. |

## 5.5 Decouple station (fixed, robot-tended) — mandatory in base A–C; deleted under Variant D (Section 9)

The decoupler is a fixed station, not a wrist tool: the robot only loads pairs and extracts the freed part; the station performs clamping, loosening and flap opening. This isolates the highest-risk function into a bench-testable fixture.

| ID | Requirement |
| --- | --- |
| DS-01 | Self-centering vise (pneumatic or servo), hardened V-jaws, clamp force 2–4 kN, clamps the LOWER coupler of the pair as loaded by the robot in a vision-derived orientation. |
| DS-02 | DC servo nutrunner, programmable 20–160 N·m with torque + angle monitoring and per-event data logging; 22 mm A/F socket with magnetic retention; mounted on a compliant slide (± 5 mm float) with automatic engagement onto the nut located per VS-06. Reference class: Atlas Copco QST / Desoutter ECS / ESTIC. |
| DS-03 | Sequence: clamp → back the nut off 4–6 turns ONLY (nut and T-bolt remain captive, FR-04) → actuated flap opener releases the jaw → robot gripper extracts the upper coupler along a programmed arc clearing the jaw geometry → both parts released to routing. |
| DS-04 | Station cycle (clamp-to-release) ≤ 25 s. Automatic success ≥ 90% (PR-03). |
| DS-05 | Failure handling: torque ceiling reached (seized > 150 N·m), thread damage, deformed body, nut not found — pair ejected to reject chute with reason code and image; no manual intervention required to continue AUTO. |
| DS-06 | Station fully guarded; no human co-working; robot-tended only. |

## 5.6 Software and HMI

| ID | Requirement |
| --- | --- |
| SW-01 | Zone editor: operator draws identification-zone, grab-zone and exclusion polygons directly on the live camera view; stored per recipe. |
| SW-02 | Sort-location teaching: jog-and-save bin poses; named bins; drag-and-drop class-to-bin routing table; multiple classes may share a bin. |
| SW-03 | Pair decision-matrix editor implementing Section 2.2, Client-editable per class-pair (sort-linked / decouple / reject). |
| SW-04 | Manual operation interface: Cartesian and joint jog with speed override from the HMI, plus the OEM teach pendant; macro buttons (home, spread cycle, decouple dry-run, purge/recirculate). |
| SW-05 | Dashboard: live per-class counts, throughput, decouple success %, reject Pareto with images, OEE; shift and batch reports. |
| SW-06 | Recipes: save/load complete configuration (zones, bins, thresholds, matrix, active model version). |
| SW-07 | Integration: REST API + OPC UA + webhooks for counts, states and batch events (Client will integrate with existing Google Workspace tooling). |
| SW-08 | Operator UI: icon-first design usable by a multilingual workforce; language packs minimum English, Hindi, Urdu; role-based access; full audit log. |
| SW-09 | Remote support via Client-controlled VPN only. |

## 5.7 Safety

| ID | Requirement |
| --- | --- |
| SF-01 | Compliance: ISO 10218-1/-2:2025 and ISO 13849-1; safety functions Category 3 / PL d minimum. CE-equivalent technical file and UAE compliance documentation delivered. |
| SF-02 | Perimeter fencing ≥ 2.0 m with guard-locked interlocked gates; E-stops at HMI, gates and load point; stack light and siren. |
| SF-03 | Stillage exchange: either a scanner-supervised muted load zone permitting forklift swap without full cell stop, or an interlocked full-stop swap — in both cases meeting PR-07 (≤ 90 s). |
| SF-04 | LOTO isolation points for electrical, pneumatic and hydraulic energy; safe-torque-off for maintenance mode. |

## 5.8 Environment and utilities (Dubai outdoor duty)

| ID | Requirement |
| --- | --- |
| EN-01 | Design ambient +5 to +50 °C. All equipment shaded by a cell canopy (Supplier to specify/supply). Any derating above 45 °C shall be explicitly documented with its throughput effect. |
| EN-02 | Dust: fine desert dust and periodic sandstorms. Field devices ≥ IP65; robot wrist IP67, body ≥ IP54 plus protective dust jacket; a defined sandstorm shutdown/park mode. |
| EN-03 | Electronics: all drives, PLC and the compute of MI-01 in air-conditioned cabinets maintaining ≤ 35 °C internal at 50 °C ambient. |
| EN-04 | Humidity to 95% non-condensing (coastal); structures hot-dip galvanised or C4-rated paint; stainless fasteners. |
| EN-05 | Power: 400 V, 3-phase, 50 Hz, TN-S. Supplier states connected load per configuration (indicative: A ≈ 20 kVA, B ≈ 35 kVA, C ≈ 30 kVA). |
| EN-06 | Compressed air: 6 bar dry air available; Supplier states consumption. |
| EN-07 | All part-contact tooling rated for 70 °C part surface temperature (see 3.2). Noise ≤ 80 dB(A) at 1 m. |
| EN-08 | Night operation: shrouded imaging volumes are lighting-invariant per VS-03; in addition, operator, forklift and offtake zones shall have ≥ 200 lx task lighting at night; all luminaires rated ≥ 50,000 h for 24/7 duty. |

# 6. Configuration A — Single-Robot Long-Reach Cell

## 6.1 Concept and layout

One long-reach six-axis robot on a central pedestal performs every handling task. A dosing tipper meters parts from the stillage onto a vibratory spread table (1.2 × 1.2 m) inside the shrouded identification/grab zone. Sort bins (12–16) are arranged in an arc at radius 1.5–3.0 m. The decouple station (5.5) and reject/debris chutes sit inside the reach envelope. Footprint ≈ 8 × 7 m excluding canopy.

## 6.2 Configuration-specific equipment

| Item | Specification |
| --- | --- |
| Robot | Six-axis, reach ≥ 3,000 mm, payload ≥ 20 kg, repeatability ≤ ±0.1 mm, IP67 wrist, TCP speed ≥ 2.5 m/s. Reference class: FANUC M-710iC/20L, Yaskawa GP20HL, KUKA KR 20 R3100, or equivalent long-arm models meeting this line. |
| Stillage handling | Hydraulic dosing tipper, 2,000 kg capacity, dribble-feed control onto spread table; guarded. |
| Singulation | Vibratory spread table 1.2 × 1.2 m, hardened deck, VFD; robot pick-shake-redrop routine for clusters (PR-05). |
| Common core | Per Section 5 in full. |

## 6.3 Cycle analysis (design basis, to be confirmed by Supplier study)

- Pick-verify-place over average 2.5 m moves ≈ 4.5–5 s → ≈ 12/min ceiling; with verification and cluster retries, net 8–10 picks/min.

- Each mixed pair costs ≈ 16 s of robot time (load + later unload) while the station cycles autonomously (≤ 25 s). At the 10% pair-rate design basis this taxes sorting throughput by ≈ 25–30%.

- Contractual PR-01 for Configuration A: ≥ 480 parts/h at 10% pair incidence.

## 6.4 Best suited for

Smallest footprint; lowest subsystem count and integration complexity; one robot program; simplest maintenance. The correct choice where yard space is tight and measured pair incidence is LOW (< 5%).

## 6.5 Known limitations (accepted by design)

- Sorting and decoupling compete for the same arm: throughput degrades linearly with pair rate.

- Single robot = zero redundancy; any robot fault stops all functions.

# 7. Configuration B — Dual-Robot Sorting Line

## 7.1 Concept and layout

Mechanical singulation replaces vision-led pile picking. A hydraulic stillage tipper discharges into a hopper feeding a step/elevator feeder rated for forged steel parts, which meters parts onto a 600 mm flat belt with encoder tracking. The belt passes through the shrouded vision tunnel (VS-01/VS-03). Robot 1 picks-and-sorts singles into a bin matrix flanking the belt and diverts detected pairs to a buffered pair lane. Robot 2 tends the decouple station from the buffer, fully in parallel. Unpicked or overlapped parts run off the belt end into a recirculation chute back to the hopper — nothing is ever lost or force-handled. Footprint ≈ 15 × 6 m excluding canopy.

## 7.2 Configuration-specific equipment

| Item | Specification |
| --- | --- |
| Stillage tipper | Hydraulic box tipper, 2,000 kg, full discharge into hopper; guarded and interlocked. |
| Step feeder | Steel-part-rated step/elevator feeder, hardened contact surfaces, 1.5–2.0 t/h, VFD-controlled dosing. |
| Conveyor | Flat belt 600 mm wide, variable speed, encoder for part tracking, sidewalls; end-of-belt recirculation chute to hopper. |
| Robot 1 (sorter) | Six-axis, reach ≥ 1,700 mm, payload ≥ 15 kg, ≤ ±0.05 mm, IP67 wrist. Reference class: FANUC M-20iD/25, Yaskawa GP25, ESTUN/EFORT equivalent. |
| Robot 2 (decouple tender) | Six-axis, reach ≥ 900 mm, payload ≥ 7 kg. Reference class: FANUC LR Mate 200iD/7L, Yaskawa GP8, or equivalent industrial (cobots acceptable only if the Supplier cycle study still meets PR-02). |
| Common core | Per Section 5 in full; VS-01 mounted over the belt tunnel; VS-02 at the pair presentation stand. |

## 7.3 Cycle analysis (design basis)

- Robot 1 short moves (≈ 1.2 m) at 3.5–4 s per pick → 12–15 picks/min gross; contractual PR-01 for Configuration B: ≥ 720 parts/h.

- Decoupling is fully decoupled from sorting: station 25 s + Robot 2 handling → ≥ 100 pairs/h capacity (PR-02 override). Pair rate has near-zero effect on sorting throughput.

- Recirculation loop absorbs occlusion and residual entanglement without robot retries.

## 7.4 Best suited for

Highest sustained throughput; throughput independent of pair incidence; graceful degradation (sorter down — decoupling continues; feeder down — Robot 1 can pile-pick at reduced rate as an optional mode); cleanest scaling path (mirror a second line). Both robots are commodity sizes that many integrators, including Chinese vendors, quote routinely.

## 7.5 Known limitations (accepted by design)

- Largest footprint and subsystem count (tipper, feeder, conveyor, two robots) — the most integration interfaces; mitigated by the FAT batch test of Section 9.

- Step feeders require tuning to the part family during commissioning; linked pairs and chains must pass the feeder without jamming (Supplier to verify at FAT).

# 8. Configuration C — Gantry Portal Cell

## 8.1 Concept and layout

A heavy Cartesian portal spans the entire process field: stillage dock, spread plate, shrouded identification/grab zone, a large bin matrix (20+ bins) and the decouple station. The gantry picks DIRECTLY from the stillage with vertical tool access (no tipper); entangled clusters are lifted to the spread plate, dropped and rescanned. Footprint ≈ 10 × 7 m excluding canopy.

## 8.2 Configuration-specific equipment

| Item | Specification |
| --- | --- |
| Portal axes | X 6,000 mm × Y 3,000 mm × Z 1,500 mm; rack-and-pinion X/Y; carriage payload ≥ 50 kg; positioning ≤ ±0.2 mm; speed ≥ 2 m/s (X/Y). |
| Wrist | 2-axis servo wrist (yaw ± 270°, pitch ≥ ±120°) carrying the Section 5.4 EOAT — or, alternatively, a compact six-axis robot inverted on the carriage. |
| Rail protection | Bellows plus positively pressurised rail/rack housings; wipers; IP54 minimum for all linear elements; documented preventive-maintenance intervals for the dust duty of EN-02. |
| Common core | Per Section 5 in full. |

## 8.3 Cycle analysis (design basis)

- Long travels (avg ≈ 3 m at 2 m/s plus accel) + pick/place ≈ 7–8 s per cycle → 7–8 picks/min. Contractual PR-01 for Configuration C: ≥ 420 parts/h.

## 8.4 Best suited for

Direct-from-stillage operation with no tipper or feeder; very large sorting matrices; maximum structural rigidity for extraction pulls; straightforward future expansion by adding a second carriage on the same portal.

## 8.5 Known limitations (accepted by design)

- Exposed linear rails in desert dust are a permanent maintenance liability even with EN-02 mitigations.

- Deep-in-stillage picking of entangled steel is the single hardest perception-and-motion problem in this document; Configuration C retains that risk, whereas A partially and B fully engineer it away.

- A 2-axis wrist has less dexterity for shake-off and extraction arcs than a six-axis arm.

- Fewer integrators hold references for ruggedised outdoor gantries of this class.

# 9. Variant D — Deferred Decoupling (applies to base Configuration A, B or C)

## 9.1 Concept

Variant D removes the decouple station from day-one scope. Every linked pair is still fully identified — BOTH members classified, logged and reported — but is routed to a dedicated linked-pair offtake for downstream manual decoupling or a future robotic decouple module. The base cell is otherwise unchanged. The variant trades capital cost and integration risk for a controlled downstream task, and creates a clean, pre-engineered Phase-2 upgrade slot that any supplier can later fill.

## 9.2 Variant requirements

| ID | Requirement |
| --- | --- |
| DL-01 | Linked-pair offtake: chute or drop station into an exchangeable stillage/bin ≥ 0.5 m³ with fill-level detection and HMI alert; exchangeable by forklift within PR-07 (≤ 90 s). |
| DL-02 | Routing policy switch in the HMI decision matrix (SW-03): route ALL linked pairs to the offtake, or mixed-type pairs only (same-type linked pairs then go to their class bin). |
| DL-03 | Pair manifest: for every pair sent to the offtake, both member classes, confidences and an image are logged; batch reports state pair-composition counts per offtake stillage; exportable via MI-06. The identification data outlives the deferred action. |
| DL-04 | Phase-2 interface reservation: general-arrangement floor space for a future decouple module; ≥ 16 spare DI/DO plus 2 safe inputs; one spare configurable safety zone; network drop; 6-bar air stub; robot reach allocation or conveyor stub to the reserved area. The MII pair-assessment stream (MI-05) is published from day one so a future module — from any supplier — consumes it directly. |
| DL-05 | OPTIONAL supply item (price separately): manual decouple bench — quick-clamp vise, torque-controlled 22 mm nutrunner (battery or DC), ergonomic standing height, freed-singles chute returning parts to the cell infeed/recirculation; capacity ≥ 60 pairs/h per operator. |
| DL-06 | Contractual values under this variant: PR-02 and PR-03 do not apply; PR-01 takes the Variant-D values of Section 4; all other requirements unchanged, including FR-04. |

## 9.3 Best suited for

Fastest, lowest-risk route to a working sorter. Correct when pair incidence is uncertain, when downstream manual labour is available, or when the Client prefers to award the decouple module later as a competitive Phase 2 — which DL-03/DL-04 make possible without supplier lock-in.

## 9.4 Known limitations (accepted by design)

- Manual decoupling labour persists downstream: at the 10% design-basis pair rate and 600 parts/h, roughly one bench operator keeps pace.

- Linked pairs occupy disproportionate offtake volume; stillage exchange frequency rises with pair rate.

- Deferral does not relax FR-04 (captive fasteners) or Section 5.5 requirements for the future module.

# 10. Acceptance Testing

## 10.1 Factory Acceptance Test (FAT)

- Client supplies a 500-part mixed batch including ≥ 50 linked pairs (of which ≥ 25 mixed-type, including corroded worst-case fasteners) and representative foreign objects.

- Measured: PR-01 throughput; PR-04 placement (≥ 99.9%); PR-03 decouple success (≥ 90%); PR-05 pick verification; 8-hour endurance run; every reject logged with image and reason code.

- Model-swap test: Client loads a new .engine and validates via the MI-04 golden set; AUTO resumed within 10 minutes (PR-08).

- Compute demonstration: the MI-01 training capability floor executed on the delivered device; MI-08 72-hour soak evidence presented; MI-07 latency measured over full 4K frames.

## 10.2 Site Acceptance Test (SAT)

- Three consecutive production days on site under real ambient conditions (scheduled in hot season where possible); availability ≥ 95% (PR-06); all Section 4 values at contract levels; stillage exchange per PR-07/SF-03 demonstrated with Client forklift.

Payment milestones tied to FAT and SAT are commercial matters outside this document, but Suppliers shall structure quotations to accommodate acceptance-gated payments.

# 11. Deliverables and Documentation

- Complete 3D CAD (STEP) of the cell; general-arrangement and foundation drawings.

- Electrical schematics (EPLAN or PDF + editable source), pneumatic/hydraulic schematics.

- PLC and robot programs as source, or licensed binaries with full configuration access; complete MII API documentation (5.2).

- Calibration procedures and fixtures (VS-05); maintenance manual with preventive-maintenance schedule tuned for EN-01/EN-02 duty.

- Recommended spare-parts list with lead times; consumables list (jaws, sockets, belts, filters).

- Operator and maintenance training on site; operator work instructions in visual/icon format with English, Hindi and Urdu text.

- Warranty ≥ 12 months from SAT; statement of UAE or regional service capability and response times.

- All documentation and data become Client property and shall be sufficient for the Client to replicate, maintain and modify the cell.

# 12. Quotation Requirements (what your offer must contain)

- Clause-by-clause compliance matrix against every numbered requirement in this document (Comply / Deviate / Alternative), with all deviations itemised and justified.

- Configuration(s) quoted (A–C, with or without Variant D) with layout drawing and a cycle-time study substantiating PR-01/PR-02, including stated sensitivity to pair rate.

- Named makes/models for every major component (robot(s), 3D camera, nutrunner, feeder, grippers, GPU workstation), with datasheets.

- Delivery schedule to FAT and SAT; FAT/SAT test plans per Section 9.

- Utilities: connected electrical load and compressed-air consumption.

- Service: spare-parts pricing, warranty terms, remote and on-site support model for the UAE.

- SUPPLIER ENGINEERED RECOMMENDATION (mandatory): propose the path YOU would take for this task — one of Configurations A–C with or without Variant D, a hybrid, or an alternative architecture — with recommended makes and models for EVERY major component (robot(s), 3D and 2D cameras, compute device, grippers, magnet, nutrunner, feeder/singulation, safety), engineering rationale against the design bases of Sections 2–4 (pair-rate sensitivity, look-alike classes, 24/7 desert duty), and a phasing plan (e.g., Variant D day one, decouple module as Phase 2 into the DL-04 slot). This section is evaluated on engineering merit separately from the compliance matrix; deviations that genuinely improve the outcome are welcomed here.

- Proposed compute device named, with evidence (benchmark data or a binding test commitment) that the MI-01 training capability floor and the MI-07/MI-08 duty are met on that device.

- Confirmation of the responsibility split of Sections 1.2 and 5.3, and of unconditional compliance with MI-01…MI-08 (the Model Integration Interface is non-negotiable).

# Appendix A — Reference Component Classes

References define a performance class, not a mandated brand. Equivalents meeting the numbered requirements are acceptable and must be named in the compliance matrix.

| Category | Reference examples |
| --- | --- |
| Long-reach 6-axis (Config A) | FANUC M-710iC/20L · Yaskawa GP20HL · KUKA KR 20 R3100 · ESTUN / SIASUN long-arm equivalents |
| Mid 6-axis (Config B, Robot 1) | FANUC M-20iD/25 · Yaskawa GP25 · ESTUN ER20 series · EFORT equivalents |
| Small 6-axis (Config B, Robot 2) | FANUC LR Mate 200iD/7L · Yaskawa GP8 · Dobot / JAKA industrial equivalents |
| 3D camera | Mech-Mind Mech-Eye PRO M / LSR L · Photoneo MotionCam-3D M+ · Zivid 2+ M60 · Percipio industrial series |
| 2D inspection camera | Hikrobot MV-CS050 class · Daheng MER2 5 MP global shutter |
| Servo gripper | DH-Robotics PGI / AG series · Schunk EGU |
| Electro-permanent magnet | Magswitch AR series · SMC MHM · Schmalz |
| DC servo nutrunner | Atlas Copco QST · Desoutter ECS · ESTIC |
| Compute device | Neousys / Vecow industrial RTX class; assembled RTX 5090 / RTX 6000 Ada workstation acceptable when housed per EN-03; NVIDIA Jetson Thor class ONLY where the MI-01 training benchmark is demonstrably met |
| Safety components | SICK microScan3 · Pilz · Schmersal guard locking, or equivalent PL d parts |

# Appendix B — Glossary

| Term | Meaning |
| --- | --- |
| .engine | Serialized NVIDIA TensorRT inference model, GPU-architecture-specific. |
| YOLO26 | Ultralytics object-detection model family (end-to-end, NMS-free) trained by the Client. |
| yolo26x-obb | Largest YOLO26 variant with an oriented-bounding-box head; the box angle is used directly as grasp yaw. |
| Tiled inference | Slicing a full-resolution frame into overlapping tiles, batch-inferring, and merging detections — preserves native 4K pixel density within the latency budget. |
| EOAT | End-of-arm tooling. |
| EPM | Electro-permanent magnet gripper. |
| MII | Model Integration Interface, Section 5.2. |
| Pair / Chain | Two linked couplers / three or more linked couplers. |
| Singulation | Presenting bulk parts as pickable individuals. |
| FAT / SAT | Factory / Site Acceptance Test. |