AL LAITH YARD

METHOD STATEMENT

Ref: ALS-MS-EPC-YARD-2000

FACILITIES MANAGEMENT AND YARD SUPPORT ACTIVITIES

WITHIN AL LAITH GROUP DIC YARD

JOB NUMBER: 2000

# CONTENTS

1.0 TASK DETAILS
 1.1 Customer
 1.2 Project Name
 1.3 Site Address / Location
 1.4 Description of the Task / Activity

2.0 REFERENCES
 2.1 Specification
 2.2 Reference Drawings / Documents
 2.3 Documentation Availability / Management of Change

3.0 SCOPE AND OBJECTIVES

4.0 PROGRAM

5.0 KEY RESOURCES
 5.1 Roles, Responsibilities and Authority
 5.2 Selection of People / Competence
 5.3 Worker Participation and Consultation

6.0 CONTROLS AND RESOURCES
 6.1 Controls
 6.2 Accidents and Incidents
 6.3 Corrective and Preventive Action
 6.4 Materials
 6.5 Other Essential Equipment
 6.6 Plant and Equipment
 6.7 Yard Traffic / Plant Movement Control
 6.8 Identified Risks
 6.9 Site-Specific Rules and Restrictions
 6.10 Housekeeping and Inspections

7.0 SEQUENCE OF OPERATION
 7.1 Permits, Interfaces, Area Control and Weather Considerations
 7.2 Pre-Job Planning and Start Criteria
 7.3 Traffic Management, Deliveries and Vehicle Movement
 7.4 Materials Handling, Storage and Stacking
 7.5 Manual Handling
 7.6 Hand Tools, Power Tools and Minor Maintenance Equipment
 7.7 Maintenance Coordination
 7.8 Housekeeping, Waste Management and Cleaning Coordination
 7.9 Inspection, Reporting and Close-Out
 7.10 Carpentry Works
 7.11 AC Repair & Maintenance (Split Units Only)
 7.12 Plumbing & Waste Tank Cleaning (incl. Chemicals)
 7.13 Lifting Operations

8.0 SPECIALIST AND HIGH-RISK ACTIVITIES

9.0 SUPPORT WORK AND STORAGE ARRANGEMENT
 9.1 Temporary Supports, Barriers and Access Restrictions
 9.2 Storage Arrangements
 9.3 Permit to Work
 9.4 Safe Working Loads and Load-Controlled Areas

10.0 SAFETY REQUIREMENTS
 10.1 Specific Identified Residual Hazards
 10.2 Required Personal Protective Equipment
 10.3 Hazardous Substances
 10.4 Hot Weather and Heat Stress Controls
 10.5 Residual Risk Monitoring
 10.6 Night Work
 10.7 Specialist Testing, Inspection and Certification
 10.8 Simultaneous Operations

11.0 QUALITY MONITORING
 11.1 Daily Monitoring Checklist

12.0 EMERGENCY PROCEDURES

13.0 ACCESS AND EGRESS POINTS

14.0 MANUAL HANDLING

15.0 ENVIRONMENTAL AND WASTE MANAGEMENT ARRANGEMENTS

16.0 THIRD PARTY INTERACTIONS

17.0 PERMIT-TO-WORK CONTROL

18.0 EMERGENCY CONTACT DETAILS

19.0 REVIEW AND AMENDMENTS

20.0 METHOD STATEMENT BRIEFING RECORD

| # 1.0 Task Details |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1.1 | Customer | Name: Al Laith Group | Address: P O Box 27349, Saih Shuaib 3 |  |  |
| 1.2 | Project Name | Yard Activities |  |  |  |
| 1.3 | Site Address/Location | Saih Shuaib 3, Dubai Industrial City (DIC), Dubai, UAE - DIC Yard only |  | Date: | Daily |
|  |  |  |  | Time: | As per Yard Working Hours / Approved Schedule |
| 1.4 | Description of the Task/Activity | ### This Method Statement outlines the safe system of work for the coordination, inspection, monitoring, reporting, follow-up, and control of general facilities management and yard support activities conducted within the Al Laith Group yard at Dubai Industrial City. ### The activities covered under this Method Statement are primarily related to facilities coordination, yard upkeep, maintenance support, housekeeping control, contractor coordination, defect reporting, welfare facility monitoring, emergency access checks, and general support to routine yard operations. ### The Facilities Coordinator is responsible for planning, reporting, coordination, follow-up, and escalation of facilities-related tasks. Specialist or high-risk work shall only be carried out by competent and authorized personnel under separate approved controls where required. ### This Method Statement does not authorize to carry out specialist or high-risk activities. Where such activities are required, they shall be assigned only to competent and authorized personnel and controlled through separate task-specific MSRA, permit-to-work, inspection records, certificates, and supervision as applicable. |  |  |  |
| # 2.0 References |  |  |  |  |  |
| 2.1 | Specification | All works shall be planned, coordinated, supervised, inspected, and controlled in accordance with the applicable requirements of: - UAE Federal Labour Law and applicable occupational health and safety regulations. - Dubai Municipality health, safety, and environmental requirements, where applicable. - Dubai Industrial City / site-specific rules and requirements, where applicable. - ISO 45001:2018 Occupational Health and Safety Management Systems. - Al Laith Group internal HSE procedures. - Al Laith Group yard rules and traffic management requirements. - Permit-to-Work procedure, where applicable. - Emergency response procedure. - Waste management procedure. - Contractor management procedure. - Manufacturer instructions for tools, equipment, and materials used by competent personnel. - Approved maintenance procedures and service provider documents, where applicable. Where specialist or high-risk activities are required, the applicable standards, manufacturer instructions, permits, statutory requirements, third-party certificates, and task-specific MSRA shall be reviewed and implemented before work begins. Examples of such specialist activities include lifting operations, electrical works, hot works, work at height, scaffold works, MEWP works, confined space entry, excavation, pressure system works, and fire system impairment. |  |  |  |
| 2.2 | Reference (Drawings) |  |  |  |  |
| 2.3 | This Method Statement shall be read together with the approved Risk Assessment and any applicable supporting documents listed in Section 2.2. Controls identified in the Risk Assessment shall be implemented through the work sequence and control measures described in this Method Statement. Any deviation, change, or unforeseen condition shall trigger a stop, reassessment, and escalation to the responsible supervisor, manager, or HSE representative before work proceeds. The following documents shall be available at the workface, supervisor file, or facilities records where applicable: - Approved Method Statement and Risk Assessment. - Latest revision register. - Facilities inspection checklist. - Maintenance request log. - Defect register. - Corrective action tracker. - Contractor attendance records. - Contractor MSRA, permits, licenses, and certificates where applicable. - Daily toolbox talk records and attendance where required. - Plant and equipment inspection records where applicable. - Lifting gear certificates where lifting operations are separately approved. - Cleaning and waste collection records. - Pest control service reports. - Fire safety observation reports. - Before-and-after photographs for completed corrective actions. - Permit-to-work documents for high-risk activities. - Management of Change records where applicable. ## Management of Change / Stop Work Rule Work shall stop immediately and the Method Statement and Risk Assessment shall be reviewed where any change or unsafe condition occurs, including but not limited to: - Change in work area or yard layout. - Change in access route, traffic route, or segregation arrangement. - Change in work sequence. - Introduction of new contractor, plant, equipment, or service provider. - Change in staffing, supervision, or competency level. - Discovery of damaged infrastructure, exposed services, electrical defect, leak, unstable storage, blocked emergency route, or unsafe condition. - SIMOPS conflict with other yard activities. - Weather conditions affecting safe access, visibility, or outdoor work. - Any incident, near miss, damage, complaint, or safety observation. - Any task requiring permit-to-work or specialist assessment. Work shall only resume once the hazard has been assessed, suitable controls have been implemented, and approval has been given by the responsible person. |  |  |  |  |
| # 3.0 Scope & Objectives |  |  |  |  |  |
| The purpose of this Method Statement is to define the operational framework for the coordination, inspection, monitoring, reporting, follow-up, and control of general facilities management and yard support activities within the Al Laith Group DIC Yard, ensuring that activities are planned, coordinated, monitored, and completed safely. The scope covers routine facilities inspection, maintenance coordination, defect reporting, housekeeping control, waste management coordination, contractor control, emergency access monitoring, welfare facility checks, and general support to yard operations. This Method Statement does not authorize the Facilities Coordinator or general yard personnel to physically carry out specialist or high-risk activities. Where such activities are required, they shall be assigned only to competent and authorized personnel and controlled through separate task-specific MSRA, permit-to-work, inspection records, certificates, and supervision as applicable. The objectives of this document are to: - Establish safe working procedures for general facilities management and yard support activities. - Define responsibilities for the Facilities / Operations Department, Facilities Coordinator, HSE Department, maintenance personnel, contractors, employees, and yard users. - Ensure hazards are identified, reported, controlled, and closed out in a timely manner. - Ensure maintenance and contractor activities are coordinated safely and do not create uncontrolled risk to yard users. - Maintain safe access routes, emergency exits, fire points, welfare areas, and general yard infrastructure. - Ensure that housekeeping, waste management, cleaning, and pest control activities are properly coordinated. - Ensure high-risk or specialist activities are not carried out unless covered by separate approved MSRA, permit-to-work, competent personnel, and required certification. - Apply the hierarchy of controls in accordance with ISO 45001 principles, prioritizing elimination, substitution, engineering controls, administrative controls, and PPE as the final line of defence. - Support safe, efficient, and compliant yard operations through inspection, reporting, coordination, and corrective action close-out. |  |  |  |  |  |
| # 4.0 Program |  |  |  |  |  |
| Facilities management and yard support activities shall be planned and carried out in accordance with daily operational requirements, approved work schedules, and site instructions. |  |  |  |  |  |

## 5.0 KEY RESOURCES

### 5.1 Roles, Responsibilities and Authority

Roles and responsibilities for general facilities management follow the Al Laith yard structure. All personnel shall comply with yard rules and the approved MS/RA.

#### Facilities Coordinator

Responsible for the overall planning, reporting, and coordination of yard support and facilities maintenance activities, including:

- Coordinate routine maintenance and facility upkeep tasks.

- Liaise between yard management and maintenance teams/contractors.

- Ensure general yard housekeeping and asset maintenance standards are met.

- Monitor the progress of facilities-related tasks and escalate issues as required.

- Maintain records of facilities inspections and routine yard support actions.

#### HSE Department

Responsible for health, safety, and environmental oversight, including:

- Conduct periodic safety audits and inspections of yard activities.

- Verify that all personnel have received the necessary safety inductions and briefings.

- Review and approve risk assessments and method statements for yard tasks.

- Provide guidance on PPE requirements and emergency procedures.

- Monitor compliance with local safety regulations and company H&S policies.

- Authority to stop any work deemed unsafe or not in compliance with the MSRA.

#### Maintenance Team / Approved Contractors

Responsible for the execution of technical and physical maintenance tasks, including:

- Carry out maintenance and support activities as per the approved method statement.

- Ensure all tools and equipment used are in good condition and certified where required.

- Maintain a clean and organized work area (housekeeping).

- Report any hazards or incidents encountered during work to the Coordinator/HSE.

- Utilize appropriate PPE at all times during the execution of tasks.

In addition to the general yard-support roles above, the following trade roles apply to the carpentry, AC, plumbing and lifting activities (7.10 - 7.13):

#### Carpentry Team Leader / Carpenters

Manage and carry out offloading, stock control, cutting, assembly, finishing, tool management and yard cabin / FlexiLoo works; ensure machine guards and PPE are used; only certified operators use the table saw and angle grinder.

#### AC Supervisor / AC Technicians

Service and repair split-unit AC; carry out refrigerant recovery / recharge in-house; apply electrical isolation / lock-out; control work at height via ladders; competency for refrigerant handling to be verified (see 5.2 gap).

#### Plumbing Supervisor / Plumbers

Pipe installation, fixture replacement, leak repair, pressure / leak testing; operate the vacuum tanker for external tank cleaning; ensure no tank entry occurs.

#### Lifting / Crane Supervisor

Supervises lifting operations on the ground; briefs personnel via toolbox talk; liaises with the Appointed / Competent Person on any material change to the lift.

#### Appointed / Competent Person (Lifting)

Plans crane lifts, prepares the lift plan / MS / RA, carries full responsibility for safe completion of the lift, and briefs the Crane Supervisor (BS 7121).

#### Slinger / Signaller (Banksman / Rigger)

In-house certified; selects, inspects and attaches lifting tackle and directs loads by agreed hand signals / radio; directs vehicle / plant reversing.

#### Forklift / Crane Operators

In-house certified and authorized; carry out daily pre-use checks; operate within SWL and manufacturer limits; respond to the signaller and to any stop signal.

Stop-Work Authority: the Site Supervisor and the HSE Officer are empowered to stop any activity considered unsafe or non-compliant with this MS / RA. Any worker may also stop work and raise a concern without reprisal.

#### Employees, Yard Users, and Visitors

General responsibilities for all personnel within the yard:

- Adhere to yard safety rules, signage, and designated pedestrian/vehicle routes.

- Wear the minimum required PPE (Hard Hat, Safety Boots, High-Vis) while in operational areas.

- Report any observed unsafe acts or conditions to yard supervision immediately.

- Cooperate with any emergency drills or evacuation instructions.

- Maintain a high standard of personal safety awareness and avoid entry into restricted zones.

Note on High-Risk Activities: Carpentry, AC repair & maintenance, plumbing / tank cleaning and lifting operations are covered under this Method Statement (Sections 7.10 - 7.13) when carried out by competent, authorized personnel. Excavation, confined space entry, MEWP / roof access, fire / gas-system impairment, scaffold and temporary-structure works, and non-carpentry hot works are NOT covered under the general scope of this Method Statement. These activities strictly require a separate task-specific Method Statement and Risk Assessment (MSRA), a valid Permit-to-Work (PTW), and must be executed only by competent, authorized personnel.

### 5.2 Selection of People (Competence)

Al Laith shall assign competent personnel based on training, experience, skills, site induction, task requirements, and medical fitness where applicable. Personnel involved in general facilities management and yard support activities shall be competent for the assigned activity, including facilities inspection, defect reporting, housekeeping control, contractor coordination, waste management coordination, emergency access monitoring, welfare facility checks, and general yard movement. Specialist activities such as scaffolding, lifting/rigging, banksman duties, plant operation, work at height, hot works, electrical works, excavation, confined space entry, MEWP operation, pressure system works, and fire system impairment shall only be carried out by trained, competent, and authorized personnel under separate task-specific MSRA and permit-to-work where required.

### 5.3 Worker Participation and Consultation

Worker participation is used to improve safety and performance:

- Daily TBTs capture hazards, site conditions and improvement suggestions.

- Feedback is recorded and reviewed by Yard Supervisor/Manager and HSE.

- Where improvements are required, controls shall be updated through MS/RA (MoC) and re-briefed.

- Safety concerns raised by workers shall be acted on immediately where they affect safe working.

| # 6.0 B Key Resource |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.1 | Controls |  |  | Control measures shall be selected and applied in accordance with ISO 45001 principles and the hierarchy of controls: Eliminate → Substitute → Engineering Controls → Administrative Controls → PPE. PPE shall be treated as the last line of defence and shall not be used as the only control where higher-level controls are reasonably practicable. The responsible Supervisor / Facilities Coordinator / Department Representative shall ensure that personnel assigned to facilities and yard support activities are competent for their assigned tasks. Competency requirements may include, as applicable: Site induction, Yard safety awareness, Traffic management awareness, Manual handling awareness, Fire safety and emergency response awareness, Housekeeping and waste management awareness, Contractor control awareness, Permit-to-work awareness, and Task-specific competency for maintenance personnel and approved contractors. Specialist works such as electrical repair, hot works, work at height, lifting operations, scaffold works, MEWP works, confined space entry, excavation, pest control treatment, fire system works, or other controlled activities shall only be undertaken by competent, trained, and authorized personnel under separate approved task-specific MSRA and permit-to-work where required. Evidence of competence shall be maintained in the Training Matrix, Training Register, contractor records, or competency file, as applicable. Where competence cannot be confirmed, the person shall not be assigned to that activity. Refresher training shall be provided in line with local requirements, company requirements, risk exposure, incident trends, or where deemed necessary by Al Laith Group. |  |  |  |  |  |  |  |  |  |  |
| 6.2 | Accidents & Incidents |  |  | All incidents, near misses, unsafe conditions, environmental spills, property damage, fire-related concerns, and facilities-related hazards shall be reported immediately to the responsible Supervisor, Facilities Coordinator, Manager, and HSE team. Initial reporting shall be made as soon as practicable and without delay. A formal incident or near-miss report shall be completed within 48 hours, or sooner where required by Al Laith Group internal procedures or the severity of the event. Examples of reportable issues include, but are not limited to: - Injury or illness. - Near miss. - Unsafe condition. - Damaged facilities or infrastructure. - Electrical defects. - Water leaks or flooding. - Fire safety concerns. - Blocked emergency exits or fire points. - Spillages. - Pest infestation. - Vehicle or plant-related incidents. - Contractor non-compliance. - Damage to property, equipment, doors, gates, fencing, lighting, welfare areas, or yard infrastructure. All incidents and near misses shall be investigated in accordance with company procedure, and corrective actions shall be identified, assigned, tracked, and closed out. |  |  |  |  |  |  |  |  |  |  |
| 6.3 | Corrective and Preventive Action — CAPA |  |  | The responsible Supervisor / Facilities Coordinator / Manager and HSE team shall ensure that corrective and preventive actions are assigned, tracked, verified, and closed out for all health, safety, fire, environmental, facilities, and housekeeping-related findings. Findings may arise from: Facilities inspections, Yard inspections, HSE inspections, Internal audits, Toolbox talks, Incident and near-miss reports, Contractor observations, Employee reports, Fire safety observations, Maintenance reports, Client or management observations. Findings shall be recorded in the Corrective Action Register / Preventive Action Register within 24 hours, where applicable. Each record shall include, as applicable: Action ID, Source of finding, Location, Description of issue, Risk level, Immediate control taken, Root cause or contributing factors, Required corrective action, Action owner, Target completion date, Current status, Evidence of completion, Verification comments, Closure sign-off. Actions shall be reviewed regularly by the responsible Manager / Supervisor and HSE team. Overdue or repeated actions shall be escalated to senior management. Closure shall require suitable verification evidence such as photographs, documents, inspection records, maintenance reports, service reports, or other completion evidence. Where practicable, closure verification shall be completed by a competent person who is independent from the person who carried out the corrective action. |  |  |  |  |  |  |  |  |  |  |
| 6.4 | Materials |  |  | Materials used or stored within the yard shall be suitable, safely stored, and controlled to prevent injury, obstruction, fire risk, damage, or environmental impact. Facilities and yard support materials may include, but are not limited to: - Cleaning materials - Waste bins and waste containers - Barricade tape, cones, and signage - Maintenance consumables - Office and welfare supplies - Replacement fixtures and fittings - Plumbing, lighting, HVAC, and general repair materials - Pest control materials used only by approved pest control service providers - Spare parts and minor maintenance materials - General yard support materials All materials shall be inspected visually before use where applicable. Damaged, expired, leaking, contaminated, unstable, or unsuitable materials shall not be used and shall be reported to the responsible person. Storage controls shall include: - Materials stored in designated areas only. - Walkways, access routes, fire points, emergency exits, and electrical panels kept clear at all times. - Materials stacked safely to prevent collapse or falling items. - Damaged materials segregated and clearly marked “NOT FOR USE” where applicable. - Chemicals stored in labelled containers with SDS available where required. - Waste stored only in designated waste areas. - Combustible materials kept away from ignition sources. - Storage areas inspected regularly. Unwanted or unfit materials removed from service and disposed of through approved channels. Scaffolding materials, lifting accessories, plant attachments, electrical components, and other specialist materials shall be controlled under the relevant department procedure, manufacturer requirements, inspection regime, and separate task-specific MSRA where applicable. - Trade materials also include the following. Carpentry: timber, plywood / sheet materials, fixings and hardware, adhesives and consumables. - AC: refrigerant gases in certified cylinders, coils, compressors, filters, capacitors, controls and copper pipe. - Plumbing: PVC / GI / copper pipes and fittings, valves, taps, fixtures, sealants, adhesives and gaskets. - Lifting: slings, shackles, chains and other lifting tackle (certified and inspected). - Chemicals: paint & thinners, chemical soap / degreaser, AC coil cleaner, and pipe sealants / adhesives (stored in the dedicated controlled storage area; SDS register to be established - see 7.12). |  |  |  |  |  |  |  |  |  |  |
| 6.5 | Other Essential Equipment: |  |  | Equipment required for facilities and yard support activities may include: - Inspection checklist. - Maintenance request tracker. - Mobile phone or tablet for reporting and photographs. - Two-way radio, where applicable. - Warning cones. - Barricade tape. - Temporary barriers. - Safety signage. - Wet floor signs. - Spill response materials, where available and applicable. - Basic hand tools used only by competent maintenance personnel. - Cleaning tools used only by assigned cleaning personnel. - Approved access equipment used only by trained and authorized personnel. - Fire safety and emergency equipment provided at designated locations. All equipment shall be suitable for the intended use, maintained in good condition, and inspected before use where applicable. Damaged, defective, or unsuitable equipment shall not be used. It shall be removed from service, reported, and replaced or repaired by competent personnel. Tools and equipment required for high-risk works such as hot works, electrical works, lifting operations, work at height, scaffold works, or MEWP works shall only be used by competent and authorized personnel under separate approved controls. |  |  |  |  |  |  |  |  |  |  |
| 6.6 | Plant, Equipment |  |  | Plant and equipment used within the yard shall be operated only by competent, trained, and authorized personnel. All drivers and plant operators shall comply with Al Laith Group alcohol and drug policies. No person shall operate vehicles, plant, tools, or equipment if under the influence of alcohol or drugs, unwell, fatigued, or taking medication that may impair performance. Supervisors shall intervene and reassign duties where impairment, unsafe behaviour, or lack of competence is suspected. General controls include: - Competent and authorized operators only. - Valid inspection and certification where required. - Daily pre-use checks completed and recorded where applicable. - Defects reported immediately. - Defective equipment isolated and removed from service. - No mobile phone use while driving or operating plant. - Seat belts used where fitted. - Speed limits followed. - Reversing controlled in accordance with yard traffic rules. - Exclusion zones established where plant movement may affect pedestrians. - Personnel kept out of the line of fire. - Plant movement coordinated with pedestrians, vehicles, storage areas, and work activities. - Loading and unloading activities controlled under separate approved arrangements where applicable. Facilities personnel shall not operate plant or equipment unless trained, competent, authorized, and assigned to do so. |  |  |  |  |  |  |  |  |  |  |
| 6.7 | Yard Traffic / Plant Movement Control |  |  | Traffic management shall follow Al Laith Group yard traffic rules, designated traffic routes, pedestrian walkways, crossings, speed limits, signage, and segregation arrangements. Controls shall include: - Use of designated pedestrian routes where available. - High-visibility vest worn in operational yard areas. - Vehicles and plant operated only by authorized personnel. - Note on Excluded Activities: Scaffolding, lifting, and work at height are strictly excluded from this scope. - Reversing minimized where practicable. - Safe parking in designated areas only. - No obstruction of emergency access routes, fire points, gates, or walkways. - Pedestrians kept clear of moving plant and vehicles. - Drivers and operators to remain alert to pedestrians, contractors, visitors, and other yard users. - Mobile phone use prohibited while driving or operating plant. - Poor visibility, blind spots, congested areas, or restricted access to be controlled before movement continues. A banksman / flagman shall be used where required, including when: - Reversing. - Visibility is restricted. - Access is tight or congested. - Movement is near pedestrians. - Movement is near storage areas or work zones. - Movement is near loading or unloading activity. - The operator requests assistance. - The supervisor identifies additional risk. Plant movement shall stop immediately if the banksman / flagman is not in position, communication is lost, pedestrians enter the exclusion zone, or the route is unsafe. |  |  |  |  |  |  |  |  |  |  |
| 6.8 | Identified Risks |  |  | Key risks associated with general facilities management and yard support activities include: - Slips, trips, and falls. - Vehicle and plant movement. - Pedestrian interaction with forklifts, trucks, and other mobile equipment. - Poor housekeeping. - Blocked walkways, emergency exits, fire points, or access routes. - Damaged flooring, potholes, uneven ground, or unstable surfaces. - Water leaks, wet floors, and drainage issues. - Electrical defects. - Poor lighting. - Fire hazards from poor storage, waste accumulation, or electrical faults. - Manual handling of minor items. - Contractor activities. - Cleaning chemical exposure. - Pest control activities. - Waste handling and waste accumulation. - Heat and humidity exposure during outdoor activities. - Falling or unstable stored materials. - Poor communication or incomplete handover. - Unauthorized or uncontrolled maintenance works. - Unknown or unreported unsafe conditions. High-risk activities such as work at height, lifting operations, scaffold works, hot works, electrical repairs, confined space entry, excavation, MEWP works, and pressure system works are not considered routine facilities activities under this general MSRA. These activities shall require separate task-specific MSRA, permit-to-work, competent personnel, and suitable supervision. Refer to the approved Risk Assessment for full details of hazards, persons at risk, control measures, and residual risk ratings. |  |  |  |  |  |  |  |  |  |  |
| 6.9 | Site Specific Rules & Restrictions |  |  | Site-specific rules and restrictions shall be communicated through site induction, toolbox talks, MSRA briefings, signage, supervision, and management instructions. All personnel shall comply with Al Laith Group DIC Yard rules, including but not limited to: - Follow designated pedestrian and vehicle routes. - Wear required PPE in operational areas. - Obey speed limits and traffic signage. - Do not obstruct emergency exits, fire points, access roads, gates, or walkways. - Report unsafe conditions, defects, spills, damage, or near misses immediately. - Do not enter restricted areas without authorization. - Do not operate plant, vehicles, or equipment unless authorized. - Do not carry out high-risk works without approved MSRA, permit-to-work, and competent personnel. - Maintain good housekeeping. - Store materials only in designated areas. - Follow waste management requirements. - Smoking is allowed only in designated areas, where applicable. - Contractors shall comply with approved work controls and site instructions. - Stop work immediately where an unsafe condition exists. Any breach of site rules shall be reported to the responsible Supervisor / Manager and may result in removal from the work area. |  |  |  |  |  |  |  |  |  |  |
| 6.10 | Housekeeping & Inspections |  |  | Good housekeeping and material management shall be maintained before, during, and after all facilities and yard support activities. Housekeeping controls shall include: - Keeping walkways, access routes, emergency exits, fire points, electrical panels, and welfare areas clear. - Removing waste from work areas and placing it in designated waste bins or collection points. - Preventing accumulation of combustible waste. - Cleaning spills immediately or isolating the area until cleaning is completed. - Using wet floor signs where required. - Keeping tools, materials, and equipment stored safely. - Segregating damaged or unfit materials. - Maintaining safe stacking and storage arrangements. - Reporting damaged flooring, potholes, leaks, poor lighting, pest issues, or unsafe conditions. - Ensuring work areas are left clean and safe after maintenance or contractor activity. Inspections shall be carried out by the responsible Supervisor, Facilities Coordinator, HSE team, or assigned competent person, as applicable. Inspection frequency may include: - Daily visual checks for active or high-use areas. - Weekly planned facilities inspections. - Post-maintenance or post-contractor work inspections. - Additional inspections following incidents, near misses, complaints, adverse weather, or changes in yard layout. Inspection findings shall be recorded, assigned to an action owner, tracked, verified, and closed out with suitable evidence. |  |  |  |  |  |  |  |  |  |  |

# 7.0 SEQUENCE OF OPERATION

This section defines the operational sequence and control measures for general facilities management and yard support activities within Al Laith Group Yard at Dubai Industrial City. The activities covered under this section include facilities inspections, defect reporting, maintenance coordination, housekeeping control, waste management coordination, contractor and service provider coordination, emergency access checks, welfare facility checks, temporary area control, and close-out of corrective actions.

All works shall be carried out in accordance with the approved Method Statement, Risk Assessment, Al Laith Group internal procedures, site rules, manufacturer guidance where applicable, permit-to-work requirements where applicable, and applicable UAE legal and HSE requirements.

Specialist or high-risk activities such as lifting operations, scaffold works, work at height, MEWP works, hot works, electrical repair, excavation, confined space entry, pressure system works, and fire system impairment are not covered as direct works under this general Method Statement. Such activities shall require separate task-specific MSRA, permit-to-work, competent personnel, and required approvals before commencement.

## Management of Change / Stop Work Rule

Work shall stop immediately where any change, unsafe condition, or uncontrolled hazard is identified. Examples include, but are not limited to: Change in work area or yard layout, Change in access routes, pedestrian routes, or traffic arrangements, Introduction of new contractor, service provider, plant, or equipment, Damaged infrastructure, exposed services, leaks, unstable storage, poor lighting, or blocked emergency routes, Conflict with other yard activities or simultaneous operations, Weather conditions affecting safe outdoor work, Discovery of electrical defects, fire hazards, spills, pest infestation, or structural damage, Any incident, near miss, property damage, complaint, or HSE observation, Any activity requiring permit-to-work or specialist assessment.

The affected area shall be made safe using suitable controls such as signage, cones, barricade tape, temporary barriers, isolation, or access restriction. The issue shall be escalated to the responsible Supervisor, Facilities Coordinator, Manager, or HSE representative before work resumes. Work shall only resume once the hazard has been assessed, suitable controls have been implemented, and approval has been given by the responsible person.

## Hold Point Release Authority

Hold points within this section shall be released by the responsible Supervisor, Facilities Coordinator, Manager, HSE representative, or competent person, depending on the nature of the activity. Where client, authority, or permit-to-work approval is required, work shall not proceed until the required approval is obtained.

## 7.1 Permits, Interfaces, Area Control, and Weather Considerations

### 7.1.1 Permit Requirements

A permit-to-work shall be obtained where required by Al Laith Group procedure, site rules, or the nature of the activity. Permits or specific authorisations may be required for activities such as:

- Hot work.

- Electrical isolation or electrical repair.

- Work at height.

- Roof access.

- MEWP works.

- Lifting operations.

- Confined space entry.

- Excavation.

- Fire system impairment.

- Pressure system works.

- Any non-routine high-risk activity.

Routine facilities inspection, reporting, housekeeping coordination, and general monitoring activities may not require a permit unless they interface with controlled or high-risk work.

### 7.1.2 Permit Display and Control

Where a permit-to-work is required, the live permit shall be available at the work area, access point, or supervisor file as required by the applicable procedure. The responsible Supervisor or permit holder shall ensure that permit conditions are communicated to all affected personnel.

### 7.1.3 Permit Change / Suspension

If permitted conditions change, work shall stop immediately. The permit shall be suspended, returned, revalidated, or reissued as required before work resumes. Personnel shall be re-briefed before restarting the activity.

### 7.1.4 Area Control, Segregation, and Protection of Others

Where facilities or maintenance activities may create risk to yard users, the affected area shall be controlled using suitable measures such as:

- Warning signage.

- Cones.

- Barricade tape.

- Temporary barriers.

- Wet floor signs.

- Controlled access.

- Communication to affected departments.

- Banksman or marshal where vehicle or plant interface exists.

Access routes, emergency exits, fire points, welfare facilities, pedestrian walkways, and operational areas shall not be blocked unless an approved temporary control arrangement is in place.

### 7.1.5 Coordination and Interface Management

Before facilities or maintenance activities begin, the responsible person shall confirm:

- Work area and access requirements.

- Impact on employees, contractors, visitors, drivers, and yard users.

- Pedestrian and vehicle segregation.

- Interface with ongoing yard activities.

- Emergency routes and fire points remain accessible.

- Welfare facilities remain safe and usable.

- Required signage, barriers, and communication are in place.

- Any permit-to-work requirement has been identified.

Any unsafe interface or conflict with other yard activities shall result in stop work and re-planning.

### 7.1.6 Weather Considerations

Weather conditions shall be considered for outdoor facilities inspections, maintenance coordination, cleaning, waste collection, and contractor activities. Outdoor activities shall be postponed, stopped, or reassessed where weather conditions create unsafe conditions, including:

Heavy rain causing slippery surfaces or flooding.

- Strong wind affecting loose materials, signage, temporary barriers, or safe movement.

- Lightning.

- Poor visibility.

- Extreme heat or humidity.

- Standing water affecting pedestrian or vehicle routes.

Additional controls shall be implemented where required, including restricting access, delaying work, increasing supervision, providing hydration, or escalating to HSE or management.

## 7.2 PRE-JOB PLANNING & START CRITERIA

Before facilities or yard support activities commence, the responsible Supervisor / Facilities Coordinator / Department Representative shall ensure:

- Personnel are briefed on the planned activity.

- Required PPE is available and worn.

- Work area is checked for visible hazards.

- Any affected department or yard user has been informed.

- Required permits or authorisations are obtained where applicable.

- Competent personnel or approved contractors are assigned where maintenance work is required.

- Tools and equipment are suitable and inspected where applicable.

- Signage, cones, barricades, or temporary barriers are available where required.

- Emergency routes, fire points, and pedestrian access are not blocked.

- Defects or unsafe conditions are recorded and escalated.

- Contractor documents, permits, certificates, and MSRA are checked where applicable.

- Toolbox talk or task briefing is completed where required.

Start Hold Point
Work shall begin only when the work area is safe, personnel are briefed, required controls are in place, and the responsible person authorises the activity to proceed.

## 7.3 TRAFFIC MANAGEMENT, DELIVERIES & VEHICLE MOVEMENT

Traffic management shall follow Al Laith Group yard traffic rules, designated traffic routes, pedestrian walkways, speed limits, signage, and segregation arrangements. Controls include:

- Authorized drivers and operators only.

- Site speed limits shall be followed.

- Pedestrian walkways and vehicle routes shall be respected.

- High-visibility vest shall be worn in operational yard areas.

- No mobile phone use while driving or operating plant.

- Reversing shall be minimized where practicable.

- Banksman / flagman shall be used where visibility is restricted, access is congested, or pedestrians may be affected.

- Vehicles shall park only in designated areas.

- Emergency access routes, fire points, gates, and walkways shall not be obstructed.

- Unsafe route conditions such as standing water, potholes, unstable ground, poor visibility, or congestion shall be reported and controlled.

- Deliveries shall report to the responsible yard control point or designated receiving area.

- Drivers and visitors shall be briefed on yard safety requirements where applicable.

- Spill response arrangements shall be available where vehicle movement or deliveries may create environmental risk.

Facilities personnel shall not direct complex vehicle or plant movements unless trained and authorized. Where specialist movement, lifting, or unloading is required, separate approved controls shall apply.

## 7.4 MATERIALS HANDLING, STORAGE, AND STACKING

Materials shall be handled and stored in a controlled manner to prevent injury, obstruction, fire risk, material damage, or environmental impact.

Controls include:

- Materials shall be stored only in designated areas.

- Walkways, access routes, emergency exits, fire points, gates, and electrical panels shall be kept clear.

- Storage areas shall be kept tidy and free from trip hazards.

- Materials shall be stacked on firm and level ground.

- Stacking height shall be controlled to prevent collapse.

- Long, round, or unstable items shall be restrained where required.

- Damaged, unsuitable, leaking, contaminated, or unsafe materials shall be segregated and reported.

- Combustible materials shall be kept away from ignition sources.

- Chemicals shall be labelled, stored correctly, and supported by SDS where required.

- Waste shall be placed only in designated waste areas.

- Heavy or awkward items shall not be manually handled without assistance or mechanical aid.

- Unfit materials shall be removed from service and disposed of through approved channels.

Scaffolding materials, lifting accessories, electrical parts, plant attachments, and other specialist materials shall be controlled under the relevant department procedure, inspection requirements, and separate task-specific MSRA where applicable.

## 7.5 MANUAL HANDLING

Manual handling shall be avoided or reduced where practicable by using mechanical aids, trolleys, team lifting, or assistance from competent personnel. Where manual handling is required:

- Assess the load before lifting.

- Consider weight, shape, size, grip, stability, and travel route.

- Use team lifting for long, heavy, or awkward items.

- Keep the load close to the body.

- Avoid twisting while carrying.

- Turn using the feet.

- Keep the route clear before movement.

- Do not carry loads that block visibility.

- Do not lift beyond personal capability.

- Stop and seek assistance if the load is unstable, too heavy, awkward, or the route is unsafe.

Manual handling of heavy equipment, plant attachments, scaffold materials, or other bulky items shall only be carried out under suitable task-specific controls.

## 7.6 Hand Tools, Power Tools, and Minor Maintenance Equipment

Hand tools, power tools, and minor maintenance equipment shall only be used by competent and authorized personnel.

Controls include:

- Tools shall be suitable for the task.

- Tools shall be inspected before use.

- Defective tools shall be removed from service and reported.

- Guards and safety devices shall be fitted and functional where applicable.

- Electrical tools shall be used with suitable protection such as RCD where required.

- Cables shall be routed to prevent trip hazards and damage.

- Tools shall not be used in wet conditions unless designed and approved for such use.

- Dust, noise, sparks, and flying particles shall be controlled.

- Correct PPE shall be worn.

- No temporary repairs shall be made to faulty tools.

- Tools shall be stored safely after use.

Hot works, grinding, cutting, electrical works, and other higher-risk tool use shall require separate approved controls and permit-to-work where applicable.

## 7.7 INSPECTION, REPORTING, AND CLOSE-OUT

Facilities inspections shall be conducted across all yard areas to identify defects, safety concerns, and maintenance requirements. Inspection areas include:

- Offices, Welfare facilities, and Toilets.

- Workshops, Stores, and Storage areas.

- Yard access routes, Pedestrian walkways, and Parking areas.

- Waste collection points, Fire points, and Emergency exits.

- Lighting, Drainage, Doors, gates, fencing, and locks.

- Signage and General housekeeping.

Defect Identification and Close-Out Process:

- 1. Record exact location of the defect or hazard.

- 2. Take photographs as evidence.

- 3. Apply temporary controls (signage, barriers) where risk exists.

- 4. Report finding to the Facilities Coordinator / Supervisor.

- 5. Assign corrective action to a competent person/contractor.

- 6. Track progress in the CAPA or maintenance register.

- 7. Verify safety and quality of completed work.

- 8. Close the action in the register with verification evidence.

End-of-Activity / Shift Checklist:

- Remove waste and debris from the work area.

- Secure tools and equipment in designated storage.

- Confirm all emergency routes and fire points remain clear.

- Communicate outstanding actions or handover items to next shift.

- Update facilities records and maintenance logs.

- Verify completed works meet required standards.

## 7.10 Carpentry Works

Carpentry is carried out in the DIC yard workshop and, where required, at yard locations for cabin / portacabin repairs and FlexiLoo build and maintenance. Machinery is predominantly handheld (handheld saws, sanders, hand routers, drills and other handheld power tools) with ONE stationary / table saw as the only non-handheld machine. All electrical tools are inspected and, where required, third-party certified.

### 7.10.1 Offloading and goods-in

- Inspect incoming timber, plywood / sheet and hardware for quality and record deliveries against orders.

- Use a forklift (competent operator, banksman where required) to offload heavy items; segregate into designated storage zones.

### 7.10.2 Stock segregation and storage

- Store by size / type / usage with clear labelling and FIFO rotation.

- Timber stacked horizontally on racks with battens for air circulation; plywood / sheets stored vertically with edge protection; items kept off the ground on racks / pallets; heavy low and light on top; rack limits observed.

### 7.10.3 Task allocation and preparation

- The Supervisor assigns tasks verbally at the daily toolbox talk (no formal work-order system); crew split into cutting / assembly / finishing / logistics as needed.

- Pre-cut timber / plywood using workshop machinery; inspect pre-cut components for quality; partial assembly where it speeds installation.

### 7.10.4 Machinery and power-tool use

- Only trained, competent operators use machinery. The single stationary / table saw shall have its guard, riving knife and emergency stop functional and be used per the manufacturer manual with push-sticks as required.

- Handheld saws, sanders, routers and drills are inspected before use; guards fitted; RCD protection; on-tool / local dust extraction used where available; hearing protection and dust mask worn; no loose clothing or jewellery.

- Angle grinder use requires specific angle-grinder operator certification - only certified operators may use angle grinders. Correct disc for the material; guard in place; face shield plus eye and hearing protection; no grinding near combustibles / dust without hot-work controls.

### 7.10.5 Tool management

- Daily tool inspection; faulty tools tagged "NOT FOR USE" and removed; check-in / check-out tracking; tools stored in racks / cabinets with power tools in padded slots.

### 7.10.6 Yard tasks - cabin / portacabin repairs and FlexiLoo build and maintenance

- Carried out in a designated, segregated yard area; manual-handling controls applied; any work at height via ladder only where unavoidable and controlled (see 7.11.2); housekeeping maintained throughout.

### 7.10.7 Environmental and waste practices

- Finished components stored separately and covered; wood scraps and packaging segregated and recycled where possible; cutting patterns optimized to reduce off-cuts.

- Dust extraction used and dust disposed of responsibly; end-of-day cleaning by vacuum / damp methods (no dry sweeping).

## 7.11 AC Repair & Maintenance (Split Units Only)

AC works cover split units only (no ducted, package or chiller systems). Scope includes routine servicing and major repairs including compressor-level work. Refrigerant recovery, recharge and handling are carried out in-house by Al Laith technicians. Height access is via ladders.

Competency gap (F-Gas): F-Gas / refrigerant-handling certification status is not confirmed. This is an open competency gap to be formalized. Refrigerant handling shall only be performed by personnel whose competence has been verified; obtaining and recording formal F-Gas / refrigerant certification is an open action.

### 7.11.1 Preparation and isolation

- Inspect indoor / outdoor split units, refrigerant lines and electrical connections.

- Electrically isolate the unit and apply lock-out / tag-out before work; verify isolation; discharge capacitors before handling controls.

### 7.11.2 Access at height (ladders)

- Ladders inspected before use and set on firm, level ground at the correct angle, secured or footed; maintain three points of contact; no overreaching.

- Where the fall risk warrants and a suitable anchor exists, a harness is used (as required); higher-level access (MEWP / roof) is out of scope - see Section 8.

### 7.11.3 Servicing

- Clean filters, condenser and evaporator coils using approved AC coil cleaner (chemical controls per 7.12); check and clear condensate drainage; inspect contactors, relays and capacitors.

### 7.11.4 Refrigerant recovery / recharge (in-house)

- Recover refrigerant into certified cylinders using the in-house recovery unit before opening the system - no venting of refrigerant to atmosphere.

- Leak-test, evacuate / vacuum, then recharge to manufacturer specification; handle and store cylinders upright and secured.

### 7.11.5 Major repairs (including compressor)

- Compressor and component-level repairs carried out by competent technicians.

- Where brazing / soldering is required this constitutes HOT WORK and shall be controlled under a hot-work permit with a fire watch and extinguisher available (see Sections 8 and 17).

### 7.11.6 Testing and handover

- Test cooling performance, controls and safety devices; restore power only after checks are complete; record the works.

## 7.12 Plumbing & Waste Tank Cleaning (incl. Chemicals)

Plumbing tasks include pipe installation, fixture replacement and leak repair. Waste tank cleaning covers Portaloo / portable toilet tanks, septic tanks and general holding / water tanks (grease traps are NOT in scope). Tank cleaning uses an in-house vacuum tanker owned and operated by Al Laith.

HARD RULE - NO CONFINED SPACE ENTRY: Tank cleaning is always performed externally via hose / pump from outside the tank. No person shall enter any tank under this Method Statement. Any future requirement for entry would require a separate confined-space entry permit and specialist controls and is out of scope of this document.

PPE: Standard PPE only (gloves, boots, hi-vis, eye protection). No specialist respiratory or chemical suits are currently held; if a task assessment identifies a need for respiratory / chemical protection, work stops and is re-planned.

### 7.12.1 Plumbing - pipe / fixture / leak work

- Inspect pipelines, valves, fittings and fixtures; isolate and shut down the water supply before work.

- Drain residual water safely (pumps where required); cut, replace or repair pipes / fixtures with correct tools; carry out pressure and leak testing; flush, restore supply and confirm zero leaks before handover.

### 7.12.2 Waste tank cleaning - external vacuum method

- Position the vacuum tanker on firm, level ground with wheel chocks; establish an exclusion zone with signage.

- Connect the suction hose externally and vacuum the contents into the tanker - never enter the tank; rinse / flush externally as required.

- Disconnect and secure hoses; clean up any spillage with a spill kit; decontaminate equipment after use.

### 7.12.3 Waste disposal

- Collected waste is disposed of through a licensed waste subcontractor; waste transfer notes / manifests are obtained and retained; disposal only at approved facilities.

### 7.12.4 Chemicals

Chemicals used across the yard trades: paint & thinners, chemical soap / degreaser, AC coil cleaner, and pipe sealants / adhesives. Stored in a dedicated, controlled storage area; disposal via the licensed waste contractor.

GAP - SDS register: Safety Data Sheets are not currently formalized. Control measure: establish and maintain an SDS register for all chemicals used, accessible at point of use. Until formalized, chemicals are used strictly per label instructions with appropriate PPE and ventilation.

- Keep in labelled containers; no decanting into unlabelled containers; flammable paint / thinners kept away from ignition sources.

- Ventilation provided; correct PPE (chemical-resistant gloves, eye protection); spill kit available; controls briefed at the daily TBT.

## 7.13 Lifting Operations

Lifting operations in the yard cover three modes, all applicable day-to-day: (1) forklift operations, (2) manual handling (see 7.5 and Section 14), and (3) mobile crane hire. Forklifts are owned and operated in-house; riggers, slingers and banksmen are in-house certified staff. Typical loads include stock / materials, cabins / structures and event structures. Mobile crane lifting is carried out in compliance with BS 7121-1:2006, BS 7121-3:2000 and LOLER 1998.

### 7.13.1 Forklift operations

- Only trained, certified, authorized operators; daily pre-use checks recorded; SWL not exceeded; loads secured and mast tilted back.

- Banksman for reversing / restricted visibility; pedestrians kept clear; travel with forks low; no passengers or riders on forks.

### 7.13.2 Roles for crane lifting operations (BS 7121)

- Appointed / Competent Person - plans the lift, prepares the lift plan / method statement / risk assessment, carries full responsibility for safe completion, and briefs the Crane Supervisor.

- Crane Supervisor - ensures all personnel are briefed via toolbox talk before work, supervises the lift on the ground, and liaises with the AP on any material change to site conditions / method.

- Slinger / Signaller - selects, inspects and attaches lifting tackle and directs the load by agreed hand signals or radio.

- Crane Operator - fully trained and competent on the machine, operates per the manufacturer manual, and responds only to the appointed signaller (and to a stop signal from anyone).

### 7.13.3 Certification, lifting tackle and PPE

- Statutory test certificates and inspection reports are available before the lift.

- The Crane Supervisor inspects all lifting tackle (slings, shackles, chains) before each use; damaged tackle is marked "NOT FOR USE" and returned to the yard for disposal; nylon slings are protected from sharp edges with sling guards or burlap.

- Standard PPE (hard hat, safety boots, hi-vis) plus harness and eye protection as required.

### 7.13.4 General conditions

- The project manager / site provides item weights, dimensions and lifting-point / slinging requirements and is responsible for load integrity.

- Ensures firm, level ground capable of the imposed loads; safe access / egress; traffic management in place before positioning; the area barriered and non-essential personnel excluded.

- The crane is left in a safe condition if unattended. A suitable alternative crane / tackle may be substituted only with AP / Crane-Supervisor agreement and without infringing this Method Statement.

### 7.13.5 Weather / wind limits

- Maximum wind speed for general lifting operations is 11 m/s (24.6 mph). For man-riding duties the maximum reduces to 7 m/s (16 mph).

- Local conditions and personnel safety may require the Crane Supervisor / Operator to suspend at lower wind speeds. Lifting stops during storms or any risk of lightning. A weather forecast is obtained for the lift.

### 7.13.6 Ground conditions

- Visual inspection (and client ground-survey report where provided); suitable outrigger mats / plates supplied where required; the client / site advises of any underground services, voids or vaults per BS 7121.

### 7.13.7 Load and radius control (lift-plan framework - complete per lift)

Gross lift weight = load + hook block + lifting tackle (+ fly / jib where applicable). The crane is selected so its SWL at the required radius provides an adequate safety margin (minimum 1.33). The centre of gravity is considered; manufactured lifting points are certified / tested or an AP-agreed slinging method is used. The values below are the framework carried over from the source lift plan; indicative values are shown and shall be recalculated and confirmed for each lifting operation.

Calculated maximum lift weights (indicative - confirm per lift):

- Weight of load: max 2.4 t.

- Weight of hook block: 0.4 t.

- Weight of lifting tackle: 0.2 t.

- Weight of fly / S-lift on boom: N/A.

- GROSS LIFT WEIGHT: 3.0 t.

- Maximum required radius: 30 m.

Crane information (indicative - confirm per lift):

- Crane (make / model): e.g. XCMG mobile crane.

- Capacity: e.g. 130 t.

- Boom length required: e.g. 38 m.

- SWL of crane at required radius: up to 75% utilisation.

- Safety margin: minimum 1.33.

### 7.13.8 Crane set-up procedure (BS 7121)

- 1. The crane and its operating crew arrive on site at the agreed date and time.

- 2. The Crane Supervisor meets the site manager and conducts the toolbox talk with all personnel involved; all sign the MS register; site inductions are completed as required.

- 3. Any traffic management, ancillary plans, mats or temporary roadways are set up under the Crane Supervisor instruction before the crane manoeuvres.

- 4. The crane manoeuvres into position as shown on the drawing, under the guidance of a member of the lifting crew.

- 5. The crane commences set-up and rigs to the required configuration; personnel assisting are under the crane operator guidance; set-up is per the manufacturer manual.

- 6. The operator carries out procedural safety and computer checks, implementing operational requirements (slewing restrictions, short-rig or fly-jib configurations).

- 7. The Crane Supervisor carries out a communications check between the operator and the lifting crew.

- 8. The hook block is lowered and the required lifting tackle placed on the hook; the lift supervisor confirms the correct tackle is selected.

- 9. The operator hoists the tackle clear of obstructions and positions over the first lift; the lift proceeds under the signaller direction, the load kept clear of personnel, with no one beneath a suspended load.

# 8.0 SPECIALIST AND HIGH-RISK ACTIVITIES

Covered under this Method Statement. Carpentry (7.10), AC repair & maintenance of split units (7.11), plumbing & waste tank cleaning (7.12) and lifting operations - forklift and mobile crane (7.13) - ARE authorized under this Method Statement when carried out by competent, authorized personnel with the competencies, controls and (where applicable) permits described in Section 7.

Still out of scope - the following require a separate task-specific MSRA, permit-to-work and competent personnel before commencement:

- Excavation.

- Confined space entry - including any entry into tanks (see 7.12; entry is never performed under this MS).

- Fire detection / suppression or gas-system isolation or impairment.

- Hot works other than the controlled AC brazing / soldering in 7.11 and incidental carpentry cutting - any welding, cutting or brazing is controlled under a hot-work permit; non-carpentry hot works are out of scope.

- Scaffold erection / alteration / dismantling and temporary-structure works.

- MEWP operation and roof access.

- Pressure-system works beyond routine AC / plumbing testing.

- Live electrical work beyond isolation / lock-out for AC servicing (licensed electrical works remain separately controlled).

- Any activity requiring third-party certification, statutory approval or licensed-trade competency not held.

Before any out-of-scope activity begins, confirm: approved task-specific MS / RA; permit-to-work; competent and authorized personnel; certificates / inspection records; supervision; PPE and emergency arrangements; and segregation / exclusion zones.

| # 9.0 Support Work & Storage Arrangement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9.1 | Temporary Supports and Props needed to facilitate the works |  |  | Temporary supports, props, barriers, or access restrictions may be required where facilities defects, maintenance works, damaged infrastructure, unsafe storage, or temporary hazards are identified. Examples may include: - Temporary barricades around damaged flooring, potholes, wet areas, leaks, or unsafe surfaces. - Warning cones and signage around maintenance or cleaning areas. - Temporary barriers to prevent access to unsafe or restricted zones. - Temporary covers or protection for minor openings, damaged surfaces, or exposed areas, where suitable and safe. - Temporary supports or protection for damaged doors, gates, fencing, fixtures, or fittings pending repair. - Temporary segregation between pedestrians, vehicles, contractors, and maintenance work areas. Any temporary support or protection shall be suitable for the hazard being controlled and shall not create additional risk. Temporary supports or props shall not be used for structural support, lifting support, scaffold support, plant support, or any engineered load-bearing purpose unless designed, checked, and approved by a competent person. Where structural instability, collapse risk, damaged load-bearing elements, unsupported services, or any engineered support requirement is identified, the area shall be isolated immediately and escalated to the responsible Manager, HSE team, and competent technical person. No person shall remove, alter, or bypass temporary controls unless authorized and the area has been confirmed safe. |  |  |  |  |  |  |  |  |  |  |
| 9.2 | Storage Arrangements |  |  | Materials, tools, equipment, waste, and maintenance items shall be stored in designated zones within the yard to prevent instability, obstruction, fire risk, damage, environmental impact, or interference with yard operations. Storage requirements shall include: Scaffold materials, lifting accessories, plant attachments, structural components, electrical components, and other specialist materials shall be stored and controlled under the relevant department procedure, inspection requirements, and task-specific MSRA where applicable. - Materials shall be stored only in approved or designated areas. - Pedestrian routes, vehicle routes, access roads, emergency exits, fire points, gates, welfare areas, and electrical panels shall be kept clear at all times. - Materials shall be stacked on firm, level ground. - Stacking height shall be controlled to prevent collapse or falling materials. - Long, round, loose, or unstable items shall be restrained where required. - Damaged, leaking, expired, contaminated, or unsuitable materials shall be segregated and reported. - Items not fit for use shall be marked “NOT FOR USE” where applicable. - Combustible materials shall be kept away from ignition sources. - Chemicals shall be stored in approved, labelled containers with Safety Data Sheets available where required. - Hazardous materials shall be stored in accordance with manufacturer instructions, SDS requirements, and Al Laith Group procedures. - Waste shall be stored only in designated waste collection areas. - Storage areas shall be inspected regularly for housekeeping, stability, access, fire safety, and environmental compliance. - Materials shall not be stored in a manner that obstructs visibility, traffic movement, emergency response, or safe access. - Storage arrangements shall be reviewed where yard layout, operational activities, or traffic routes change. |  |  |  |  |  |  |  |  |  |  |
| 9.3 | Permit to Work |  |  | PTWs are issued by the Client HSE/Authoriser and accepted by the Al Laith Supervisor. Validity and any extensions are stated on the permit. Signed PTWs are displayed at the workface, and originals are filed in the PTW Register. |  |  |  |  |  |  |  |  |  |  |
| 9.4 | SWL’s |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Permit to Work (PTW) Workflow & Control Sheet |  |  |  |
| --- | --- | --- | --- |
| Step | Activity | Responsible | Evidence / Record |
| 1. Identify Requirement | Identify whether the activity requires permit control, such as hot work, electrical isolation, work at height, roof access, MEWP works, lifting, confined space, excavation, fire system impairment, or other non-routine high-risk activity. | Supervisor / Facilities Coordinator / HSE / Department Representative | Permit requirement noted in maintenance request, work plan, or supervisor record |
| 2. Prepare Request | Prepare the permit request with relevant information such as work description, location, risk assessment, method statement, contractor documents, certificates, isolation requirements, and affected areas. | Supervisor / Department Representative / Contractor | Permit request pack |
| 3. Review and Issue | Authorized issuer reviews the work area, controls, interfaces, competence, and supporting documents before issuing the permit with conditions and validity period. | Authorized Permit Issuer / HSE / Responsible Manager | Issued permit-to-work with conditions and validity |
| 4. Accept and Brief | Permit holder accepts the permit, communicates permit conditions, briefs affected personnel, and confirms required controls are in place before work starts. | Permit Holder / Supervisor / Contractor Supervisor | Accepted permit, toolbox talk record, briefing attendance |
| 5. Display and Monitor | Live permit is displayed or made available as required. Work is monitored to ensure permit conditions, MSRA controls, barricading, access controls, and housekeeping are maintained. | Permit Holder / Supervisor / HSE where required | Permit display, supervisor log, inspection record |
| 6. Stop or Suspend | Work stops immediately if scope, method, location, weather, access, interface, personnel, or control conditions change. Permit is suspended, returned, revalidated, or reissued before restart. | Permit Holder / Supervisor / Authorized Issuer | Suspension record, revalidation record, re-brief record |
| 7. Extend if Required | Permit extension is requested before expiry. Authorized issuer confirms that conditions remain valid before extension or reissue. | Permit Holder / Authorized Issuer | Permit extension or reissued permit |
| 8. Close Out / Hand Back | On completion, the area is inspected, made safe, temporary controls are removed or adjusted as authorized, and the permit is closed or handed back. Outstanding hazards are communicated. | Permit Holder / Supervisor / Authorized Issuer | Permit close-out, hand-back record, completion evidence |

| # 10.0 Safety Requirements |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Safety Gears Required | Yes | No | Safety Equipment Required | Yes | No |
| Safety Helmet (Hard Hat) | AR |  | Safety Gloves – Cloth or leather | √ |  |
| Hearing Protection | AR |  | Safety Footwear – Boots | √ |  |
| Safety Glasses | √ |  | Dust Mask or Face Mask | √ |  |
| Coveralls | √ |  | Full Body Harness (Double Hook) > 1.8 meters | √ |  |
| High Visibility Vest – Orange for banksman and/or signallers | √ |  | Inertia Reel/Retractable | √ |  |
| Life Jackets | N/A |  | Life Line | AR |  |

√ = Required, R = Recommended, AR = As Required, N/A = Not Applicable; hearing protection selection will follow measured/expected noise levels and posted area signage.

| 10.1 | Specific Identified Residual Hazards |  |  | Refer to Risk assessment |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10.2 Required Personal Protective Equipment (select all applicable) In addition to the PPE grid above, the following task-specific PPE is provided as required per the Risk Assessment: dust mask / respiratory protection (carpentry and tank work); hearing protection (power tools, saw, grinder); face shield (angle grinder / cutting); cut-resistant gloves (carpentry); chemical-resistant gloves (chemicals and tank work); full-body harness (ladder work at height above 1.8 m); and welding / hot-work PPE (AC brazing). PPE remains the last line of defence in the hierarchy of controls. |  |  |  | Safety Boot |  |  |  |  | Yes | Hard Hats |  |  |  |  | Yes |
|  |  |  |  |  |  |  |  |  | No |  |  |  |  |  | No |
|  |  |  |  | Eye Protection |  |  |  |  | Yes | Respiratory Protection |  |  |  |  | Yes |
|  |  |  |  |  |  |  |  |  | No |  |  |  |  |  | No |
|  |  |  |  | Safety Gloves |  |  |  |  | Yes | Hearing Protection |  |  |  |  | AR |
|  |  |  |  |  |  |  |  |  | No |  |  |  |  |  | No |
|  |  |  |  | His-Vis Clothings |  |  |  |  | Yes | Harness / lanyard |  |  |  |  | AR |
|  |  |  |  |  |  |  |  |  | No |  |  |  |  |  | No |
|  |  |  |  | Coveralls |  |  |  |  | Yes |  |  |  |  |  | AR |
|  |  |  |  |  |  |  |  |  | No |  |  |  |  |  | No |
| 10.3 | Hazardous Substances (select all applicable) |  |  | Very Toxic |  |  |  |  | Yes | Harmful / Irritant |  |  |  |  | Yes |
|  |  |  |  |  |  |  |  |  | No |  |  |  |  |  | No |
|  |  |  |  | Corrosive |  |  |  |  | Yes | Dangerous for the environment |  |  |  |  | Yes |
|  |  |  |  |  |  |  |  |  | No |  |  |  |  |  | No |
|  |  |  |  | Oxidizing |  |  |  |  | Yes | Highly flammable |  |  |  |  | Yes |
|  |  |  |  |  |  |  |  |  | No |  |  |  |  |  | No |
|  |  |  |  | Explosives |  |  |  |  | Yes | Anything else not listed above |  |  |  |  | Yes |
|  |  |  |  |  |  |  |  |  | No |  |  |  |  |  | No |
| 10.4 | Hot Weather and Heat Stress Controls - Follow the mandatory midday break and any Client/authority heat-stress rules. Plan heavy work for cooler periods where practicable and limit non-essential outdoor tasks during peak heat. - Provide cool potable drinking water at the workface and encourage frequent drinking. Provide shaded or air-conditioned rest areas. - Supervisors to monitor heat stress indicators during pre-start and mid-shift checks and record in the Daily Brief/TBT (and Heat Stress log where used). - Acclimatisation: new/returning workers shall be gradually introduced to hot work and monitored closely. Use buddy checks. - Sun protection: long sleeves where practicable, hat/helmet shade where permitted, and SPF-rated sun protection on exposed skin. - Heat illness trigger: cramps, dizziness, confusion, fainting, or hot dry skin → STOP, move to shade/cool area, cool and hydrate, call First Aider/medical support as required, and record the event. Records: Midday-break compliance, water/rest provision, and any heat-related interventions recorded in Daily Brief/TBT and Heat Stress log (if used). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10.5 | Residual Risk Monitoring - The PM/Supervisor shall complete the Daily Residual Risk Checklist before work starts and after major changes (weather, layout, method, interfaces). Any new controls or pauses shall be recorded and briefed. Minimum check items: - Specialist/High-Risk Coordination: Monitor that separate task-specific MSRA and PTW controls are in place for any high-risk activity (e.g., WAH, lifting) occurring within the yard area. - Traffic interface: plant–pedestrian conflict during deliveries/offloading; banksman, exclusion zones, and TMP controls in place. - Adverse weather: wind/rain/heat affecting stability and access; apply wind limits, stop during lightning/heavy rain/unsafe visibility, enforce heat controls. Drift control: If residual risk increases (controls not effective, near miss, unsafe condition), take immediate corrective action and update/re-brief at TBT. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10.6 | # Night Work # Night work shall only proceed with adequate supervision, lighting, segregation, and communications. # Chargehand/Supervisor conducts regular rounds and records checks in the Night Work Supervision Log (lighting, exclusion zones, housekeeping, worker alertness). # Provide task and route lighting to maintain safe visibility; check emergency lighting at shift start where required. # Reflective PPE required. Maintain barriers/signage and apply site speed limits and banksman control for any vehicle movement. # Fit-for-duty checks at start and mid-shift. Fatigue indicators → reassign task, increase breaks, or stand-down and record. # No lone working for high-risk tasks (WAH, lifting, work near/over water, confined access areas). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10.8 | # Simultaneous Operations Where multiple activities occur simultaneously within the yard, supervisors shall coordinate work to prevent conflicts between lifting operations, scaffold erection and vehicle movement. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| # 11.0 Quality Monitoring |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Detailed Inspection & Test plan shall be submitted along with each method statement. |  |  |  |  |  |

| Activity | Who checks | Verification |
| --- | --- | --- |

| # 11.1 Daily Monitoring Checklist |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The Site Supervisor will complete a Residual Risk Daily Checklist for the following: manual handling, scaffold modification/dismantling, hot weather, night work, live fueling/plant, and vehicle/pedestrian interface. Any “No” triggers immediate corrective action and re-brief. File with the Daily Briefing Form. |  |  |  |  |  |

| # 12.0 Emergency Procedures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12.1 |  |  |  | Name of On-Site First Aider: |  |  |  |  |  | (Complete at prestart ) Name: Phone/radio: |  |  |  |  |
|  |  |  |  | First Aid Room Location: |  |  |  |  |  | First Aid Cabin - Located next to Security Gate 1 (Truck Entrance). |  |  |  |  |
|  |  |  |  | Location of Nearest Hospital: |  |  |  |  |  | Cedars Jebel Ali International Hospital, 04-8814000 OR Advanced Care Medical Centre, +97143397664 |  |  |  |  |
| 12.2 | Muster Point: |  |  | The designated Muster Points for the yard are located at the Gate 1 and Gate 2. In the event of an evacuation alarm, all personnel must stop work, safely secure their equipment, and proceed immediately via the designated pedestrian walkways to the nearest Muster Point. |  |  |  |  |  |  |  |  |  |  |
| 12.3 | Welfare Requirements |  |  | Provide potable water, shade/rest (hot-weather), sanitary facilities, and lighting for night work as applicable. |  |  |  |  |  |  |  |  |  |  |
| 12.4 | Emergency Arrangements |  |  | Emergency arrangements within the yard include: - first aid facilities - fire extinguishers in workshop areas - emergency evacuation routes - emergency contact numbers displayed All personnel must report incidents immediately to the supervisor. |  |  |  |  |  |  |  |  |  |  |
| 12.5 | Emergency Numbers |  |  | 998 (Ambulance), 997 (Fire) |  |  |  |  |  |  |  |  |  |  |
| 12.6 | Emergency Procedures |  |  | In case of emergency Al Laith staff operatives & Sub contractors follow the action plan as mentioned in the site Health & Safety plan. |  |  |  |  |  |  |  |  |  |  |

| # 13.0 Access & Egress Points |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| - Access and egress routes shall follow the approved site layout / traffic management plan (Ref. 2.2, where applicable). - The Project Manager / Supervisor shall verify routes at the start of each shift and after adverse weather or any site layout change. - Routes shall be kept clear of obstructions, stored materials, waste, and trailing cables. Barriers, signage, and pedestrian/vehicle segregation shall be maintained. - Access routes shall be adequately lit, have safe, stable walking surfaces, and include edge protection on ramps, stairs, and elevated walkways where required. - Emergency exits, fire points, and muster routes shall remain clear and unobstructed at all times. - Trenches, services, and cables crossing routes shall be covered, protected, and secured, and re-checked after rain or wind. - Any change to access/egress routes shall be briefed at pre-start, updated on site signage/plans where used, and old or conflicting signage removed. HOLD POINT: If safe access or egress cannot be maintained, STOP work and re-plan before continuing. |  |  |  |  |  |
| 14.0 Manual Handling |  |  |  |  |  |
| - Manual handling shall follow the hierarchy of control: eliminate where possible, use mechanical aids first, then team handling, with PPE as the last line of defence. - Before handling, carry out a quick check of load weight/shape, sharp edges, route condition and lighting, and individual capability. - Use trolleys, dollies, skates, pallet trucks, or lifting machinery where practicable. Materials shall be delivered as close as possible to the point of use to minimise double-handling. - Team lifts shall be used for bulky or awkward loads. The route and set-down point shall be agreed before lifting. - Keep loads between knee and shoulder height where practicable and avoid twisting. - No manual handling shall be carried out on ladders. - Wear suitable gloves where required to protect against sharp edges and to maintain grip. - STOP and re-plan if visibility is restricted, the load becomes unstable, or the route becomes unsafe. Records: Any additional controls (e.g. team lift, mechanical aid, route change) shall be recorded on the Daily Residual Risk Checklist and/or toolbox talk notes where required. |  |  |  |  |  |
| # 15.0 Environmental and Waste Management Arrangements |  |  |  |  |  |
| # Waste management & Pest Control # Manage waste to Client standards and the project Environmental Management Plan (EMP). # Segregate waste (reusable/recyclable/general/hazardous) using labelled containers/skips. # Hazardous waste/COSHH: store sealed and labelled with SDS available; use secondary containment; dispose via approved/licensed contractor and retain transfer notes/manifests. # Remove waste to the designated refuse area daily (or as scheduled) and do not obstruct access/egress. # Pest control: keep food waste in sealed bins and remove daily; follow client bait-station plan (where applicable) and report infestations to Facilities/QHSE. |  |  |  |  |  |
| Noise # Use quieter methods/plant where practicable; position equipment to reduce impact; rotate tasks and use barriers/screens where practical. # Hearing protection requirements shall follow the Risk Assessment and site rules. # Where monitoring or site conditions indicate elevated noise, implement signage and additional controls as required. # Maintain any noise monitoring records in the work pack where required. |  |  |  |  |  |
| # Spill Prevention and Response (Fuel, Oils, Paint) Prevention - Monitor that refueling activities are conducted only at designated points by authorized personnel using approved spill prevention controls. - Spill kits available and checked routinely (including vehicles where applicable). - Protect drains where relevant and keep SDS available where substances are used. Response (record in Spill/Environmental Log) STOP → CONTAIN → PROTECT DRAINS → NOTIFY Supervisor/QHSE → CLEAN UP → DISPOSE via approved contractor → RECORD / CAPA If soil/water contamination is suspected: stop, contain, notify QHSE, and arrange approved clean-up and disposal; retain disposal records. |  |  |  |  |  |
| Dust - Use low-dust methods where practicable; use on-tool extraction/LEV where available, otherwise water spray/sheeting as suitable (without creating slip hazards). - Use RPE where required by the Risk Assessment. - Stop work if dust affects visibility or creates unacceptable exposure. - Clean using vacuum or damp methods; no dry sweeping. - Controls briefed at daily TBT and recorded where required. |  |  |  |  |  |
| # 16.0 Third Party Interactions |  |  |  |  |  |
| - Al Laith shall coordinate works to protect third parties, adjacent trades, facility users, and the public. - Daily coordination via TBT and/or coordination meeting to confirm work zones, interfaces, communication method, and hold points. - Maintain segregation using barriers, signage, and banksman/marshal where required. - Avoid conflicting work at height: no work over others/public unless protected (e.g., overhead protection/debris netting) or re-sequenced. - Visitor control: sign-in, escort, required PPE, and brief on exclusion zones and emergency arrangements. - Any changes affecting the public interface (route closures, access changes, noisy work, lifting near routes) shall be communicated to the Client/Main Contractor in advance where practicable. Attachments / Records (as applicable) - Site TMP extract / drawings reference - Daily Briefing & Residual Risk Checklist - Housekeeping / Environmental Log (incl. spill log) - Rescue kit inspection sheet (where WAH or water interface applies) |  |  |  |  |  |

| # 17.0 Permits Key Resource |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Items Attached: (Please indicate in the box below) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17.1 | Accidents & Incidents |  |  | Yes |  |  |  |  |  | No |  |  |  |  |
| 17.1 | Sketches/ Date sheets |  |  | x |  |  |  |  |  |  |  |  |  |  |
| 17.2 | Certification of Plant/operator competency |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17.3 | Program of Works |  |  | x |  |  |  |  |  |  |  |  |  |  |
| 17.4 | Risk Assessments |  |  | x |  |  |  |  |  |  |  |  |  |  |
| 17.5 | Excavation permit |  |  |  |  |  |  |  |  | x |  |  |  |  |
| 17.6 | Daily briefing form |  |  | x |  |  |  |  |  |  |  |  |  |  |
| 17.7 | Emergency contact details |  |  | x |  |  |  |  |  |  |  |  |  |  |
| 17.8 | Emergency arrangements |  |  | x |  |  |  |  |  |  |  |  |  |  |
| 17.9 | Method Statement Amendments and Review |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17.10 | Wind Log, Spill Kit Checklist, Residual Risk Checklist, and Housekeeping Log to be kept at the workface or site office for audit. |  |  |  |  |  |  |  |  |  |  |  |  |  |

| # 18.0 Emergency Contact Details |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Johan Tredoux QHSE Manager M: 056 179 3189 E: johan.tredoux@allaith.com Nearest Hospital Cedars Jebel Ali International Hospital 04-8814000 Emergency Numbers 998 (Ambulance) / 997 (Fire) First aid location: yard warehouse / office |  |  |  |  |  |

| # 19.0 Review and Amendments |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| REVIEW DATES: This Method Statement will be reviewed every 3 months, and after any incident, near miss, or significant change in scope or conditions. LAST REVIEW DATE: 09/07/2026 NEXT REVIEW DATE: 09/10/2026 AMENDMENTS AUTHORIZED BY: AMENDMENTS COMMUNICATED TO: All operatives involved |  |  |  |  |  |

## 20.0 Method Statement Briefing Record

Daily pre-start / TBT confirmation (as applicable): radio channel/backup method, emergency numbers and muster point, and rescue arrangements for the day’s activities (WAH / lifting interfaces / work near or over water).

### Immediate Actions (Any Emergency / Incident)

- STOP work and make the area safe (isolate energy sources/plant movements where applicable).

- Raise the alarm via site procedure (call site emergency number / notify Site Control/HSE).

- First aid by trained first aider; request medical support as required.

- Rescue only if it can be done safely, by competent persons, with suitable equipment.

- Keep access clear for emergency responders and guide them to the location.

### Reporting / Preservation

- Report all incidents and near misses as per site and company procedure.

- Preserve the scene/equipment where safe and instructed (do not disturb unless required for rescue or to prevent further harm).

- Abu Dhabi (where WAH applies): rescue arrangements shall align with the site Fall Prevention Plan (OSHAD CoP 49) where applicable.

| # 19.0 Method Statement Briefing Record |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Briefing delivered by………………………………………………………….…… Position…………Supervisor or Project Manager………………….. Date…………………………………………………………………………………. We (the undersigned) have read and understood the attached method statement and will comply with the specified requirements and control measures. If the work activity changes or deviates from that originally envisaged, we will seek further advice and request an amended method statement. |  |  |  |  |  |

| Name (Print) | Signature | Date |
| --- | --- | --- |