| Risk Assessment Procedural Requirements |  | System Document Control # EV-MS-RA-2000 |
| --- | --- | --- |
|  |  | Revision: 000 |
|  |  | RA: 2000 |
|  |  | Original Assessment Date: 09/07/2026 |
|  |  | Last Review Date: 09/07/2026 |
|  |  | Next Review Date: 09/10/2026 |
|  |  | Activity Location: Saih Shuaib 3, Dubai Industrial City (DIC), Dubai, UAE — DIC Yard only |
| Responsible Leader: Responsible Member: | Prepared by: | Approved by: |
| Process: Yard Activities (Carpentry, AC, Plumbing, Lifting) | Date: | Date: |
| Division: Site Services | Signature: | Signature: |

PPE requirements for the operation

| Safety Gears Required | Yes | No | Safety Equipment Required | Yes | No |
| --- | --- | --- | --- | --- | --- |
| Safety Helmet (Hard Hat) | √ |  | Safety Gloves – Cloth or leather | √ |  |
| Hearing Protection | AR |  | Safety Footwear – Boots | √ |  |
| Safety Glasses | √ |  | Dust Mask or Face Mask | √ |  |
| Coveralls | √ |  | Full Body Harness (Double Hook) > 1.8 meters | AR |  |
| High Visibility Vest – Orange for banksman and/or signallers | √ |  | Inertia Reel/Retractable | AR |  |
| Life Jackets | AR |  | Life Line | AR |  |

√ = Required, R = Recommended, AR = As Required, N/A = Not Applicable

Legal Notice.

Al Laith Projects Services LLC Health & Safety Risk Assessments endeavor to allow for the identification of hazards, risks and control measures associated with all Al Laith Site Services yard activities carried out at the DIC Yard — namely carpentry, air-conditioning (AC) repair & maintenance, plumbing & waste tank cleaning, and lifting operations.  Also to identify the Significant Findings and the Relevant Persons at risk from them. This Risk Assessment is not valid if responsible members did not sign.

The Responsible Person and/or their Representatives should ensure that this is achieved.

It should be noted that it is the responsibility of the Responsible Person and/or their Representative to implement any required actions identified by the H&S Risk Assessment and not the responsibility of the QHSE Department.

The Responsible person/or their Representative should note that nothing within the Risk Assessment overrides any requirements needed to comply with the statutory obligations, under the Health and Safety at Work act 1974, and all associated regulations and approved codes of practise.

The Risk Assessment does not override, where appropriate, the application requirements for Building Control, Local Authority, Licensing and any other consent.

The Risk Assessment is a continuous, live process and must be monitored and audited, reviewed and revised with any structural and material changes to the yard works (carpentry, AC, plumbing, waste tank cleaning and lifting operations), the usage of, and any changes to processes carried out. The Risk Assessment should be reviewed with any significant changes to staff and occupancy, and following any “near miss”, incidents or accidents.

Any change could lead to new risks and hazards needing to be considered and no liability rests with the QHSE Department in this respect. The Risk Assessment should be programmed for review at intervals not exceeding three months.

LIKELIHOOD (L)

Risk Rating

RR 1 = 25-30 = Intolerable

RR 2 = 20-24 = High

RR 3 = 13-18 = Substantial

RR 4 = 11-12 = Moderate

RR 5 = 7-10 = Tolerable to Moderate

RR 6 = 1-6 = Tolerable

RISK MATRIX

Key to Risk Assessment: (L = LIKELIHOOD, S = SEVERIY, RR – RISK RATING)

| L | Likelihood | S | Severity | RR | Resultant |
| --- | --- | --- | --- | --- | --- |
| 5 | Likely to occur repeatedly. | 5 | Fatality / Permanent Disability. | 16 – 25 | Work may not proceed – consider alternative methods. |
| 4 | Likely to occur more than once. | 4 | Major Injury / Long term Absence. | 10 – 15 | Re – assess mitigation measures to reduce the risk. |
| 3 | Likely to occur. | 3 | LTI injury / Temporary Disability. | 6 – 9 | Work may proceed subject to implementation of mitigation measures, if no alternative available. |
| 2 | Unlikely to occur. | 2 | Injury requiring additional medical treatment. | 1 – 5 | Acceptable – Work may proceed. |
| 1 | Extremely unlikely to occur. | 1 | First Aid (e.g. Bruising, abrasion, etc.) |  |  |

| 1 | Pre-Job Planning & Competency | H.1 Untrained/unauthorised staff; expired competency certificates. H.2 Inadequate maintenance of inspection and tool-tag records. H.3 Unclear method statements, scope or drawings; missing permits. H.4 Inadequate risk assessment / inappropriate control measures. H.5 Poor planning to protect operatives, other trades and visitors. H.6 Incorrect material or equipment selection. | 1. Personal injury or fatality due to inexperienced staff. 2. Loss of record control / non-compliance. 3. Asset or property damage. 4. Incidents affecting other trades, visitors or public. | All staff Other trades Visitors Contractors | 5 | 4 | 20 | B.1 Only trained, competent and authorised staff to work; competency verified by Supervisor before task allocation. B.2 Daily toolbox talk (TBT) by the Supervisor prior to work. B.3 Approved, task-specific method statement and risk assessment briefed to the team. B.4 Regular inspection of tool-tag register, TBT records and certificates. B.5 Report any deviation from the method statement to Site Supervisor / HSE. B.6 Inspect all materials and equipment before use. B.7 Establish permit-to-work for high-risk tasks where required. | 5 | 1 | 5 | Site Supervisor HSE Officer | R.1 RA & Method Statement Procedure. R.2 Competency verification by Supervisor. R.3 Stop-work authority (Site Supervisor / HSE Officer). |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | Driving to Site / Delivering Materials | H.1 Driver not competent, unqualified or medically unfit to drive in the UAE. H.2 Transporting materials, supplies and personnel between locations. H.3 Tired, ill, on medication, or under the influence of alcohol/drugs. H.4 Distraction (mobile phone, eating, navigation device). H.5 Third-party road users; poor or temporary road conditions. | 1. Road traffic accident, major injury or fatality. 2. Personal injury due to impact. 3. Asset damage; breakdown in remote location. | Drivers Pedestrians Third parties | 5 | 4 | 20 | B.1 Valid UAE driving licence for the vehicle driven; vehicle business-insured. B.2 Manager completes Driving-for-Work checks and H&S induction. B.3 No use of in-car devices, eating or drinking whilst driving. B.4 Do not drive if ill, tired or on impairing medication; no drink/drug driving. B.5 Adhere to designated loading/unloading areas and speed limits. B.6 Implement traffic management (cones, barricades, signage, flagmen) as required. | 5 | 1 | 5 | Site Supervisor HSE Officer Driver | R.1 Land Transport Procedure. R.2 Alcohol & Drug Procedure. |
| 3 | Yard Traffic, Plant & Vehicle Movement | H.1 Forklift, vehicle and mobile plant movement in the yard. H.2 Unauthorised or untrained operators. H.3 Pedestrian/plant interface; reversing in congested zones. H.4 Poor yard route conditions (potholes / uneven ground). H.5 Distraction (mobile phones); poor lighting. H.6 Fuel/oil leaks; inadequate barricading and signage. | 1. Struck-by / run-over injuries to pedestrians. 2. Vehicle or plant collision causing injury. 3. Asset / property damage. | Drivers Operators Pedestrians Visitors Contractors | 5 | 4 | 20 | B.1 Only competent, authorised operators with valid licences to operate plant/vehicles. B.2 Enforce yard speed limit, designated routes and one-way systems where practicable. B.3 Segregate pedestrians from vehicles; maintain crossings and exclusion zones. B.4 Trained banksman mandatory for reversing and congested-area manoeuvres. B.5 Reverse alarms and mirrors/cameras fitted and functional. B.6 Keep emergency and access routes clear; maintain lighting. B.7 Daily pre-use inspection of plant; report defects. | 5 | 1 | 5 | Site Supervisor Banksman / Traffic Marshal HSE Officer | R.1 Yard Traffic Management Plan. R.2 Vehicle / Plant Inspection Record. |
| 4 | General Manual Handling | H.1 Lifting heavy or awkward loads beyond personal capacity. H.2 Twisting, top-heavy bending, pushing/pulling. H.3 Incorrect lifting technique. H.4 Poor underfoot conditions. | 1. Musculoskeletal injuries, strains and sprains. 2. Dropped loads / crush injuries. 3. Aggravation of pre-existing conditions; possible chronic injury. | All Al Laith Site Services staff | 3 | 4 | 12 | B.1 Manual handling by trained personnel; competency verified by Supervisor. B.2 Use mechanical aids (trolleys, pallet jacks, hoists) wherever practicable. B.3 Team lifting for heavy/bulky loads where mechanical means not practical. B.4 Assess load, route and underfoot conditions before lifting. B.5 Reduce load size/weight; deliver materials close to work area. B.6 Individuals to inform Supervisor of any health condition affecting lifting. B.7 Correct gloves and footwear selected. | 3 | 1 | 3 | Site Supervisor Trade Supervisors | R.1 Manual Handling Procedure. R.2 Manual Handling awareness / TBT. |
| 5 | Housekeeping & Waste Management | H.1 Waste and material accumulation in work and walkway areas. H.2 Debris causing slips, trips and falls. H.3 Incorrect waste segregation. H.4 Combustible waste build-up (fire hazard). H.5 Runoff into drains. | 1. Slips, trips and falls. 2. Fire hazard. 3. Environmental contamination / regulatory fines. | All yard personnel Waste handlers | 4 | 3 | 12 | B.1 “Clean-as-you-go” policy; keep walkways and exits clear. B.2 Labelled bins for segregated waste streams; covered skips to prevent windblown debris. B.3 Combustible waste (offcuts, sawdust, packaging) removed regularly. B.4 Spill kit available for immediate cleanup; protect drains. B.5 Waste collected by licensed subcontractor; retain waste transfer notes. | 4 | 1 | 4 | Site Supervisor HSE Officer Approved Contractor | R.1 Housekeeping log. R.2 Waste transfer notes file. |
| 6 | Damaged Flooring, Potholes & Ground Conditions | H.1 Cracks and potholes in asphalt/concrete. H.2 Loose gravel or uneven soil. H.3 Degradation of heavy-load / plant standing areas. H.4 Trip hazards from poorly maintained surfaces. | 1. Slips, trips and falls; ankle/knee injuries. 2. Forklift / vehicle instability. 3. Damage to plant/equipment. | All yard personnel Operators Visitors | 4 | 3 | 12 | B.1 Weekly recorded yard surface inspections. B.2 Rapid repair protocol for potholes / damaged surfaces. B.3 Temporary barricading and signage around damaged areas. B.4 Enforce yard speed limit; reduce speed on uneven ground. B.5 Ensure firm, level ground for plant standing and crane outrigger areas. | 4 | 1 | 4 | Site Supervisor Maintenance Team HSE Officer | R.1 Yard Inspection Log. |
| 7 | Slips, Trips & Falls | H.1 Poor housekeeping / debris on floor. H.2 Trailing cables and hoses. H.3 Changes in floor level. H.4 Slippery surfaces from spills, water leaks or cleaning. | 1. Minor bruises/abrasions. 2. Fractures or head injuries. 3. Secondary line-of-fire injuries. | All yard personnel Visitors Office staff | 4 | 3 | 12 | B.1 Clean-as-you-go; keep walkways 100% clear. B.2 Cable/hose management using ramps or overhead routing. B.3 Mandatory slip-resistant safety boots. B.4 “Wet Floor” signage; immediate cleanup of spills/leaks. B.5 Maintain lighting on walkways. | 4 | 1 | 4 | Site Supervisor HSE Officer | R.1 Housekeeping Log. R.2 TBT Records. |
| 8 | Electrical Defects & Temporary Power | H.1 Damaged sockets/plugs, exposed wiring in yard/welfare/workshop. H.2 Overloaded circuits; defective temporary distribution boards. H.3 Damaged insulation on leads/generators; poor earthing. H.4 Trip hazards from cables; water near live parts. | 1. Electric shock or electrocution. 2. Electrical fire; burns. 3. Equipment damage. | Maintenance staff All trades Yard personnel | 5 | 4 | 20 | B.1 Only certified/competent electricians to perform electrical repairs. B.2 PAT / periodic inspection of portable tools and leads; colour-coding. B.3 Visual pre-use checks of cables and distribution boards. B.4 Use of RCDs/ELCBs; verified earthing. B.5 Use 110V tools where practicable; IP-rated connectors. B.6 Cable management and covers; keep power away from water. B.7 Defective equipment tagged out and removed from service. | 5 | 1 | 5 | Qualified Electrician Site Supervisor HSE Officer | R.1 Electrical Maintenance Records. R.2 PAT Register. |
| 9 | Poor Lighting | H.1 Failed yard floodlights; shadow areas in storage zones. H.2 Dim lighting in workshop areas. H.3 Lack of emergency lighting in welfare/exits. | 1. Slips/trips/falls. 2. Vehicle/plant collisions. 3. Eye strain and errors. | Night-shift personnel Drivers Security | 4 | 3 | 12 | B.1 Weekly night-time lighting checks. B.2 Maintain stock of spare bulbs/fittings. B.3 Provision of portable lighting towers for night loading. B.4 Functional backup/emergency lighting at all exits. | 4 | 1 | 4 | Maintenance Team Site Supervisor HSE Officer | R.1 Lighting Inspection Record. |
| 10 | Heat Stress & Adverse Weather | H.1 High temperature and humidity. H.2 Dehydration / electrolyte imbalance; UV exposure. H.3 Heat fatigue causing poor judgement. H.4 High winds, rain, lightning, poor visibility (dust/rain). | 1. Heat exhaustion / heat stroke. 2. Slips/trips from fatigue. 3. Struck-by / dropped-object incidents in wind. | All yard personnel Operators Visitors | 5 | 4 | 20 | B.1 Comply with UAE midday break rule (12:30–15:00, mid-June to mid-September). B.2 Schedule heavy tasks for cooler periods; provide shaded rest areas. B.3 Provide cool potable water and electrolytes; encourage regular hydration. B.4 Provide shade/shelter and cooling fans; SPF and broad-brim helmet shades. B.5 Monitor forecasts; brief weather triggers at TBT; buddy checks. B.6 STOP wind-sensitive / elevated tasks at unsafe limits; stop for lightning or poor visibility. B.7 Supervisor re-check of routes/structures before restart. | 5 | 1 | 5 | Site Supervisor HSE Officer | R.1 Heat Stress Protocol / TBT. R.2 Weather Hold / Restart Log. |
| 11 | Fire Safety & Emergency Readiness | H.1 Blocked fire extinguishers, fire points or emergency exits. H.2 Combustible waste accumulation (wood dust, packaging, oils). H.3 Electrical defects; hot works (brazing/soldering, grinding sparks). H.4 Smoking outside designated areas. H.5 Fire alarm / emergency system impairment. | 1. Fire spread; injury or fatality. 2. Property damage. 3. Regulatory non-compliance. | All yard personnel Contractors Visitors | 5 | 4 | 20 | B.1 Daily visual checks of fire points and emergency access; keep clear. B.2 Combustible waste removed regularly; controlled chemical/fuel storage. B.3 Hot-work permit, fire watch and extinguishers for brazing/soldering/grinding. B.4 Smoking only in designated areas; electrical defects isolated. B.5 Maintain emergency signage, lighting and functional fire systems. B.6 Emergency response plan communicated; drills conducted. | 5 | 1 | 5 | Site Supervisor HSE Officer | R.1 Fire Inspection Records. R.2 Emergency Response Plan. |
| 12 | Night Work | H.1 Poor lighting/visibility at workfaces and routes. H.2 Worker fatigue / reduced alertness. H.3 Increased pedestrian-plant interface risk. H.4 Communication failure or lone working. | 1. Struck-by or fall injuries. 2. Property/asset damage. 3. Fatigue-related error. | Night-shift personnel Operators Visitors | 4 | 3 | 12 | B.1 Mandatory task/route lighting; STOP if inadequate. B.2 Fatigue checks at start/mid-shift; adequate rest breaks. B.3 Reflective high-visibility PPE enforced. B.4 Strict traffic rules and banksman control; segregate work zones. B.5 No lone working for high-risk tasks; confirm radio/emergency communication. | 4 | 1 | 4 | Site Supervisor HSE Officer Security / Traffic Marshal | R.1 Night Lighting Inspection Record. R.2 TBT Record. |
| 13 | Interface with Public, Other Trades & Trespassers | H.1 Work clash / two-tier works between trades. H.2 Unauthorised entry / trespassers. H.3 Slip/trip hazards created for others. | 1. Injury to personnel or public. 2. Delay to work; work clash. 3. Property/asset damage. | All personnel Other trades Public | 5 | 4 | 20 | B.1 Plan and coordinate works with all interested parties in advance. B.2 Cordon off work areas with barricades; restrict entry via banksman/signage. B.3 Display warning boards and signage. B.4 Close supervision maintained; segregate concurrent activities. | 5 | 1 | 5 | Site Supervisor Trade Supervisors HSE Officer | R.1 TBT on coordination & housekeeping. |
| 14 | Use of Power & Hand Tools (incl. Angle Grinder) | H.1 Contact with sharp edges / rotating parts; kickback. H.2 Electricity / fire / noise / flying particles. H.3 Angle grinder disc shatter, sparks and entanglement. H.4 Defective, incorrect or unguarded tools. | 1. Amputation, deep lacerations, eye injuries. 2. Electric shock / burns. 3. Flying-particle / material damage; hearing damage. | All trades operatives | 4 | 4 | 16 | B.1 Only trained operatives to use power/hand tools; only certified/competent operators to use angle grinders. B.2 Correct guarding fitted; use the right tool and correct disc for the job. B.3 Tools inspected, colour-coded and PAT tested periodically; use 110V where practicable. B.4 Insulated connections; cable management. B.5 Damaged tools quarantined and tagged out; store-issue inspection. B.6 Mandatory PPE: eye protection, hearing protection, gloves, face shield for grinding. B.7 Adequate supervision; TBT on safe tool use. | 4 | 1 | 4 | Site Supervisor Trade Supervisors | R.1 TBT on safe use of power & hand tools. R.2 Angle grinder operator certification. |
| 15 | Carpentry Works | H.1 Contact with sharp edges/blades (saws, chisels, knives). H.2 Flying debris / splinters. H.3 Noise (power tools). H.4 Wood dust inhalation. H.5 Impact injuries (hammering, falling objects). H.6 Pinch points (clamping, moving materials). H.7 Manual handling of timber and materials. | 1. Lacerations, amputations. 2. Eye injuries; hearing loss. 3. Respiratory issues from wood dust. 4. Bruises, fractures, crush injuries. 5. Musculoskeletal injuries. | Carpenters Adjacent workers | 5 | 4 | 20 | B.1 PPE: safety glasses, hearing protection, gloves, dust mask/respirator, safety boots. B.2 Power tools in good condition, guarded, inspected and colour-coded. B.3 Dust extraction and good housekeeping to remove offcuts and sawdust. B.4 Secure workpieces with clamps; keep tools sharp. B.5 Insulated connections and RCDs on electrical tools. B.6 Safe manual handling / mechanical aids for heavy timber. B.7 TBT on safe tool use, dust control and electrical safety; first aid available. | 5 | 1 | 5 | Carpentry Team Leader Site Supervisor HSE Officer | R.1 Awareness sessions. R.2 Site briefings / TBT. |
| 16 | AC Repair & Maintenance — Refrigerant Handling (Split Units) | H.1 Refrigerant leaks during recovery/recharge (asphyxiation / frostbite). H.2 Chemical exposure; pressurised refrigerant lines. H.3 Electrical hazards within units. H.4 Compressor-level / major repair hazards (stored pressure, hot components). | 1. Frostbite, asphyxiation, chemical exposure. 2. Electric shock / burns. 3. Environmental release of refrigerant. | AC technicians Nearby workers | 5 | 4 | 20 | B.1 In-house refrigerant recovery/recharge using recovery units and certified cylinders; no venting to atmosphere. B.2 Confirm and formalise F-Gas / refrigerant-handling certification for all technicians performing refrigerant work (certification status currently NOT confirmed — action to close). B.3 Calibrated gauges and leak detectors; work in ventilated areas. B.4 Isolate and lock off electrical supply before working on units (LOTO); test-before-touch. B.5 Gloves and eye protection against refrigerant; no hot work near refrigerants. B.6 SDS for refrigerants/oils available; TBT before task. | 5 | 1 | 5 | AC Supervisor HSE Officer | R.1 Evacuate / ventilate; first aid for inhalation/frostbite. R.2 Report environmental release. |
| 17 | AC Work at Height — Ladder Access (Rooftop / Elevated Split Units) | H.1 Falls from ladders / elevated positions. H.2 Improper ladder use / set-up; fragile or unstable edges. H.3 Dropped tools; weather/wind exposure. | 1. Serious injury or fatality from falls. 2. Struck-by injuries from dropped tools. | AC technicians Workers below | 4 | 4 | 16 | B.1 Access plan; ladders inspected, industrial-grade, set at 4:1 and secured/tied. B.2 Maintain three points of contact; do not overreach. B.3 Tool lanyards; barricade area below. B.4 Full body harness/anchor where a suitable anchor exists and ladder work is prolonged (AR). B.5 Wind and rain stop rules; do not work at height in adverse weather. B.6 Rescue plan in place; competency verified by Supervisor. | 4 | 1 | 4 | AC Supervisor Site Supervisor HSE Officer | R.1 Fall rescue / first aid; emergency services. |
| 18 | Plumbing Installation, Leak Repair & Pressure Testing | H.1 Burst fittings / projectile caps during pressure testing. H.2 Water hammer; sudden release of stored pressure. H.3 Slips from leaks/spills; water near electrics. H.4 Manual handling of pipes/reels; use of hand/power tools. | 1. Impact / laceration injury. 2. Flooding; electrical contact. 3. Slips and falls. | Plumbers Adjacent workers | 5 | 4 | 20 | B.1 Barriers / exclusion zone during pressure testing. B.2 Rated gauges and hoses; gradual pressurisation; cap restraint. B.3 Keep cables/panels dry; isolate electrics near water. B.4 Signage; mop kits for spills; isolate valves before work. B.5 Safe manual handling; correct tools inspected before use. | 5 | 1 | 5 | Plumbing Supervisor Site Supervisor HSE Officer | R.1 Isolate valves; clean-up; electrical isolation; first aid. |
| 19 | Waste Tank Cleaning — External Vacuum / Hose Operation | H.1 Spill / splash of effluent during hose/pump operation. H.2 Odour / H2S / biological-hygiene exposure (external, not entry). H.3 Manual handling of heavy hoses and equipment. H.4 Vacuum tanker vehicle movement and operation. | 1. Skin/eye contact; biological infection. 2. Respiratory irritation from odour/gases. 3. Musculoskeletal injury; struck-by tanker. | Plumbing/waste operatives Banksman Nearby workers | 4 | 4 | 16 | B.1 Tanks cleaned externally via hose/pump only — NO confined space entry is performed. Any future entry requirement would need a separate confined-space entry permit and is currently out of scope. B.2 Chemical-resistant gloves, eye/face protection, coveralls and boots; wash facilities and hygiene controls. B.3 Work upwind where possible; monitor for odour/gas; ventilate around openings. B.4 Safe manual handling / team lift for hoses and equipment. B.5 Banksman for tanker positioning and reversing; wheel chocks. B.6 Waste collected/disposed by licensed subcontractor; retain waste transfer notes. | 4 | 1 | 4 | Plumbing Supervisor HSE Officer | R.1 Emergency wash / first aid for exposure. R.2 Waste transfer notes. |
| 20 | Chemical Handling, Storage & Use (COSHH) | H.1 Paint, thinners, chemical soap/degreaser, AC coil cleaner, pipe sealants/adhesives. H.2 Skin/eye/respiratory irritation. H.3 Flammability (paint/thinners). H.4 Incompatible chemical mixing. | 1. Chemical burns, irritation, dermatitis. 2. Fire from flammable liquids. 3. Toxic reaction from incompatible mixing. | All trades using chemicals | 4 | 4 | 16 | B.1 Establish and maintain an SDS register for all chemicals used, accessible at point of use (SDS not currently formalised — action to close). B.2 Store chemicals in the dedicated controlled storage area; segregate incompatibles. B.3 Labelled containers; keep flammables away from ignition sources / hot works. B.4 Ventilation at point of use; secondary containment / drip trays. B.5 PPE per SDS (gloves, eye protection, RPE as required). B.6 Spill kit available; dispose of chemical waste via licensed contractor. | 4 | 1 | 4 | Site Supervisor HSE Officer Trade Supervisors | R.1 SDS / COSHH register (to be established). R.2 Spill response. |
| 21 | Forklift Loading / Unloading of Materials | H.1 Untrained operator; pinch points and sharps. H.2 Load falling from forks / lift. H.3 Loss of unsecured load. H.4 Personnel struck by plant; manual handling injuries. | 1. Musculoskeletal injuries, bruises, concussion. 2. Fatality; loss of limbs (amputation). 3. Property/asset damage. | Operators Banksman Pedestrians Operatives | 5 | 4 | 20 | B.1 Machine in good working condition; valid third-party certification and UAE documents. B.2 Competent operator with valid licence; regular servicing and daily pre-use inspection. B.3 In-house trained banksman/signaller to assist in critical areas. B.4 Reverse alarm and mirrors/camera functional; no persons in operating area. B.5 Loads secured; correct equipment selected for the task. B.6 No mobile phone use during operation; park in designated area after use. | 5 | 1 | 5 | Lifting / Yard Supervisor Site Supervisor HSE Officer | R.1 Manual Handling Procedure. R.2 Lifting & Hoisting Procedure. |
| 22 | Mobile Crane Lifting Operations (BS 7121 / LOLER) | H.1 Crane set-up on inadequate ground; outrigger loading / instability. H.2 Incorrect rigging / slinging; damaged lifting tackle (slings, shackles, chains). H.3 Load or radius exceedance; overturning. H.4 Overhead lifting / dropped load; personnel in fall zone. H.5 High winds / adverse weather. | 1. Crane overturn; dropped load. 2. Fatality or major injury; struck-by. 3. Major property/asset damage. | Lifting crew Banksman Adjacent personnel | 5 | 4 | 20 | B.1 Lift planned to BS 7121 / LOLER by in-house Appointed Person; lift plan and RA briefed via TBT before lifting. B.2 In-house certified crane operator, rigger/slinger and banksman/signaller (Al Laith Site Services in-house competent persons). B.3 Firm, level ground verified; outrigger mats/plates sized for point loads; check for underground services/voids. B.4 Lifting tackle inspected before each use; damaged tackle marked “NOT FOR USE” and removed; sling guards on sharp edges. B.5 Loads within SWL and radius with safety margin; centre of gravity and lifting points verified. B.6 Exclusion zone / barriers; no personnel under suspended loads; hand signals / radio comms. B.7 Suspend lifting above 11 m/s wind (general lifting) and 7 m/s (man-riding); stop for lightning. | 5 | 1 | 5 | Lifting / Crane Supervisor Appointed Person HSE Officer | R.1 Lift Plan (BS 7121 / LOLER). R.2 Statutory test certificates & tackle inspection register. |
| 23 | Rescue & Emergency Information | H.1 Lack of personnel preparedness / first aiders. H.2 Lack of communication in emergency. H.3 Delayed emergency response. | 1. Aggravated injury; fatality. 2. Property/asset damage. | All personnel Visitors Operatives | 5 | 4 | 20 | B.1 Notify site HSE and management immediately; dial site control room / emergency services. B.2 Trained First Aiders available on site; first aid kit maintained. B.3 Emergency response and evacuation plan communicated; drills conducted. B.4 Avoid lone working; maintain means of communication. B.5 Certified rescue retrieval for height / lifting operations. | 5 | 1 | 5 | Site Supervisor HSE Officer First Aider | R.1 First Aid Training. R.2 Emergency First Aid Plan. R.3 Medical Emergency Response Procedure. |