AL LAITH YARD

METHOD STATEMENT

YARD OPERATIONS

INSTALLATION, ALTERATION AND DISMANTLING OF SCAFFOLD STRUCTURES

(MOCK-UPS AND GENERAL YARD USE)

LIFTING, MOVING, SHIFTING, LOADING/UNLOADING AND STORAGE OF MATERIALS AND EQUIPMENT

SURFACE PREPARATION AND COATING WORKS (WHERE AUTHORISED)

FABRICATION AND WORKSHOP ACTIVITIES (CUTTING/GRINDING/DRILLING/WELDING AS AUTHORISED)

PLUMBING, ELECTRICAL AND A/C WORKS WITHIN YARD/WORKSHOP AREAS (BY COMPETENT PERSONS ONLY)

CARPENTRY WORKS

JOB NUMBER: 2000

# CONTENTS

1.0 TASK DETAILS
 1.1 Customer
 1.2 Project Name
 1.3 Site Address/Location
 1.4 Description of the Task/Activity

2.0 REFERENCES
 2.1 Specification
 2.2 Reference (Drawings)

3.0 SCOPE & OBJECTIVES

4.0 PROGRAM

5.0 KEY RESOURCES
 5.1 Persons / Responsibility
 5.2 Accidents & Incidents
 5.3 CAPA
 5.4 Materials
 5.5 Other Essential Equipment
 5.6 Mandatory PPE
 5.7 Task Specific PPE
 5.8 Plant Equipment
 5.9 Purpose
 5.10 Identified Risks
 5.11 Site Specific Rules & Restrictions

6.0 SEQUENCE OF OPERATION

8.0 SUPPORT WORK & STORAGE ARRANGEMENTS

9.0 SAFETY REQUIREMENTS
 9.1 Specific Identified Residual Hazards
 9.2 Required Personal Protective Equipment Applicable
 9.3 Hazardous Substances Applicable

10.0 QUALITY MONITORING

11.0 EMERGENCY PROCEDURES

12.0 ACCESS & EGRESS POINTS

13.0 ENVIRONMENTAL IMPACT

14.0 THIRD PARTY INTERACTION

15.0 PERMITS KEY RESOURCE

16.0 EMERGENCY CONTACT DETAILS

17.0 REVIEW AND AMENDMENTS
 17.1 Rescue Arrangements

18.0 EMERGENCY PROCEDURES

19.0 METHOD STATEMENT BRIEFING RECORD

| # 1.0 Task Details |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1.1 | Customer | Name: Al Laith Project Services / Site Services (Yard Operations) | Address: Saih Shuaib 3 Dubai Industrial City, Dubai, UAE |  |  |
| 1.2 | Project Name | Yard Activities |  |  |  |
| 1.3 | Site Address/Location | P O Box 27349, Saih Shuaib 3 Dubai Industrial City, Dubai, UAE |  | Date: | Daily |
|  |  |  |  | Time: | As per Yard Working Hours / Approved Schedule |
| 1.4 | Description of the Task/Activity | ### This Method Statement covers yard-based activities, including the planning, execution, supervision, inspection and control of: ### Scaffold mock-up erection, alteration, inspection/tagging and dismantling ### Lifting, moving, shifting, loading/unloading and storage of materials ### Fabrication and workshop works (cutting, grinding, drilling, welding) ### Painting and shot blasting (surface preparation and coating) ### Plumbing, electrical and A/C works within yard/workshop areas ### Carpentry works |  |  |  |
| # 2.0 References |  |  |  |  |  |
| 2.1 | Specification | All works shall be planned, supervised, executed, inspected and controlled in accordance with: - UAE Federal Laws and applicable Dubai Municipality / UAE requirements - ISO 45001 Occupational Health & Safety Management principles - Manufacturer instructions and good industry practice - Approved drawings/calculations (where required) and internal yard procedures Work at Height (where applicable): A Fall Prevention Plan shall be implemented, covering: - Inspection regime and records - Collective protection first (edge protection, proper platforms); PPE as last line of defence - Competent supervision and worker competence - Rescue arrangements, equipment availability and drills (as required) Scaffold (where applicable): Scaffolds shall be erected/altered/inspected/dismantled in line with BS EN 12811-1 and relevant NASC guidance (tube & fitting), plus applicable UAE requirements. |  |  |  |
| 2.2 | Reference (Drawings) | Attache Approved Design / Sketch / Plan / Lifting Plan where required. ALS-FAB-0095-01-REV00 SCAFFOLDING STRUCTURE.pdf |  |  |  |
| 2.3 | This Method Statement shall be read together with the approved Risk Assessment and drawings listed in Section 2.2. Controls identified in the Risk Assessment are implemented through the planned sequences and controls in this MS. Any deviation, change, or unforeseen condition shall trigger a STOP and escalation to the Supervisor/Manager before proceeding. Documents to be available at workface / Supervisor file (as applicable): - Approved MS/RA and latest revision register - Approved drawings/calculations (where required) - PTW / task authorisations (where required) - Daily TBT records and attendance - Plant/equipment inspection records (forklift/telehandler/MEWP, lifting gear certificates) - Scaffold inspection/tagging records (Scafftag) and weekly inspection sheets - Tie register / anchor log (where applicable) - Harness inspection log (only where fall arrest is used) - Wind/weather hold-restart records (where applicable for tall/covered structures) - Any structure-specific checklists (netting, sheeting, kentledge/ballast where applicable) ## Management of Change (MoC) / Stop Work Rule Work shall STOP immediately and the MS/RA shall be reviewed and updated when any change occurs, including (but not limited to): - Design/sketch/drawing revisions - Change in work area or yard layout/segregation - Ground/support condition changes - Change in load class / SWL / material type - Weather deterioration affecting stability or lifting/WAH - Introduction/change of plant, MEWP or lifting interface - Change in work sequence / SIMOPS conflict - Change in staffing, competency or supervision level - Any incident, near miss, or unsafe condition |  |  |  |  |
| # 3.0 Scope & Objectives |  |  |  |  |  |
| This Method Statement outlines the execution strategy, supervision, coordination and control measures required for the safe delivery of yard activities at Al Laith Yard (DIC). Scope includes: - Receiving, loading/unloading and yard storage/stacking - Internal yard movement and segregation (plant/pedestrian control) - Scaffold mock-up erection, alteration, inspection/tagging and dismantling - Fabrication, welding/cutting/grinding and workshop operations - Painting and shot blasting activities including surface prep and coating - Electrical, plumbing and A/C works within yard/workshop areas - Carpentry works and general yard maintenance support ### Objectives - Protect workers, visitors and authorised yard personnel - Prevent falls, dropped objects, struck-by and vehicle/plant incidents - Ensure structural stability of scaffold mock-ups and temporary works at all times - Control SIMOPS, lifting and hazardous work interfaces - Maintain compliance with applicable legal requirements and ISO 45001 principles - Deliver works without incident, damage or environmental harm |  |  |  |  |  |
| # 4.0 Program |  |  |  |  |  |
| Works shall be delivered as per the approved yard plan and daily schedule. Activities shall be planned using daily coordination and look-ahead planning. Sequencing will be controlled by the Yard Supervisor to maintain safe access, segregation and interface control between trades and plant movements. |  |  |  |  |  |

## 5.0 KEY RESOURCES

### 5.1 Roles, Responsibilities and Authority

Roles and responsibilities shall follow Al Laith yard supervisory structure and this Method Statement. All personnel shall comply with yard rules, internal work authorisation requirements (PTW where applicable), and the approved MS/RA.

#### Project / Yard Manager

Responsible for overall delivery and safe management of yard activities, including:

- Ensure adequate resources (people, equipment, materials, access and time) are provided.

- Ensure work is planned and controlled in line with the approved MS/RA and applicable drawings/sketches.

- Ensure yard interfaces are managed (plant movements, deliveries, storage areas).

- Ensure Management of Change is applied when conditions change (stop work, review, update and re-brief).

- Ensure incidents, near misses and non-conformances are reported, investigated and actions closed out.

#### Supervisor / Workshop Supervisor

Responsible for day-to-day control of works, including:

- Allocate tasks and confirm competence for each activity (scaffolders, riggers/banksman, plant operators, welders, painters, electricians, etc.).

- Implement and maintain exclusion zones, barriers, signage, and pedestrian/plant segregation.

- Ensure daily TBT/briefings are delivered and recorded before work starts and when conditions change.

- Coordinate SIMOPS to avoid clashes (e.g., lifting near painting/blasting, hot works near flammables).

- Ensure housekeeping and safe storage standards are maintained at each workface.

- Stop work if unsafe conditions are identified and escalate for corrective action.

#### Scaffold Foreman / Competent Scaffold Inspector

Responsible for scaffold control and compliance (where scaffolding is involved), including:

- Ensure erection, alteration and dismantling follow approved drawings/design (where required), manufacturer guidance and this MS.

- Complete interim checks at key stages and confirm stability (base, bracing, ties, plumb/level, access).

- Carry out formal inspections and issue/update Scafftag and inspection records as per schedule.

- Maintain scaffold register, tie register (where applicable) and handover records, including load class/SWL signage.

- Stop work and report immediately if deviations, missing components, damage or unsafe access is identified.

#### HSE Manager / HSE Officers

Responsible for assurance and compliance support, including:

- Verify controls in the Risk Assessment are implemented and effective.

- Verify permits/work authorisations are in place where required by Al Laith procedures.

- Verify plant, lifting gear and tools are inspected and certified; non-conforming items are tagged and quarantined.

- Verify staff induction and training/competency records are valid for tasks assigned.

- Conduct inspections/audits, support investigations, and follow up corrective actions to closure.

- Support emergency preparedness and drills, including rescue arrangements for work at height where applicable.

#### Stores / Asset Controller

Responsible for issue/return and control of tools, lifting accessories and plant, including:

- Maintain registers, certification files, colour coding/calibration (where used) and quarantine defective items.

- Ensure storage is controlled to prevent damage, loss and unauthorised use.

#### Plant Operators (Forklift / Telehandler / MEWP / Other)

Responsible for safe operation of assigned plant, including:

- Operate only within valid licence/authorisation and equipment SWL.

- Complete pre-use checks and report defects immediately.

- Follow designated routes, speed limits, exclusion zones and yard traffic rules.

- Use a Flagman/Banksman where pedestrians, blind spots, reversing, tight access, or shared routes exist.

- Stop operations if visibility is lost or exclusion zone is not maintained.

#### Flagman / Banksman (Yard Plant Movements)

Responsible for controlling and guiding plant/vehicle movements, including:

- Establish and maintain exclusion zones and pedestrian segregation during movements.

- Guide reversing and tight manoeuvres, maintain clear communication with the operator.

- Stop movements immediately if persons enter the path, visibility is compromised, or ground conditions are unsafe.

- Coordinate with the Yard Supervisor for deliveries, offloading and lifting interface areas.

#### First Aiders and Fire Wardens

Responsible for first response and emergency control, including:

- Provide first response, maintain first-aid kits/AED (if available) and arrange escalation to emergency services.

- Maintain fire points/extinguishers in assigned areas (as applicable) and support muster/roll-call.

### 5.2 Selection of People (Competence)

Al Laith shall assign competent personnel based on training, experience, skills and medical fitness. Only authorised/competent persons shall carry out high-risk activities such as scaffolding, lifting/rigging/banksman duties, plant operation, electrical works, welding/hot works, and blasting/painting as applicable.

### 5.3 Worker Participation and Consultation

Worker participation is used to improve safety and performance:

- Daily TBTs capture hazards, site conditions and improvement suggestions.

- Feedback is recorded and reviewed by Yard Supervisor/Manager and HSE.

- Where improvements are required, controls shall be updated through MS/RA (MoC) and re-briefed.

- Safety concerns raised by workers shall be acted on immediately where they affect safe working.

| # 5.0 B Key Resource |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.1 | Controls |  |  | Control measures shall be selected and applied in line with ISO 45001 using the hierarchy of controls: Eliminate → Substitute → Engineering → Administrative → PPE (last line of defence). The Yard Manager/Supervisor shall confirm worker competence before task allocation (as applicable): scaffolders, MEWP (IPAF), plant operators (forklift/telehandler), lifting team (riggers/slinger/banksman), electricians, welders, blasters/painters. Evidence shall be maintained in the Training Matrix / Training Register. Refresher training (minimum): - Manual Handling - Work at Height (WAH) - Hand and Power Tool Safety Cycle: every 3 years (or more frequent where required by Al Laith procedures or legal/site rules). Where competence cannot be confirmed, the person shall not be assigned to that activity. |  |  |  |  |  |  |  |  |  |  |
| 5.2 | Accidents & Incidents |  |  | All incidents, near misses, unsafe conditions, environmental spills, and property damage shall be reported immediately to the Yard Supervisor and HSE team. - Initial reporting: as soon as practicable and without delay - Formal report: within 24 hours (or sooner if internally required) |  |  |  |  |  |  |  |  |  |  |
| 5.3 | CAPA |  |  | The Yard Manager/Supervisor and HSE team shall ensure actions are assigned, tracked and closed out for all H&S and fire-related findings. - Findings from audits, inspections, TBTs, incidents and near misses shall be recorded in the CAR/PAR Register within 24 hours. - Each record to include (as applicable): ID, source, description, risk level, contributing factors/root cause, action, owner, due date, status, evidence, closure sign-off. - Reviewed weekly by Yard Manager and HSE. Overdue actions escalated to senior management. - Closure requires verification evidence (photo/document/record) and sign-off by a competent person (preferably not the same person who carried out the action). |  |  |  |  |  |  |  |  |  |  |
| 5.4 | Materials |  |  | All scaffold materials and components shall be suitable, compatible with the system, and in good condition. Materials shall be inspected on delivery and before use. Damaged or non-conforming items shall be segregated, tagged “NOT FOR USE” and quarantined. - Tubes / Steel components: scaffold grade; no significant dents, cracks, deformation, excessive corrosion, or unauthorised repairs. - Fittings / Couplers: no cracks/distortion/thread damage; moving parts function correctly. - Timber boards (if used): compliant boards only; no splits/rot/major cracks/warping or contamination that hides defects. - System scaffold components (Cuplock/Allround/proprietary): manufacturer compliant and compatible; locking devices functional. - Ladder beams / steel beams: no cracks/distortion/corrosion or damaged end connections. Unfit materials shall be removed from service and removed from the yard as soon as practicable. |  |  |  |  |  |  |  |  |  |  |
| 5.5 | Other Essential Equipment: |  |  | Spanners, ratchets, levels, hammers, podger tools; lifting accessories where required. Tools used at height shall be secured where practicable (e.g., tool lanyards) to reduce dropped-object risk. |  |  |  |  |  |  |  |  |  |  |
| 5.6 | Plant, Equipment |  |  | All drivers and plant operators shall comply with the Al Laith Alcohol and Drug Procedure. No person shall operate vehicles or plant if under the influence, unwell, fatigued, or taking medication that may impair performance. Supervisors shall intervene and reassign duties if impairment is suspected. - Driving for work: competent drivers only; no mobile phone use while driving. - Plant and lifting equipment: valid inspection/certification where required; daily pre-use checks recorded; defects reported and equipment isolated. - Loading/unloading: establish exclusion zones; keep personnel out of the “line of fire”; use tag lines where required. |  |  |  |  |  |  |  |  |  |  |
| 5.7 | Yard Traffic / Plant Movement Control |  |  | Traffic management shall follow the yard traffic plan/rules: defined routes, segregation, crossings, speed control, and signage. Flagman/Banksman requirement (yard): A Flagman/Banksman shall be used for forklift/telehandler/MEWP and other plant movements where any of the following apply: - Reversing - Blind spots or restricted visibility - Tight access / congested work areas - Shared routes with pedestrians - Movement near loading/unloading or storage stacks - Any time the operator requests assistance Plant shall stop immediately if the Flagman/Banksman is not in position or if the exclusion zone cannot be maintained. |  |  |  |  |  |  |  |  |  |  |
| 5.8 | Identified Risks |  |  | Key risks include: working at height, manual handling, slips/trips/falls, vehicle and plant movements, deliveries and unloading, heat/humidity, flammable liquids/vapours, untrained personnel, poor housekeeping, falling materials, and scaffold instability/collapse. Refer to the approved Yard Risk Assessment for full details and controls. |  |  |  |  |  |  |  |  |  |  |
| 5.9 | Site Specific Rules & Restrictions |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.10 | Housekeeping Inspections |  |  | Housekeeping inspections shall be completed each shift for every active work area. Findings shall be recorded in the Daily Housekeeping Log. - Yard Supervisor is accountable for sign-off and closeout of actions. - Debris shall be cleared before lifting, boarding, or dismantling. - Housekeeping shall be re-checked after material movement, alterations to the work area, or weather delays. |  |  |  |  |  |  |  |  |  |  |

# 6.0 SEQUENCE OF OPERATION – AL LAITH YARD (DIC)

This section defines the operational sequence and control measures for all yard-based activities including lifting, scaffold mockups, fabrication, painting, electrical, plumbing, A/C and carpentry works.

All works shall follow approved MS/RA, internal Al Laith procedures, and applicable UAE legal requirements.

# 6.1 PERMITS & WORK AUTHORISATION

## 6.1.1 Permit Requirements

Before commencing any high-risk activity, the Yard Supervisor shall ensure required internal permits are obtained (where applicable):

- General Work Authorisation

- Hot Work Permit (welding, cutting, grinding)

- Lifting Permit / Approved Lift Plan

- MEWP Authorisation

- Electrical Isolation / LOTO

- Work at Height control (where applicable)

No high-risk activity shall commence without required approval.

## 6.1.2 Permit Validity & Change Control (MoC)

- Permits are shift-based unless otherwise stated.

- If permit expires → STOP WORK and revalidate before continuation.

- If scope, method, location, weather, load, plant, or supervision changes:

  - STOP

  - Make safe

  - Review MS/RA

  - Update if required

  - Re-brief workforce

  - Re-authorise before restart

HOLD POINT: No restart until Supervisor confirms safe conditions.

## 6.2 PRE-JOB PLANNING & START CRITERIA

Before starting each shift, the Yard Supervisor shall complete the following:

- Verify worker competence and authorisation for tasks planned (scaffolders, plant operators, riggers/banksman, welders, painters/blasters, electricians, A/C and plumbing personnel, carpenters).

- Conduct a Toolbox Talk (TBT) covering: work scope for the day, hazards, controls, exclusion zones, SIMOPS, emergency arrangements and stop-work triggers.

- Confirm the latest approved MS/RA is available at the workface / supervisor pack.

- Confirm permits and authorisations required for the day are identified and obtained where applicable (hot work, lifting, work at height, electrical isolation/LOTO, blasting/painting controls).

- Confirm plant and equipment pre-use inspections are completed and valid (FLT/telehandler/MEWP, lifting accessories, grinders/welders, compressors, blasting equipment).

- Inspect materials before use and quarantine any damaged/non-conforming items as “NOT FOR USE”.

- Confirm segregation, barriers, signage and exclusion zones are in place for the planned activities.

- Appoint a named Flagman/Banksman where plant movement, lifting, or deliveries are involved.

- Complete the Daily Residual Risk Checklist and record any additional controls required.

START HOLD POINT – Work shall only start when:

- Personnel are competent and briefed

- Permits/authorisations (where required) are approved and understood

- The area is segregated and access controlled

- Materials and equipment are inspected and suitable

- The Supervisor authorises commencement

## 6.3 YARD TRAFFIC MANAGEMENT & VEHICLE MOVEMENT

- UAE licensed and authorised drivers/operators only (as applicable).

- Follow yard speed limits and designated routes.

- No mobile phone use while driving/operating plant.

- Maintain pedestrian segregation and keep walkways clear.

Mandatory Flagman/Banksman use is required for:

- Reversing

- Blind spots / restricted visibility

- Tight access / congested work areas

- Shared pedestrian routes / crossings

- Loading/unloading operations

STOP rule: If the Banksman is not visible or the route is not clear → STOP movement immediately.

## 6.4 DELIVERIES & MATERIAL OFFLOADING

- Deliveries report to Yard Control / Supervisor.

- Offload only in designated laydown zones.

- Establish an exclusion zone during unloading and keep non-essential persons out of the line-of-fire.

- Use certified operators and approved lifting accessories where lifting is required.

- Confirm SWL, load stability, and ground condition before lifting or offloading.

- No lifting over people.

- Use tag lines where required for long/awkward loads, without placing persons in the line-of-fire.

HOLD POINT – Weather / Surface:
 Stop lifting when wind ≥ 9.72 m/s (≈ 35 km/h) or when surface/ground conditions are unsafe (slippery, soft ground, unstable laydown).

## 6.5 STORAGE & STACKING

- Stack materials on firm, level ground with suitable dunnage.

- Maintain safe stacking heights and stable configurations.

- Strap/chock cylindrical materials to prevent rolling.

- Separate flammables from ignition sources and store in approved areas.

- Keep emergency routes, fire points and access/egress clear.

STOP rule: Unstable or unsafe stack → STOP and re-stack safely before continuing.

## 6.6 MANUAL HANDLING

- Plan the lift and the route before handling.

- Use mechanical aids first (FLT/telehandler/trolley/pallet truck) where practicable.

- Use team lift for bulky or awkward items.

- Maintain neutral posture and avoid twisting; turn with feet.

- Push rather than pull where safe.

- Do not carry loads that block forward visibility; use a spotter if needed.

STOP rule: Stop and re-plan if the load is unstable, too heavy, visibility is restricted, or route becomes unsafe.

## 6.7 FABRICATION & WORKSHOP OPERATIONS

Includes cutting, grinding, drilling, welding and assembly.

Controls:

- Machines/tools must be guarded and inspected before use.

- Use correct discs/blades for the task and ensure they are in good condition and correctly fitted.

- Secure workpieces (clamps/vice) to prevent movement.

- Control sparks and hot particles; keep combustibles away.

- Provide suitable ventilation.

- Welding screens installed to protect others.

- Fire extinguishers available at hot work area.

- Hot Work Permit obtained where required by Al Laith procedures.

STOP rules:

- Stop if guard is missing, abnormal vibration occurs, unsafe sparks/combustibles cannot be controlled, or permit conditions are not met.

## 6.8 PAINTING & SHOT BLASTING

Painting:

- Review SDS before work and brief controls.

- Ensure ventilation suitable for the product used.

- Keep ignition sources away from flammables.

- Store paints/solvents in controlled areas; lids closed when not in use.

- Spill kits available and used if required.

- Dispose of waste and contaminated materials properly.

Shot Blasting:

- Segregate blasting zone and prevent unauthorised entry.

- Install containment (as applicable) to control dust/grit migration.

- Inspect hoses and connections before use.

- Full PPE worn as per RA (including respiratory protection where required).

STOP rules:

- Stop if containment fails, dust is uncontrolled, unauthorised persons enter the zone, or equipment defects are found.

## 6.9 ELECTRICAL, A/C & PLUMBING WORKS

Electrical:

- Authorised electricians only.

- Apply LOTO before work and verify isolation.

- Use RCD protection where required.

- Protect cables from damage, water, and trip hazards.

A/C:

- Verify unit weight and lifting method before moving/installing.

- Isolate electrical supply before work.

- Refrigerant handling by competent personnel only (as applicable).

Plumbing:

- Confirm isolation before cutting or breaking into systems.

- Apply controlled pressure testing as required.

- Check for leaks before handover.

STOP rule: Stop if isolation cannot be confirmed, unsafe condition exists, or competence is not verified.

## 6.10 WORK AT HEIGHT

- Use collective protection first (guardrails, proper platforms).

- Harness only where required and attached to a rated anchor point.

- Maintain an exclusion zone below elevated works where persons could be struck by falling items.

- No throwing materials or tools.

- Rescue arrangements must be briefed before start where fall arrest is used.

- Stop work during lightning or when wind/conditions make work unsafe.

## 6.11 SCAFFOLD & TEMPORARY STRUCTURES – MATERIALS & QUALITY CONTROLS

Applies to scaffold mock-ups and yard scaffolds.

### 6.11.1 Materials

- Tubes, fittings and boards inspected before use.

- No damaged, cracked, deformed or heavily corroded components.

- System components must be compatible and locking devices functional.

- Non-conforming materials tagged NOT FOR USE and quarantined.

### 6.11.2 Foundations & Base (HOLD POINT)

- Scaffold erected only on firm, level surface.

- Base plates and sole boards installed as required.

- No erection on soft ground or unstable slab.

- Ground/slab bearing capacity verified and documented (Design Dept / Facilities confirmation of allowable bearing pressure vs. calculated base loads)

HOLD POINT: Foundation and base arrangement verified before erection above base lift.

### 6.11.3 Stability & Platform Requirements

- Standards plumb and vertical.

- Ledgers and transoms correctly installed.

- Bracing and ties installed as required.

- Platforms fully boarded and properly supported.

- Guardrails and toe boards installed where fall risk exists.

- SWL/load class assigned by Design Department (or competent scaffold designer), referenced on design calculations, displayed on scaffold signage and handover certificate

- For weight-bearing / loading bay / storage platform scaffolds: loading controls enforced (maximum distributed load, point load prohibition, material stacking plan, supervision/monitoring)

- No unauthorised removal of ties/bracing.

### 6.11.4 Tagging & Inspection

- Incomplete scaffold: RED TAG – DO NOT USE.

- Completed scaffold: GREEN TAG after competent inspection.

Inspection frequency:

- Before first use

- Every 7 days

- After alteration

- After severe weather or impact

Records maintained in Scaffold Register.

### 6.11.5 Alteration & Dismantling

- Alteration only by competent scaffolders.

- RED tag during alteration; access controlled.

- Maintain stability at all times.

- Controlled top-down dismantling.

- No throwing of materials.

- Maintain exclusion zone below.
 (Refer to Section 7.0 for detailed sequence.)

## 6.12 END OF SHIFT CONTROL

- Remove waste and debris; leave area safe and tidy.

- Secure plant and isolate equipment as required.

- Close out permits/authorisations for the shift.

- Confirm emergency routes and fire points are clear.

- Supervisor completes final walkdown and signs off.

# 7.0 SCAFFOLD ERECTION, ALTERATION & DISMANTLING – SEQUENCE

This section defines step-by-step control for erection, alteration, inspection and dismantling at Al Laith Yard (DIC). Read with Section 6.

## 7.1 PRE-ERECTION VERIFICATION

Before erection starts, Scaffold Foreman and Yard Supervisor confirm:

- Approved drawing/sketch available (where required)

- Load class/SWL defined

- Materials inspected and suitable

- Work area segregated with exclusion zone

- Ground/bearing verified (as per 6.11.2)

- Scaffolders competent and briefed

- Approved scaffold design with calculations (mandatory for all load-bearing / non-standard / storage / loading bay / cantilever / sheeted / covered / suspended scaffolds or any configuration outside standard access)

HOLD POINT – Start of Erection: No erection until verified and authorised by Supervisor.

## 7.2 SETTING OUT & BASE INSTALLATION

- Mark scaffold footprint.

- Install sole boards where required.

- Position base plates centrally and fit base jacks (if used).

- Set first standards plumb and aligned.

HOLD POINT – Base Lift Stability: Verify standards plumb, base stable, no rocking/settlement, correct bay spacing before proceeding.

## 7.3 ERECTION – LIFT-BY-LIFT METHOD

### 7.3.1 First Lift

- Install ledgers/transoms; check level and square.

- Install initial bracing.

- Install full working platform and fit guardrails/midrails immediately.

### 7.3.2 Progressive Lifts

For each lift: standards → ledgers → transoms → bracing → full decking → guardrails/toe boards → plumb/level check.

## 7.4 TIES & STABILITY CONTROL

- Install ties as per design/guidance.

- Do not exceed free-standing height limits.

- Record ties where register used.

HOLD POINT – Before Reaching Design Height: Tie pattern verified, bracing complete, stability confirmed before handover.

## 7.5 ACCESS INSTALLATION

- Install access early.

- Ladders secured top/bottom, correct angle (~75°), extend 1m above landing, landing protected.

- Stair towers installed as per system requirements with handrails/midrails.

## 7.6 EDGE PROTECTION & PLATFORM CONTROL

Platforms must be fully boarded with guardrail (min 950mm), midrail, toe board (min 150mm), controlled gaps, and no unauthorised modifications.

## 7.7 COMPLETION INSPECTION & HANDOVER

Competent inspector verifies foundation, plumb/level, ties/bracing, platforms, edge protection, access, missing components, and SWL signage.

HOLD POINT – Handover:

- If compliant → GREEN SCAFFTAG issued and recorded.

- If not → RED TAG maintained, access restricted, rectify and re-inspect.
 No use until Green Tag issued.

## 7.8 USE & MONITORING

- No overloading beyond SWL.

- No unauthorised removal of components.

- No sheeting/banners unless approved.

- Supervisor daily visual checks.

## 7.9 PERIODIC INSPECTION

Before first use, every 7 days, after alteration, after severe weather, after impact/suspected instability. Records maintained.

## 7.10 ALTERATION PROCEDURE

- Inform Supervisor; install RED TAG; maintain stability.

- Follow drawing (where applicable).

- Re-inspect and re-issue GREEN TAG after alteration; update register.

## 7.11 DISMANTLING SEQUENCE

Controlled top-down method.

### 7.11.1 Pre-Dismantling

- Confirm scaffold no longer required.

- Remove loose materials/debris.

- Exclusion zone below.

- Remove GREEN TAG; install RED TAG.

- Confirm stability before starting.

### 7.11.2 Controlled Top-Down Removal

Reverse erection order. No throwing. Do not remove critical ties too early. Lower materials in a controlled manner or by mechanical means.

### 7.11.3 Final Clearance

Remove base plates/sole boards, check ground, remove exclusion zone after Supervisor approval, stack materials safely.

## 7.12 WEATHER CONTROLS (SCAFFOLD WORKS)

Stop erection/dismantling when:

- Wind ≥ 9.72 m/s (≈ 35 km/h)

- Gusting causes loss of control

- Lightning present

- Surfaces become unsafe
 Resume only when Supervisor verifies safe conditions.

## 7.13 EMERGENCY RESPONSE (WORK AT HEIGHT)

If fall, instability or dropped object incident occurs:

- STOP all work and secure the area

- Notify Supervisor and raise alarm as per yard procedure

- Implement rescue plan where fall arrest is used

- Do not reuse scaffold until inspected and formally released

| # 8.0 Support Work & Storage Arrangement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.1 | Temporary Supports and Props needed to facilitate the works |  |  | Details of any support work to be added as per the individual job requirements |  |  |  |  |  |  |  |  |  |  |
| 8.2 | Storage Arrangements |  |  | It is envisaged that all materials brought to site will be unloaded and brought to the job location and used within a 48hr window. Any materials stored temporarily at the job site will be placed on timber bearers, evenly distributed so as not to overload the slab (slab loading information to be provided by AL LAITH) and a physical barrier installed around the materials whilst not being used. |  |  |  |  |  |  |  |  |  |  |
| 8.3 | Permit to Work |  |  | PTWs are issued by the Client HSE/Authoriser and accepted by the Al Laith Supervisor. Validity and any extensions are stated on the permit. Signed PTWs are displayed at the workface, and originals are filed in the PTW Register. |  |  |  |  |  |  |  |  |  |  |
| 8.4 | SWL’s |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Permit to Work (PTW) Workflow & Control Sheet |  |  |  |
| --- | --- | --- | --- |
| Step | Activity | Responsible | Evidence / Record |
| 1. Request | Supervisor/PM submits PTW request with MS/RA, latest drawings, and area plan. Confirms required associated permits (e.g., lifting, MEWP/WAH, hot work, electrical isolation, work near/over water) and identifies interfaces with other trades. | Supervisor / PM | PTW request pack (MS/RA, drawings, area plan); interface checklist (if used) |
| 2. Review & Issue | Authorised Issuer reviews controls, area readiness, and interfaces; issues PTW with conditions, validity period, and any hold points. | Client/Main Contractor Authorised Person (Issuer) | Issued PTW with conditions & validity |
| 3. Acceptance | Al Laith Supervisor accepts PTW, briefs crew (TBT), confirms competence, and posts the live PTW at the workface/PTW board. | Al Laith Supervisor | Signed/accepted PTW; TBT record; PTW displayed |
| 4. Display & Monitor | Supervisor monitors compliance with PTW conditions and MS/RA, maintains exclusion zone and records required checks (e.g., wind log, inspections, housekeeping). | Al Laith Supervisor | Daily PTW check / Supervisor log; wind log; inspection records |
| 5. Stop / Suspend (if conditions change) | If scope/method/location/interfaces/weather/conditions change, or a permit condition cannot be met: STOP, make safe, and suspend/hand back PTW as per site procedure. Restart only after re-authorisation and re-brief. | Al Laith Supervisor + Issuer | Suspension note / re-issue record; re-brief/TBT record |
| 6. Extension (if needed) | Before expiry, Supervisor requests extension; Issuer confirms conditions remain valid and extends/reissues PTW with updated validity. | Issuer + Al Laith Supervisor | Extension authorisation; updated PTW displayed |
| 7. Close-out / Hand-back | On completion or end of shift (as required), area is made safe, barriers adjusted/removed as authorised, and PTW is closed/handed back. Outstanding hazards are communicated to the Issuer/site team. | Al Laith Supervisor + Issuer | PTW hand-back/close-out record; end-of-shift brief note |

| SWL & Load (Note: Enter platform SWL as kN/m² (preferred) or kg per bay/component as applicable. Cross-check against design drawing references before sign-off.) |  |  |  |
| --- | --- | --- | --- |
| Structure Name | SWL (kN/m² or kg) | Drawing Ref | Check |
| ALS-FAB-0095-01-REV01 SCAFFOLDING STRUCTURE.pdf | ALS-FA-6020 Scaffold Structure Calculations.pdf | First draftALS-FAB-0095-01-REV00 SCAFFOLDING STRUCTURE.pdf | Final check after calculations: ALS-FAB-0095-01-REV01 SCAFFOLDING STRUCTURE.pdf |

| # 9.0 Safety Requirements |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Safety Gears Required | Yes | No | Safety Equipment Required | Yes | No |
| Safety Helmet (Hard Hat) | √ |  | Safety Gloves – Cloth or leather | √ |  |
| Hearing Protection | AR |  | Safety Footwear – Boots | √ |  |
| Safety Glasses | √ |  | Dust Mask or Face Mask | √ |  |
| Coveralls | √ |  | Full Body Harness (Double Hook) > 1.8 meters | √ |  |
| High Visibility Vest – Orange for banksman and/or signallers | √ |  | Inertia Reel/Retractable | √ |  |
| Life Jackets | N/A |  | Life Line | AR |  |

√ = Required, R = Recommended, AR = As Required, N/A = Not Applicable; hearing protection selection will follow measured/expected noise levels and posted area signage.

| 9.1 | Specific Identified Residual Hazards |  |  | Refer to Risk assessment |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9.2 Required Personal Protective Equipment (select all applicable) |  |  |  | Safety Boot |  |  |  |  | Yes | Hard Hats |  |  |  |  | Yes |
|  |  |  |  |  |  |  |  |  | No |  |  |  |  |  | No |
|  |  |  |  | Eye Protection |  |  |  |  | Yes | Respiratory Protection |  |  |  |  | Yes |
|  |  |  |  |  |  |  |  |  | No |  |  |  |  |  | No |
|  |  |  |  | Safety Gloves |  |  |  |  | Yes | Hearing Protection |  |  |  |  | AR |
|  |  |  |  |  |  |  |  |  | No |  |  |  |  |  | No |
|  |  |  |  | His-Vis Clothings |  |  |  |  | Yes | Harness / lanyard |  |  |  |  | AR |
|  |  |  |  |  |  |  |  |  | No |  |  |  |  |  | No |
|  |  |  |  | Coveralls |  |  |  |  | Yes |  |  |  |  |  | AR |
|  |  |  |  |  |  |  |  |  | No |  |  |  |  |  | No |
| 9.3 | Hazardous Substances (select all applicable) |  |  | Very Toxic |  |  |  |  | Yes | Harmful / Irritant |  |  |  |  | Yes |
|  |  |  |  |  |  |  |  |  | No |  |  |  |  |  | No |
|  |  |  |  | Corrosive |  |  |  |  | Yes | Dangerous for the environment |  |  |  |  | Yes |
|  |  |  |  |  |  |  |  |  | No |  |  |  |  |  | No |
|  |  |  |  | Oxidizing |  |  |  |  | Yes | Highly flammable |  |  |  |  | Yes |
|  |  |  |  |  |  |  |  |  | No |  |  |  |  |  | No |
|  |  |  |  | Explosives |  |  |  |  | Yes | Anything else not listed above |  |  |  |  | Yes |
|  |  |  |  |  |  |  |  |  | No |  |  |  |  |  | No |
| 9.4 | Hot Weather and Heat Stress Controls - Follow the mandatory midday break and any Client/authority heat-stress rules. Plan heavy work for cooler periods where practicable and limit non-essential outdoor tasks during peak heat. - Provide cool potable drinking water at the workface and encourage frequent drinking. Provide shaded or air-conditioned rest areas. - Supervisors to monitor heat stress indicators during pre-start and mid-shift checks and record in the Daily Brief/TBT (and Heat Stress log where used). - Acclimatisation: new/returning workers shall be gradually introduced to hot work and monitored closely. Use buddy checks. - Sun protection: long sleeves where practicable, hat/helmet shade where permitted, and SPF-rated sun protection on exposed skin. - Heat illness trigger: cramps, dizziness, confusion, fainting, or hot dry skin → STOP, move to shade/cool area, cool and hydrate, call First Aider/medical support as required, and record the event. Records: Midday-break compliance, water/rest provision, and any heat-related interventions recorded in Daily Brief/TBT and Heat Stress log (if used). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.5 | Residual Risk Monitoring - The PM/Supervisor shall complete the Daily Residual Risk Checklist before work starts and after major changes (weather, layout, method, interfaces). Any new controls or pauses shall be recorded and briefed. Minimum check items: - Working at height: risk of falls during erection/alteration/dismantle; maintain edge protection, harness systems where required, and rescue readiness. - Manual handling: strain from heavy/awkward loads; use mechanical aids first, team lifts, and planned routes. - Traffic interface: plant–pedestrian conflict during deliveries/offloading; banksman, exclusion zones, and TMP controls in place. - Adverse weather: wind/rain/heat affecting stability and access; apply wind limits, stop during lightning/heavy rain/unsafe visibility, enforce heat controls. Drift control: If residual risk increases (controls not effective, near miss, unsafe condition), take immediate corrective action and update/re-brief at TBT. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.6 | # Night Work # Night work shall only proceed with adequate supervision, lighting, segregation, and communications. # Chargehand/Supervisor conducts regular rounds and records checks in the Night Work Supervision Log (lighting, exclusion zones, housekeeping, worker alertness). # Provide task and route lighting to maintain safe visibility; check emergency lighting at shift start where required. # Reflective PPE required. Maintain barriers/signage and apply site speed limits and banksman control for any vehicle movement. # Fit-for-duty checks at start and mid-shift. Fatigue indicators → reassign task, increase breaks, or stand-down and record. # No lone working for high-risk tasks (WAH, lifting, work near/over water, confined access areas). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.7 | # Tie Testing Plan # Purpose: Verify tie/anchor capacity where the scaffold design requires pull testing or where substrate conditions are uncertain. # Test a sample of ties in each zone/elevation and each tie type as specified by the design / engineer / project ITP (use the design requirement as the governing rule). # Method: pull-test using calibrated equipment to the required proof/design load; hold as required; record pass/fail. # Records to include: tie/anchor ID, location/grid, substrate, date, tester, test load and result. # Failures: replace the anchor and re-test as required. Increase testing in the surrounding area and escalate to PM/QHSE and the designer if repeated failures occur. # When: initial installation (where required), after significant alteration affecting ties, and after events that could affect anchors (impact, storm, suspected overload). - New substrate type, unknown wall strength, sheeted/covered scaffold, high duty/storage scaffold, unusual geometry, post-impact damage, or any design change affecting tie pattern/load distribution - Minimum sampling: at least 2-3 ties per zone/elevation and per tie type; increased sampling after failure or in areas of concern as per design/ITP # Records: File results in the scaffold inspection/handover pack (inspection register / tie register / test certificates) and make available for audits. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| # 10.0 Quality Monitoring |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Detailed Inspection & Test plan shall be submitted along with each method statement. |  |  |  |  |  |

| Activity | Who checks | Verification |
| --- | --- | --- |
| Erection, modification, dismantle of scaffold | Site Supervisor/tag inspector | Project Manager |

| # 10.1 Daily Monitoring Checklist |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The Site Supervisor will complete a Residual Risk Daily Checklist for the following: manual handling, scaffold modification/dismantling, hot weather, night work, live fueling/plant, and vehicle/pedestrian interface. Any “No” triggers immediate corrective action and re-brief. File with the Daily Briefing Form. |  |  |  |  |  |

| # 11.0 Emergency Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11.1 |  |  |  | Name of On-Site First Aider: |  |  |  |  |  | (Complete at prestart ) Name: Yadav Phone/radio: +971 56 401 5909 |  |  |  |  |
|  |  |  |  | First Aid Room Location: |  |  |  |  |  | Near truck entry by site services yard |  |  |  |  |
|  |  |  |  | Location of Nearest Hospital: |  |  |  |  |  | Cedars Jebel Ali International Hospital, 04-8814000 OR Advanced Care Medical Centre, +97143397664 |  |  |  |  |
| 11.2 | Muster Point: |  |  | Near truck entry by site services yard |  |  |  |  |  |  |  |  |  |  |
| 11.3 | Welfare Requirements |  |  | Provide potable water, shade/rest (hot-weather), sanitary facilities, and lighting for night work as applicable. |  |  |  |  |  |  |  |  |  |  |
| 11.4 | Services to be supplied by Others |  |  | First aid will be provided on site. In case of serious injury, the injured person shall be transferred to the hospital identified in Section 11.1 in accordance with site procedures. |  |  |  |  |  |  |  |  |  |  |
| 11.5 | Emergency Numbers |  |  | 998 (Ambulance), 997 (Fire) |  |  |  |  |  |  |  |  |  |  |
| 11.6 | Emergency Procedures |  |  | In case of emergency Al Laith staff operatives & Sub contractors follow the action plan as mentioned in the site Health & Safety plan. |  |  |  |  |  |  |  |  |  |  |

| # 12.0 Access and Egress point |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| - Access and egress routes shall follow the approved site layout / traffic management plan (Ref. 2.2, where applicable). - The Project Manager / Supervisor shall verify routes at the start of each shift and after adverse weather or any site layout change. - Routes shall be kept clear of obstructions, stored materials, waste, and trailing cables. Barriers, signage, and pedestrian/vehicle segregation shall be maintained. - Access routes shall be adequately lit, have safe, stable walking surfaces, and include edge protection on ramps, stairs, and elevated walkways where required. - Emergency exits, fire points, and muster routes shall remain clear and unobstructed at all times. - Trenches, services, and cables crossing routes shall be covered, protected, and secured, and re-checked after rain or wind. - Any change to access/egress routes shall be briefed at pre-start, updated on site signage/plans where used, and old or conflicting signage removed. HOLD POINT: If safe access or egress cannot be maintained, STOP work and re-plan before continuing. |  |  |  |  |  |
| 13.0 Manual Handling |  |  |  |  |  |
| - Manual handling shall follow the hierarchy of control: eliminate where possible, use mechanical aids first, then team handling, with PPE as the last line of defence. - Before handling, carry out a quick check of load weight/shape, sharp edges, route condition and lighting, and individual capability. - Use trolleys, dollies, skates, pallet trucks, or lifting machinery where practicable. Materials shall be delivered as close as possible to the point of use to minimise double-handling. - Team lifts shall be used for bulky or awkward loads. The route and set-down point shall be agreed before lifting. - Keep loads between knee and shoulder height where practicable and avoid twisting. - No manual handling shall be carried out on ladders. - Wear suitable gloves where required to protect against sharp edges and to maintain grip. - STOP and re-plan if visibility is restricted, the load becomes unstable, or the route becomes unsafe. Records: Any additional controls (e.g. team lift, mechanical aid, route change) shall be recorded on the Daily Residual Risk Checklist and/or toolbox talk notes where required. |  |  |  |  |  |
| # 14.0 Environmental and Waste Management Arrangements |  |  |  |  |  |
| # Waste management & Pest Control # Manage waste to Client standards and the project Environmental Management Plan (EMP). # Segregate waste (reusable/recyclable/general/hazardous) using labelled containers/skips. # Hazardous waste/COSHH: store sealed and labelled with SDS available; use secondary containment; dispose via approved/licensed contractor and retain transfer notes/manifests. # Remove waste to the designated refuse area daily (or as scheduled) and do not obstruct access/egress. # Pest control: keep food waste in sealed bins and remove daily; follow client bait-station plan (where applicable) and report infestations to Facilities/QHSE. |  |  |  |  |  |
| Noise # Use quieter methods/plant where practicable; position equipment to reduce impact; rotate tasks and use barriers/screens where practical. # Hearing protection requirements shall follow the Risk Assessment and site rules. # Where monitoring or site conditions indicate elevated noise, implement signage and additional controls as required. # Maintain any noise monitoring records in the work pack where required. |  |  |  |  |  |
| # Spill Prevention and Response (Fuel, Oils, Paint) Prevention - Refuel only at designated attended points; engines OFF; use drip trays and approved hoses/nozzles. - Store fuels/oils in bunded areas (≥ 110% of the largest container). - Spill kits available and checked routinely (including vehicles where applicable). - Protect drains where relevant and keep SDS available where substances are used. Response (record in Spill/Environmental Log) STOP → CONTAIN → PROTECT DRAINS → NOTIFY Supervisor/QHSE → CLEAN UP → DISPOSE via approved contractor → RECORD / CAPA If soil/water contamination is suspected: stop, contain, notify QHSE, and arrange approved clean-up and disposal; retain disposal records. |  |  |  |  |  |
| Dust - Use low-dust methods where practicable; use on-tool extraction/LEV where available, otherwise water spray/sheeting as suitable (without creating slip hazards). - Use RPE where required by the Risk Assessment. - Stop work if dust affects visibility or creates unacceptable exposure. - Clean using vacuum or damp methods; no dry sweeping. - Controls briefed at daily TBT and recorded where required. |  |  |  |  |  |
| # 15.0 Third party Interactions |  |  |  |  |  |
| - Al Laith shall coordinate works to protect third parties, adjacent trades, facility users, and the public. - Daily coordination via TBT and/or coordination meeting to confirm work zones, interfaces, communication method, and hold points. - Maintain segregation using barriers, signage, and banksman/marshal where required. - Avoid conflicting work at height: no work over others/public unless protected (e.g., overhead protection/debris netting) or re-sequenced. - Visitor control: sign-in, escort, required PPE, and brief on exclusion zones and emergency arrangements. - Any changes affecting the public interface (route closures, access changes, noisy work, lifting near routes) shall be communicated to the Client/Main Contractor in advance where practicable. Attachments / Records (as applicable) - Site TMP extract / drawings reference - Daily Briefing & Residual Risk Checklist - Housekeeping / Environmental Log (incl. spill log) - Rescue kit inspection sheet (where WAH or water interface applies) |  |  |  |  |  |

| # 16.0 Permits Key Resource |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Items Attached: (Please indicate in the box below) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| No | Accidents & Incidents |  |  | Yes |  |  |  |  |  | No |  |  |  |  |
| 16.1 | Sketches/ Date sheets |  |  | x |  |  |  |  |  |  |  |  |  |  |
| 16.2 | Certification of Plant/operator competency |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16.3 | Program of Works |  |  | x |  |  |  |  |  |  |  |  |  |  |
| 16.4 | Risk Assessments |  |  | x |  |  |  |  |  |  |  |  |  |  |
| 16.5 | Excavation permit |  |  |  |  |  |  |  |  | x |  |  |  |  |
| 16.6 | Daily briefing form |  |  | x |  |  |  |  |  |  |  |  |  |  |
| 16.7 | Emergency contact details |  |  | x |  |  |  |  |  |  |  |  |  |  |
| 16.8 | Emergency arrangements |  |  | x |  |  |  |  |  |  |  |  |  |  |
| 16.9 | Rescue plan |  |  | x |  |  |  |  |  |  |  |  |  |  |
| 16.10 | Method Statement Amendments and Review |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16.11 | Wind Log, Spill Kit Checklist, Residual Risk Checklist, Housekeeping Log, and Scafftag Inspection Register to be kept at the workface or site office for audit. |  |  |  |  |  |  |  |  |  |  |  |  |  |

| # 17.0 Emergency Contact Details |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Michael Clark Operation Director - EPC M: 050 152 5411 E: michael.clark@allaith.com Reinhart Swart Yard Manager - EPC M: 050 653 9375 E: Reinhart.Swart@allaith.com Johan Tredoux QHSE Manager M: 056 179 3189 E: johan.tredoux@allaith.com |  |  |  |  |  |

| # 17.0 Review and Amendments |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| REVIEW DATES: This Method Statement will be reviewed every 3 months, and after any incident, near miss, or significant change in scope or conditions. LAST REVIEW DATE: 16/02/2026 NEXT REVIEW DATE: 16/05/2026 AMENDMENTS AUTHORIZED BY: Michael Clark Johan Tredoux AMENDMENTS COMMUNICATED TO: All operatives involved |  |  |  |  |  |

## 18.0 Emergency Procedures

All personnel shall complete site induction before starting work and understand: alarm method, emergency numbers, muster point(s), first aid arrangements, and emergency access routes.

Daily pre-start / TBT confirmation (as applicable): radio channel/backup method, emergency numbers and muster point, and rescue arrangements for the day’s activities (WAH / lifting interfaces / work near or over water).

### Immediate Actions (Any Emergency / Incident)

- STOP work and make the area safe (isolate energy sources/plant movements where applicable).

- Raise the alarm via site procedure (call site emergency number / notify Site Control/HSE).

- First aid by trained first aider; request medical support as required.

- Rescue only if it can be done safely, by competent persons, with suitable equipment.

- Keep access clear for emergency responders and guide them to the location.

### Reporting / Preservation

- Report all incidents and near misses as per site and company procedure.

- Preserve the scene/equipment where safe and instructed (do not disturb unless required for rescue or to prevent further harm).

- Abu Dhabi (where WAH applies): rescue arrangements shall align with the site Fall Prevention Plan (OSHAD CoP 49) where applicable.

## 18.1 Work at Height Rescue (When Fall Arrest is Used)

HOLD POINT: Where fall arrest is used, a task-specific rescue arrangement shall be in place before work starts.

Pre-start (confirm and record where required):

- Recovery approach for the work location (nearest safe platform/ground).

- Rescue kit available, accessible, and checked.

- Suitable rescue anchor point(s) identified (no handrails/guardrails/uncertified fixtures).

- Clear access/egress maintained for recovery and medical response.

- Communications confirmed (radio/phone, emergency contact method).

If a fall / suspension occurs:

- STOP work, make area safe, and establish an exclusion zone.

- Raise the alarm and request site emergency response / ambulance as required.

- Recover the casualty only using the agreed rescue method from the nearest safe position.

- Do not use plant to lift people unless a purpose-designed, approved man-riding/rescue system is authorised under site rules.

After recovery:

- Treat as a medical emergency; follow first aider and emergency services instructions.

- Record incident/near miss; quarantine any loaded rescue equipment and do not re-use until checked/replaced as required.

- Affected scaffold/structure shall be inspected and released by a competent person before re-use.

Records (where required):

- Rescue brief at TBT; rescue kit inspection record; drills where required by client/site.

## 18.2 Rescue Without Specialist Remote Kit

- Notify Site Control/HSE and request emergency services as required.

- Do not expose rescuers to additional fall risk; use the safest available access method permitted (e.g., MEWP or adjacent protected platform).

- Recover casualty to a safe area and provide first aid/medical response as required.

- Brief this contingency at TBT and ensure competent supervision and first aid arrangements are in place.

18.4 Guide to Formulating a rescue plan

Is the scaffold a routine structure founded from the ground or surface?

No

(Special structure e.g. Hung, Suspended, Cantilever, Truss-out, Bridge, Mat or Temporary Roof)

Yes

(e.g. Independent,

Tower or Birdcage)

Faller suspended within or adjacent to the scaffold structure?

Faller suspended in free-space below or away from the scaffold?

Yes

Yes

No

Is the workplace accessible by Powered Access?

Formulate Powered Access Rescue Plan by trained operatives.

Formulate Rescue Plan without specialist rescue equipment by trained operatives.

Formulate Fully Assisted Rescue Plan with specialist rescue equipment by trained operatives.

Can equipment be made immediately available with trained Operators?

Can remote rescue equipment be used?

Yes

(e.g. Crane & Basket, MEWP etc.)

Yes

Formulate Remote Rescue Plan with specialist rescue equipment by trained operatives.

No

No

No

(e.g. abseiling ascender or descender type required)

Yes

(E.g. retrievable inertia block, remote rescue kits etc.)

| # 19.0 Method Statement Briefing Record |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Briefing delivered by………………………………………………………….…… Position…………Supervisor or Project Manager………………….. Date…………………………………………………………………………………. We (the undersigned) have read and understood the attached method statement and will comply with the specified requirements and control measures. If the work activity changes or deviates from that originally envisaged, we will seek further advice and request an amended method statement. |  |  |  |  |  |

| Name (Print) | Signature | Date |
| --- | --- | --- |