| Risk Assessment Procedural Requirements |  | System Document Control # EV-MS-RA-000 |
| --- | --- | --- |
|  |  | Revision: 000 |
|  |  | RA: 1000 |
|  |  | Original Assessment Date: 17/02/2026 |
|  |  | Last Review Date: 17/02/2026 |
|  |  | Next Review Date: 17/05/2026 |
|  |  | Activity Location: Saih Shuaib 3 Dubai Industrial City, Dubai, UAE |
| Responsible Leader: Reinhart Swart Responsible Member: Calo Lategan | Prepared by: Calo Lategan | Approved by: |
| Process: All Activities in the Yard | Date: 18/02/2026 | Date: |
| Division: Events / Site Services | Signature: | Signature: |

PPE requirements for the operation

| Safety Gears Required | Yes | No | Safety Equipment Required | Yes | No |
| --- | --- | --- | --- | --- | --- |
| Safety Helmet (Hard Hat) | √ |  | Safety Gloves – Cloth or leather | √ |  |
| Hearing Protection | AR |  | Safety Footwear – Boots | √ |  |
| Safety Glasses | √ |  | Dust Mask or Face Mask | √ |  |
| Coveralls | √ |  | Full Body Harness (Double Hook) > 1.8 meters | √ |  |
| High Visibility Vest – Orange for banksman and/or signallers | √ |  | Inertia Reel/Retractable (working on suspended, cantilever, or leading-edge structures; AR for other tasks as per RA) | √ |  |
| Life Jackets | N/A |  | Life Line | AR |  |

√ = Required, R = Recommended, AR = As Required, N/A = Not Applicable

This Risk Assessment (RA) sets out the identified hazards, risk evaluations, and control measures for Al Laith Yard activities at Dubai Industrial City (DIC), including scaffold operations, lifting activities, yard traffic, workshop operations, and associated works. This RA is valid only when signed and approved by the Responsible Person and relevant authorised signatories.

Implementation of all control measures identified in this RA remains the responsibility of the Project Manager and/or Site Supervisor. The QHSE Department provides advisory, monitoring, and auditing support but does not assume operational control or implementation responsibility. For works undertaken within the UAE, applicable federal and emirate-level legislation and codes shall prevail, this includes relevant Dubai Municipality Codes of Construction Safety Practice, Trakhees EHS requirements (where applicable), and any other competent authority requirements. Recognised good practice standards such as BS EN 12811-1 and NASC TG20 shall be applied where scaffold works are involved.

This RA does not replace statutory approvals, authority permits, or licensing requirements where applicable. This document is a live document and shall be reviewed and revised: Following any structural or process change, Following significant changes in staffing or yard operations, After any incident, accident, or near miss, At intervals not exceeding three (3) months.

Control measures shall be selected and implemented in accordance with ISO 45001 (Clause 8.1.2 – Hierarchy of Controls): Elimination → Substitution → Engineering Controls → Administrative Controls → Personal Protective Equipment (PPE).

LIKELIHOOD (L)

Risk Rating

RR 1 = 25-30 = Intolerable

RR 2 = 20-24 = High

RR 3 = 13-18 = Substantial

RR 4 = 11-12 = Moderate

RR 5 = 7-10 = Tolerable to Moderate

RR 6 = 1-6 = Tolerable

RISK MATRIX

Key to Risk Assessment: (L = LIKELIHOOD, S = SEVERITY, RR – RISK RATING)

| L | Likelihood | S | Severity | RR | Resultant |
| --- | --- | --- | --- | --- | --- |
| 5 | Likely to occur repeatedly. | 5 | Fatality / Permanent Disability. | 16 – 25 | Work may not proceed – consider alternative methods. |
| 4 | Likely to occur more than once. | 4 | Major Injury / Long term Absence. | 10 – 15 | Re – assess mitigation measures to reduce the risk. |
| 3 | Likely to occur. | 3 | LTI injury / Temporary Disability. | 6 – 9 | Work may proceed subject to implementation of mitigation measures, if no alternative available. |
| 2 | Unlikely to occur. | 2 | Injury requiring additional medical treatment. | 1 – 5 | Acceptable – Work may proceed. |
| 1 | Extremely unlikely to occur. | 1 | First Aid (e.g. Bruising, abrasion, etc.) |  |  |

Review frequency: every 3 months, and after any incident/near miss or significant change in scope/conditions.

| 1 | Pre-Task Verification & Permit Control | H1 Untrained or incompetent personnel assigned to task. H2 Expired, invalid, or unverifiable competency/training records. H3 Incomplete or missing MS/RA, drawings, or work instructions at workface. H4 Incorrect or expired Permit to Work (where required). H5 Failure to conduct pre-start briefing / Toolbox Talk. H6 Inadequate task planning or failure to identify interfaces (plant, WAH, lifting, SIMOPS). H7 Poor document control or use of superseded documents. H8 Incorrect, damaged, or non-conforming materials used. H9 Failure to establish exclusion zones or segregation before work starts. H10 Failure to complete Daily Residual Risk Check. H11 Failure to apply Management of Change when conditions vary. | 1 Injury or serious injury to workforce or authorised yard personnel. 2 Unsafe use of plant, scaffold, or equipment. 3 Structural instability or unsafe condition due to incorrect method. 4 Property or asset damage. 5 Regulatory non-compliance and audit findings. | All yard personnel Supervisors Plant operators Scaffolders Other authorised workers within yard | 5 | 4 | 20 | B1 Competency verification Only trained and competent personnel shall be assigned tasks. Supervisor shall verify valid certificates, licences, and training records before allocation. B2 Site induction & briefing All personnel must complete yard induction. Supervisor shall deliver pre-start briefing covering: MS/RA, scope, sequence, exclusion zones, plant interface, emergency arrangements, and stop-work triggers. B3 Document control Latest approved MS, RA, drawings, and permits (where required) must be available at workface. Superseded documents removed. B4 Permit control (where applicable) Required permits (WAH, hot work, lifting, electrical isolation, etc.) shall be obtained, briefed, and displayed before starting. B5 Inspection & record control Maintain inspection records including plant pre-use checks, scaffold inspection register (where applicable), toolbox attendance sheets, and permit records. B6 Materials verification Inspect materials on delivery and before use. Damaged or non-conforming items tagged “NOT FOR USE” and quarantined. B7 Segregation before start Exclusion zones, pedestrian segregation, and barriers must be installed before commencing lifting, scaffold works, fabrication, or plant operations. B8 Management of Change (MoC) Any deviation in method, layout, staffing, load, weather, or plant → STOP WORK. Review MS/RA, update if required, re-brief and re-authorise before restart. B9 Daily Residual Risk Check Supervisor shall complete and record daily residual risk check before work starts and after significant changes. B10 Heat stress controls (where applicable) Follow yard heat stress procedure including water provision, rest breaks, and monitoring. . | 5 | 1 | 5 | Project Manager, QHSE Officer, Supervisor | R1 MS & RA Procedure / Management of Change R2 Training & Competency Register R3 Permit to Work Register (where applicable) R4 Toolbox Talk Record R5 Daily Residual Risk Checklist R6 Inspection Registers (plant/scaffold where applicable) R7 Material Quarantine / NCR Log R8 Heat Stress Protocol / Log |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | Yard Traffic & Vehicle Movement | H1 Driver/operator not competent or not authorised for the vehicle/plant being used. H2 Invalid/expired UAE driving licence (where applicable) or medically unfit driver/operator. H3 Fatigue, illness, impairing medication, alcohol or drugs affecting safe driving/operation. H4 Poor yard route conditions (soft edges, uneven ground, potholes, dust, reduced visibility). H5 Pedestrian interface and other yard vehicles/plant creating collision/struck-by risk. H6 Distraction while driving/operating (mobile phone/device use). H7 Reversing or manoeuvring in restricted areas (blind spots, tight turns, congested zones). H8 Loads transported on vehicles are not secured (load shift / falling objects). H9 Poor signage, lighting or housekeeping on routes (especially in low visibility). H10 Fuel/oil leaks or spills during yard movements or refuelling causing slip/environment risk. | 1 Road traffic collision/vehicle incident causing serious injury or fatality. 2 Struck-by / impact injuries to pedestrians or other yard personnel. 3 Vehicle damage, rollover/bogging and operational disruption. 4 Load shift causing injury or property damage. 5 Non-compliance with yard rules and UAE legal requirements. | Drivers/operators Passengers (if any) Pedestrians / other yard personnel Visitors Other vehicle/plant operators | 5 | 4 | 20 | ### B1 Competence & authorisation Only competent and authorised drivers/operators. Verify licence/authorisation and medical fitness where required. ### B2 Fitness for duty Driver/operator must report fatigue, illness, or medication that may impair performance. Zero tolerance for alcohol/drugs. Do not drive/operate if unfit. ### B3 Yard driving rules Seatbelts worn (where fitted). Follow yard speed limits, designated routes and signage. No passengers unless seat provided. ### B4 No distractions No mobile phone/device use while driving/operating. No texting. Stop in a safe place if communication is required. ### B5 Route and ground condition Drive to conditions. Reduce speed on uneven ground, dusty areas, poor visibility or near soft edges. Stop and assess if ground bearing is uncertain. ### B6 Pedestrian segregation Maintain pedestrian segregation and crossings. Do not proceed if pedestrians are within the path or operating envelope. ### B7 Mandatory Banksman for high-risk movements Banksman required for reversing, blind spots, tight manoeuvres, congested areas, shared routes, and loading/unloading interfaces. If Banksman not visible → STOP immediately. ### B8 Reversing control Reverse only under Banksman control where visibility is restricted. Use alarms/beacons where fitted and ensure mirrors/cameras are functional. ### B9 Vehicle condition (pre-use checks) Daily pre-use checks completed (tyres, brakes, lights, horn, mirrors, reverse alarm/beacon). Defects → tag out and do not use until cleared. ### B10 Load security Do not overload. Secure loads using suitable restraints. Check stability before movement and re-check after first movement/stop where applicable. ### B11 Deliveries and access control Deliveries only to designated areas. Keep emergency routes clear. Maintain exclusion zones during offloading/loading. ### B12 Breakdown/emergency Stop in a safe place, activate hazard lights, warn others, notify Supervisor/Yard Control, and do not attempt unsafe recovery. ### B13 Spill prevention and response (where applicable) Refuel only in designated areas using drip trays where required. Spill kit available. Any leak/spill → stop, contain, protect drains, notify Supervisor/QHSE, clean up and record. | 5 | 1 | 5 | Project Manager Site Supervisor Driver / Operator Logistics / Transport Coordinator (if applicable) QHSE Officer | R1 Driving for Work / Land Transport Procedure R2 Alcohol & Drugs Procedure R3 Driver/operator authorisation & licence check record R4 Vehicle daily pre-use check record R5 Yard traffic rules / Traffic Management Plan (if issued) R6 Load securing SOP R7 Spill response procedure / spill log (if applicable) R8 Breakdown / emergency procedure R9 Incident/near-miss reporting and CAPA register |
| 3 | Handling, Loading & Plant Interface | H1 Untrained/unauthorised plant operator or banksman; poor supervision during lifting/loading/offloading. H2 Struck-by, crush or pinch points from forks, boom/mast, loads, stillages, trailers and pinch zones. H3 Sharp edges and protrusions on materials, banding/strapping and damaged pallets causing cuts. H4 Uncontrolled operating area and poor segregation leading to pedestrian–plant interface (blind spots, reversing, crossings). H5 Ramp/dock plate failure, dislodgement or unsafe trailer interface causing falls or load drop. H6 Plant instability (tip-over/roll-over) due to overload, gradients, turning with raised load, poor ground, soft edges. H7 Mechanical failure (hydraulics, tyres, brakes, steering, forks/attachments) leading to loss of control or dropped load. H8 Slips/trips on wet ramps/decks, uneven ground, poor footing, or poor housekeeping in loading areas. H9 Load instability (load shift/falling load) due to poor securing, poor pallet condition, or incorrect handling. H10 Falls from trailer/flatbed/loading area during access or positioning. H11 Manual handling strain during positioning, strapping, or guiding loads. H12 Striking overhead/fixed objects (canopies, beams, temporary services) during lifting or travel with raised boom/load. | 1 Minor/major injuries (MSI, cuts/lacerations, bruising, fractures, crush injuries). 2 Serious injury/fatality (struck-by, crush, dropped load). 3 Property/asset damage and operational disruption. 4 Non-compliance with yard rules / PTW requirements (where applicable). | Operatives Drivers Plant operators Banksman / spotters Visitors / other yard personnel Other contractors (if present) | 5 | 4 | 20 | B1 Competence and authorisation Only trained, authorised and medically fit operators and banksman/spotters. Verify licence/authorisation validity and record. B2 Supervisor control and pre-task plan Loading/offloading and lifting activities only under Supervisor coordination with a briefed plan. Confirm roles (operator, banksman, zone controller), route, exclusion zones, landing/drop zones and stop-work triggers. B3 Plant pre-use inspection Pre-use checks each shift: SWL/capacity plate, forks/attachments and locking, mast/boom, chains/hydraulics, tyres, brakes, steering, lights/horn, reverse alarm/beacon, mirrors/camera (if fitted). Defects → isolate/tag-out and do not use. B4 Attachments and lifting method Use only approved attachments and follow OEM guidance including any derating. No improvised lifting. Suspended loads only where permitted and controlled under an approved lifting method/permit where required. B5 Exclusion zone / line-of-fire control Establish and maintain an exclusion zone with barriers/cones/signage. No pedestrians within the operating envelope or under raised loads. No lifting over people. B6 Banksman control Banksman mandatory for reversing, blind spots, tight manoeuvres, trailer/ramp interface and when pedestrians are nearby. Use agreed hand signals or dedicated radio channel. If banksman not visible → STOP. B7 Safe operating rules Seatbelt worn (where fitted). Obey yard speed limits and routes. No passengers unless seat provided. No phone/device use while moving. B8 Load handling and travel Keep within SWL. Use pallets/stillages/cages where possible. Forks fully engaged and load stable; tilt back slightly. Travel with load low. Maintain clear view; reverse if load blocks view and it is safe to do so. B9 Turning and gradients Do not turn with raised load/boom. Keep speed low. Avoid cross-slope travel. Stay within OEM gradient limits. Lower load/boom before turning. B10 Trailer security (where applicable) Trailer/vehicle stationary, brakes applied, wheels chocked where required. Position on firm level ground. Maintain a controlled loading area. B11 Ramp/dock plate control (where applicable) Use rated ramps/dock plates, correctly positioned and secured to prevent slip/dislodgement. Inspect before use. Banksman to control the interface. B12 Working on trailers / fall prevention Provide safe access (steps/ladder) where required. Maintain 3-point contact. No jumping down. Keep edges protected where practicable. B13 Wet/slippery surface control Keep ramps/decks clear of mud/water where practicable. Apply anti-slip measures where required. Suitable safety footwear worn. Stop if footing becomes unsafe. B14 Weather controls Stop for lightning. In poor visibility, heavy rain or unsafe wind, slow/hold operations and reassess ground/ramp conditions before continuing. B15 Manual handling controls Use mechanical means first. Team lift where required. Plan positioning/strapping. Keep hands clear of pinch points and never place body parts under suspended/unstable loads. B16 Sharp edge controls Wear suitable gloves (cut-resistant where required). Remove banding/straps safely and dispose of sharp waste properly. B17 Laydown and stacking control Laydown/stack only in designated bays. Keep stacks stable and within safe height. Keep routes clear. B18 Parking and isolation Park in designated area. Lower forks/boom, apply brake, control keys as required. Report defects/near misses and close out actions. | 5 | 1 | 5 | Project Manager Site Supervisor Plant Operator Banksman / Spotter QHSE Officer | R1 Forklift/Telehandler/MEWP pre-use checklist / inspection record R2 Operator and banksman competence/authorisation record R3 Yard traffic rules / pedestrian segregation plan (TMP if issued) R4 Banksman briefing / radio channel record (where used) R5 Attachment register / OEM guidance (if applicable) R6 Lifting plan / lifting permit (where required) R7 Load securing SOP / delivery control procedure R8 Housekeeping inspection record R9 Weather hold / wind log (where applicable) R10 Incident/near-miss reporting and CAPA register |
| 4 | Storage & Stacking of Materials | H1 Collapse, roll or spread of stacked materials or bundled components due to unstable stacking. H2 Slips, trips and falls caused by banding, off-cuts, loose materials or poor housekeeping in storage areas. H3 Pinch and crush points when separating nested items or during shifting of stacked members. H4 Cuts and lacerations from sharp edges, banding straps, wires or damaged materials. H5 Strap recoil or whip-back during cutting of banding. H6 Manual handling strain from lifting, separating or repositioning heavy or awkward components. H7 Line-of-fire exposure from rolling or shifting members during release of restraints. H8 Overloading of storage areas or stacking beyond safe height leading to instability. | 1 Minor/major musculoskeletal injuries, cuts/lacerations, bruising or impact injuries. 2 Crush injuries from collapsing or shifting materials. 3 Property damage due to unstable stacking. | Labouring staff Operatives Store personnel Drivers Supervisors | 3 | 3 | 9 | B1 Designated storage area Store and open bundles only in designated laydown areas on firm, level ground. Keep emergency routes and access/egress clear at all times. B2 Stack stability control Stack materials on stable ground using dunnage where required. Maintain safe stacking heights. Use chocks or restraints to prevent rolling. Do not stack on slopes or soft edges. B3 Exclusion zone / line-of-fire control Establish a controlled zone when opening bundles or cutting banding. Keep persons clear of recoil or roll path. B4 Controlled cutting of banding Use appropriate cutting tools. Stand to the side and keep face and body out of the recoil path. Cut straps in a controlled manner and release tension gradually. B5 Manual handling control Use mechanical aids where practicable. Team lift heavy or awkward components. Keep loads close to body and avoid twisting. Stop and re-plan if unstable. B6 Safe separation of materials Separate nested members slowly and one at a time. Maintain stable stacking during removal. Do not aggressively pull or drag materials. B7 Housekeeping and waste control Remove banding, off-cuts and packaging immediately. Dispose of sharp waste in designated containers. Keep storage areas tidy. B8 PPE requirements Wear safety footwear, cut-resistant gloves where required, eye protection and hard hat. Long sleeves recommended where sharp edges exist. B9 Material inspection and quarantine Inspect materials during unloading and storage. Damaged or non-conforming items tagged “NOT FOR USE” and quarantined. B10 Supervisor monitoring Supervisor to monitor stacking condition and stop work if instability or unsafe condition is observed. | 4 | 4 | 16 | Logistics Manager Project Manager Site Supervisor Store Controller | R1 Housekeeping inspection / daily housekeeping record R2 Manual Handling Procedure / SOP R3 PPE issuance/inspection record R4 Material quarantine / NCR log R5 Yard storage layout plan (where applicable) R6 Incident/near-miss reporting and CAPA register |
| 5 | Manual Handling | H1 Lifting beyond personal capability (overexertion). H2 Awkward, bulky or unbalanced loads reducing visibility or control. H3 Poor posture (bending, twisting, over-reaching) during lifting or carrying. H4 Repetitive handling leading to fatigue or strain. H5 Slips/trips during carrying due to poor underfoot conditions. H6 Dropped loads causing foot or crush injuries. H7 Manual handling on ladders or unsafe access routes. H8 Poor coordination during team lifts. | 1 Minor/major musculoskeletal injuries (strain, sprain, back injury). 2 Dropped loads causing crush injuries or fractures. 3 Slips, trips and falls during carrying. | All yard personnel Labouring staff Scaffolders Fabrication workers Store personnel | 3 | 4 | 12 | B1 Planning before lift Assess load weight, shape, route and landing area before lifting. Remove obstacles and ensure good lighting. B2 Hierarchy of control Eliminate manual handling where practicable. Use mechanical aids first (trolley, pallet truck, forklift, telehandler). B3 Team lifting Use team lift for heavy or awkward loads. Nominate one person to coordinate lift and movement. B4 Correct lifting technique Keep load close to body. Maintain neutral spine. Bend knees, not back. Avoid twisting — turn with feet. B5 Load size control Break loads into smaller manageable units where practicable. B6 Route condition Check floor/ground condition before movement. Keep route clear of debris, cables and slip hazards. B7 No manual handling on ladders Materials shall not be manually carried while climbing ladders. Use controlled lifting methods or mechanical means. B8 PPE Wear suitable gloves for grip and protection. Wear safety footwear to protect against dropped loads. B9 Fatigue control Rotate tasks and take breaks where repetitive lifting is required. Stop work if strain develops. B10 Supervisor monitoring Supervisor to intervene if unsafe lifting behaviour or overexertion is observed. | 3 | 1 | 3 | Site Supervisor Foreman Charge Hand | R1 Manual Handling Procedure / SOP R2 Training & Competency Register R3 Toolbox Talk / Daily Briefing Record R4 Daily Residual Risk Checklist R5 Housekeeping Inspection Log R6 Incident/Near Miss Reporting & CAPA Register |
| 6 | Fabrication & Workshop Operations (Tools, Cutting, Grinding, Welding) | H1 Unsafe use of hand/power tools (incorrect tool selection, poor technique, missing guards). H2 Abrasive wheel/disc failure (wrong disc, damage, incorrect fitting, over-speed). H3 Cuts/lacerations from sharp edges, swarf, burrs, blades and rotating parts. H4 Entanglement with rotating equipment (loose clothing, gloves, long hair). H5 Flying particles and sparks causing eye/face injury. H6 Noise exposure from grinding/cutting operations. H7 Hot work hazards (sparks/heat) causing fire, burns or ignition of nearby combustibles. H8 Inadequate ventilation causing fume exposure (welding fumes, solvent vapours, dust). H9 Electric shock from damaged cables, poor connections, wet conditions, or lack of RCD protection where required. H10 Poor housekeeping (offcuts, trailing leads, clutter) causing slips/trips and secondary injuries. H11 Uncontrolled work area / poor segregation exposing others to sparks, arc flash, flying debris or noise. H12 Gas cylinder hazards (where applicable): incorrect storage, unsecured cylinders, damaged regulators/hoses. | 1 Minor/major injuries (cuts, burns, eye injury, fractures). 2 Serious injury/fatality (fire, explosion, electric shock, severe trauma). 3 Occupational exposure (fumes/dust/noise) leading to ill health. 4 Property/asset damage and operational disruption. | Workshop operatives Welders/fabricators Helpers/labourers Supervisors Other authorised yard personnel nearby | 5 | 4 | 20 | B1 Competence and supervision Only competent persons to operate tools and perform welding/hot works. Supervisor to confirm competence and brief the task method and work area controls. B2 Pre-task inspection of tools and equipment Inspect tools before use. Guards must be fitted and functional. Defective tools, damaged cables or plugs → tag out and remove from service. B3 Correct consumables and fitting (grinding/cutting) Use the correct disc/blade for the tool and material. Check disc condition and expiry. Fit correctly and tighten as per manufacturer guidance. Do not use damaged or incorrect discs. B4 Workpiece securing Secure workpieces using a vice/clamps. Do not hold small pieces by hand during cutting/grinding. B5 PPE (task specific) Wear safety glasses or face shield as required, suitable gloves, safety footwear, and hearing protection where noise is present. Welding PPE as required (helmet, gloves, protective clothing). B6 Segregation and screens Set up a controlled work zone. Use welding screens/spark shields to protect others. Keep non-essential persons out of the line-of-fire. B7 Hot work control (where required) Hot Work Permit required where Al Laith rules apply. Remove combustibles from the area. Keep minimum 2 x DCP extinguishers available and accessible. Maintain fire watch where required. B8 Ventilation and fume control Provide adequate ventilation. Use local extraction where available. Stop work if fumes/dust become excessive or visibility is affected. B9 Electrical safety Use RCD protection where required. Protect cables from damage, water and sharp edges. Keep connections off wet ground. Do not use damaged leads or makeshift joints. B10 Housekeeping Keep work area tidy. Remove offcuts and swarf frequently. Route cables to reduce trip hazards. No buildup of combustible waste near hot works. B11 Cylinder control (where applicable) Store cylinders upright and secured. Fit correct regulators. Keep caps on when not in use. Separate oxygen and fuel cylinders as per procedure and keep away from heat sources. B12 Stop-work triggers Stop work immediately if: guard missing, disc vibration/abnormal noise, uncontrolled sparks/fire risk, ventilation inadequate, unauthorised persons enter the work zone, or permit conditions are not met. | 5 | 1 | 5 | Project Manager Site Supervisor Workshop Foreman QHSE Officer | R1 Tool and equipment pre-use inspection records R2 Training/competency records (tool safety / welding where applicable) R3 Hot Work Permit register (where applicable) R4 Fire extinguisher inspection record / fire point check R5 Electrical inspection/PAT records (where used) R6 Housekeeping inspection log R7 SDS register (where consumables/chemicals used) R8 Incident/near-miss reporting and CAPA register |
| 7 | Painting, Coating & Surface Preparation (Including Shot Blasting) | H1 Exposure to hazardous substances (paints, thinners, solvents, hardeners) causing skin/eye irritation or respiratory effects. H2 Inhalation of vapours/fumes due to poor ventilation. H3 Fire risk from flammable paints/solvents and ignition sources (hot works, sparks, smoking). H4 Spills/leaks during mixing, transfer, or use causing slip hazard and environmental contamination. H5 Pressure hazards (if using spray equipment): hose failure, injection injury, uncontrolled discharge. H6 Shot blasting exposure: airborne dust/grit, rebound particles, and eye/skin injury. H7 Noise exposure from blasting and associated equipment. H8 Equipment failure (hoses, couplings, nozzles, compressors) causing loss of control or whipping hoses. H9 Poor segregation allowing unauthorised entry into blasting/painting zone. H10 Manual handling strain from moving paint drums, blasting pots, hoses, and equipment. H11 Electrical hazards from lighting/tools in work area (damaged cables, wet conditions) where applicable. H12 Waste mismanagement (contaminated rags, empty containers, blasting media) causing exposure or non-compliance. | 1 Minor/major injury (eye injury, dermatitis, respiratory irritation). 2 Serious injury/fatality (fire, explosion, solvent inhalation, high-pressure injection). 3 Occupational ill health due to repeated exposure (fumes/dust/noise). 4 Environmental contamination and clean-up costs. | Painters / blasters Helpers / labourers Supervisors Other authorised yard personnel nearby | 5 | 4 | 20 | B1 Competence and briefing Only trained personnel to carry out painting/blasting and operate spray/blasting equipment. Daily TBT covers SDS controls, segregation, fire precautions, PPE/RPE and stop-work triggers. B2 SDS / chemical control Review SDS before use. Use correct mixing ratios and compatible products. Keep containers closed when not in use. No decanting into unlabelled containers. B3 Ventilation Ensure suitable ventilation for the task and products used. Stop work if fumes/vapours become excessive or cause symptoms. B4 Fire prevention / ignition control Keep paints/solvents away from ignition sources. No smoking in work zone. Hot work not permitted near flammables unless controlled by separation and permit requirements. Fire extinguishers available at the workface as required. B5 Spill prevention and response Use drip trays where required. Spill kit available. Any spill → stop, contain, protect drains, notify Supervisor/QHSE, clean up and dispose correctly, and record. B6 Equipment inspection (spray/blast systems) Inspect hoses, couplings, clamps, regulators, pots and nozzles before use. Damaged equipment → tag out. Secure hoses to prevent whipping. Depressurise before maintenance or disconnection. B7 Segregation and access control Establish a controlled zone for painting/blasting with barriers and signage. No unauthorised entry. Keep others out of line-of-fire and rebound areas. B8 Dust control / containment (blasting) Use containment where required to prevent dust/grit migration. Stop if containment fails or dust becomes uncontrolled. B9 PPE and RPE Wear task-specific PPE as per RA and SDS: goggles/face shield where required, gloves, coveralls, safety footwear. Use suitable RPE for vapours/dust (fit checked as required). Hearing protection where noise is present. B10 Housekeeping and waste control Clean as you go. No dry sweeping where dust present—use vacuum/damp methods. Dispose of contaminated rags/filters/empty containers/blast media in designated waste streams. B11 Manual handling controls Use mechanical aids for drums/pots where practicable. Team lift awkward items. Manage hoses to reduce strain and trip hazards. B12 Stop-work triggers Stop work if: ventilation inadequate, vapours/dust uncontrolled, spill cannot be contained, fire risk uncontrolled, equipment defects found, or unauthorised entry occurs. | 5 | 1 | 5 | Project Manager Site Supervisor Painting/Blasting Foreman QHSE Officer | RR1 SDS register / COSHH file (where used) R2 Equipment pre-use inspection records (compressor/hoses/blast pot/spray kit) R3 Hot Work permit record (where applicable) R4 Spill response procedure / spill log (where applicable) R5 Waste transfer / disposal records (where applicable) R6 Housekeeping / environmental inspection record R7 Training/competency records (painting/blasting/RPE awareness) R8 Incident/near-miss reporting and CAPA register |
| 8 | Electrical Works & Isolation (LOTO) | H1 Unauthorised or incompetent person carrying out electrical work. H2 Failure to isolate electrical supply before work starts (no LOTO / wrong isolator). H3 Unexpected energisation during work (lock not applied, lock removed, poor control of keys). H4 Electric shock or electrocution from live parts, damaged insulation, or incorrect testing. H5 Arc flash / burns due to short circuit, faulty equipment, or unsafe connection. H6 Use of damaged electrical tools, plugs, sockets, or cables. H7 Use of electrical equipment in wet conditions or where water/wet suppression is present. H8 Trip hazards from trailing cables and temporary leads. H9 Incorrect use or absence of RCD protection where required. H10 Contact with overhead/temporary electrical services during lifting/plant movements (where applicable). H11 Fire caused by overload, poor connections, damaged cabling, or unsuitable equipment. | 1 Electric shock, burns, or minor/major injury. 2 Serious injury/fatality (electrocution, arc flash). 3 Fire and property/asset damage. 4 Operational disruption and non-compliance. | Electricians Operatives working nearby Supervisors Other authorised yard personnel | 5 | 4 | 20 | B1 Competent electricians only Electrical works only by authorised/competent electricians. Supervisor to verify competence before task allocation. B2 LOTO mandatory before work Isolate supply at the correct source before starting. Apply lock and tag. Control keys. No work to start until isolation is confirmed. B3 Prove dead / verification Test to confirm the circuit is de-energised using an approved tester. Where required, test the tester before and after proving dead. B4 Permit/authorisation (where applicable) Obtain and comply with any required electrical permit/work authorisation as per Al Laith procedures. B5 Condition of tools and equipment Use tools and leads that are suitable and in good condition. Inspect before use. Defects → tag out and remove from service. B6 RCD protection Use RCD protection where required (temporary power, portable tools, damp areas). Do not bypass protection devices. B7 Keep electricals away from water Keep connections and extension joints off the ground and away from wet areas. Stop work if water/wet suppression creates exposure risk to electrical equipment. B8 Cable management Route cables to avoid damage and trips. Use cable ramps/overhead routing where practicable. Do not run cables through water or across sharp edges. B9 Safe distances to overhead/temporary services Check area for overhead or temporary services before lifting/plant movement. Maintain safe clearance and apply controls where required. B10 Fire prevention Do not overload sockets or use makeshift joints. Keep combustibles away from electrical work area. Extinguisher available where required. B11 Stop-work triggers Stop work immediately if: isolation cannot be confirmed, live parts exposed, damaged tools/cables identified, RCD not functioning, wet conditions create risk, or unauthorised persons enter the electrical work zone. | 5 | 1 | 5 | Project Manager Site Supervisor Authorised Electrician QHSE Officer | R1 Electrical Safety Procedure / LOTO Procedure R2 Electrical permit/authorisation records (where applicable) R3 Training/competency records (authorised electrician list) R4 Electrical tool/equipment inspection or PAT record (where used) R5 RCD test/inspection record (where used) R6 Cable management / temporary power arrangement record (where used) R7 Fire extinguisher inspection record (where applicable) R8 Incident/near-miss reporting and CAPA register |
| 9 | A/C & Refrigerant Handling | H1 Unauthorised or incompetent persons handling A/C equipment or refrigerant. H2 Electrical shock due to failure to isolate supply before work starts. H3 Manual handling / lifting injury due to heavy or awkward A/C units and components. H4 Dropped load / crush injury during lifting, positioning, or removal of units. H5 Refrigerant release causing cold burns, eye injury, or respiratory irritation (especially in poor ventilation). H6 Asphyxiation risk in poorly ventilated areas if refrigerant is released (as applicable). H7 Use of incorrect tools or unsafe tool use during installation/removal. H8 Damage to refrigerant lines causing uncontrolled release. H9 Fire risk from hot works near refrigerant/oils or from electrical faults (where applicable). H10 Slips/trips from trailing cables/hoses and poor housekeeping during installation/servicing. H11 Incorrect re-connection/pressure testing leading to leaks and rework exposure. | 1 Minor/major injury (MSI, cuts, cold burns, eye irritation). 2 Serious injury (electric shock, crush injury, significant exposure). 3 Damage to equipment/property and operational disruption. 4 Environmental impact and non-compliance (uncontrolled refrigerant release). | A/C technicians Helpers / operatives Supervisors Other authorised yard personnel nearby | 5 | 4 | 20 | B1 Competence and authorisation Refrigerant work to be carried out by competent A/C technicians only. Supervisor to verify competence/authorisation before task allocation. B2 Electrical isolation (LOTO) Isolate electrical supply before work. Apply LOTO where required and verify isolation prior to starting. B3 Lifting plan for heavy items Verify unit weight and lifting method before movement. Use mechanical aids (forklift/telehandler/chain block) where practicable and authorised. B4 Safe lifting and positioning Maintain exclusion zone during lifting/positioning. Use tag lines where required. No person under suspended loads. Use stable supports and keep hands clear of pinch points. B5 Refrigerant handling controls Recover refrigerant using approved recovery equipment where required. Do not vent refrigerant to atmosphere. Use correct fittings and control connections to prevent release. B6 Ventilation Ensure adequate ventilation during refrigerant work. Stop work if ventilation is inadequate or symptoms occur. B7 PPE Wear suitable gloves and eye protection for refrigerant handling. Additional PPE as required by task (e.g., face shield where splash risk exists). B8 Leak prevention and testing Protect refrigerant lines from damage. Pressure/leak testing only by competent persons using correct equipment. Confirm leak-free before handover. B9 Tools and equipment checks Use correct tools only. Inspect tools and gauges before use. Defective equipment → tag out. B10 Housekeeping and trip control Manage hoses/cables to prevent trips. Keep work area tidy and clear of offcuts and packaging. B11 Stop-work triggers Stop work immediately if: isolation cannot be confirmed, lifting method is unsafe, refrigerant leak/release occurs, ventilation is inadequate, or unauthorised persons enter the work zone. | 5 | 1 | 5 | Project Manager Site Supervisor A/C Technician / Foreman QHSE Officer | R1 LOTO / Electrical isolation procedure (where applicable) R2 A/C technician competence/authorisation record R3 Lifting plan / lifting permit (where required) R4 Equipment pre-use inspection records (lifting equipment / recovery unit) R5 Refrigerant handling procedure / OEM instructions (where used) R6 Leak/pressure test record (where applicable) R7 Incident/near-miss reporting and CAPA register R8 Waste disposal / environmental log (where applicable) |
| 10 | Plumbing & Pressure Testing | H1 Unauthorised or incompetent persons carrying out plumbing or pressure testing. H2 Failure to isolate water supply or connected systems before cutting or modification. H3 Sudden release of pressurised water/air during testing (hose whip / joint failure). H4 Burst pipe, fitting or test plug due to over-pressurisation. H5 Slips/trips from water discharge or leaks during testing. H6 Use of defective hoses, gauges or fittings. H7 Manual handling strain from pipes, fittings, pumps and test equipment. H8 Contact with contaminated/stagnant water (where applicable). H9 Inadequate segregation during pressure test creating struck-by risk. H10 Inadequate securing of pipework during test leading to movement. H11 Poor communication during pressurisation and release. | 1 Minor/major injury (MSI, cuts, impact injuries). 2 Serious injury (struck-by from hose whip or burst fitting). 3 Slip/fall injuries due to water discharge. 4 Property/asset damage due to uncontrolled release or flooding. | Plumbers Helpers / operatives Supervisors Other authorised yard personnel nearby | 5 | 4 | 20 | B1 Competence Plumbing and pressure testing to be carried out by competent personnel only under Supervisor control. B2 Isolation before work Confirm isolation of water supply or connected systems before cutting or modification. Verify and brief before restart. B3 Controlled pressure testing Use calibrated gauges and suitable rated hoses/fittings. Do not exceed specified test pressure. Increase pressure gradually. B4 Secure pipework and components Ensure all pipework, caps and fittings are properly secured and supported before pressurisation. B5 Exclusion zone during testing Establish a controlled area during pressurisation. No persons to stand directly in front of joints, caps or test points. B6 Controlled pressurisation and release One designated person to control pump/valve. Communicate clearly before pressurising and before releasing pressure. B7 Equipment inspection Inspect hoses, gauges, couplings and pumps before use. Defective equipment to be tagged out and removed. B8 Slip prevention Manage water discharge to prevent pooling. Maintain housekeeping and dry walking routes where practicable. B9 Manual handling controls Use mechanical aids where practicable. Team lift longer/heavier pipe sections. Avoid twisting and overreaching. B10 Stop-work triggers Stop work immediately if uncontrolled leak, excessive movement, equipment failure, pressure spike, or unauthorised entry into test zone occurs. | 5 | 1 | 5 | Project Manager Site Supervisor Plumbing Foreman | R1 Plumbing method statement R2 Pressure testing procedure R3 Equipment inspection record (gauges/hoses/pumps) R4 Daily residual risk checklist / toolbox talk record R5 Incident/near-miss reporting and CAPA register R6 Housekeeping inspection log |
| 11 | Work at Height (General Yard Activities) | H1 Falls from edges, platforms, steps or incomplete working surfaces. H2 Unsafe access/egress (incorrect ladder use, unsecured ladders, poor landing). H3 Working from unsuitable positions (over-reaching, standing on guardrails, working off unstable objects). H4 Open edges, openings or fragile surfaces not identified or protected. H5 Dropped objects striking persons below (tools, materials). H6 Use of fall arrest without suitable anchors or with defective equipment (where fall arrest is used). H7 MEWP misuse (where used): over-reaching, gate open, poor ground/bearing, incorrect harness connection. H8 Adverse weather affecting stability and visibility (wind, rain, lightning, poor visibility). H9 Poor segregation below workface (people entering drop zone). H10 No rescue arrangement in place where fall arrest is used. | 1 Minor/major injuries (sprains, fractures, head injury). 2 Serious injury/fatality from fall from height. 3 Serious injury to others from dropped objects. 4 Property/asset damage from falling items. | Workers at height Workers below work area Supervisors Other authorised yard personnel Visitors (if present) | 5 | 4 | 20 | B1 Competence and briefing Only trained and competent persons to work at height. Daily TBT covers: task method, access/egress, exclusion zones, dropped object controls, weather limits, and emergency arrangements. B2 Hierarchy of control Avoid work at height where practicable. Use collective protection first (proper platforms, guardrails, toe boards) before relying on harness. B3 Safe working platforms Use stable, correctly erected platforms or scaffold decks. Do not work from incomplete platforms or unprotected edges. B4 Access and egress control Use proper access (stairs preferred where available). If ladders are used: correct type/length, good condition, secured, correct angle, and extend above landing. Maintain 3-point contact. B5 Prohibition on unsafe positioning No working from guardrails, loose materials, or makeshift supports. No over-reaching. Reposition access equipment instead. B6 Edge protection and openings Protect all edges and openings as soon as they are created. Covers/guards must be secured and able to withstand loads. B7 Dropped object prevention Keep tools/materials away from edges. Use toe boards where applicable. Use tool lanyards where practicable. No throwing of tools/materials. B8 Exclusion zone below Establish and maintain an exclusion zone beneath the work area where there is a risk of falling objects. Prevent unauthorised entry. B9 Fall arrest controls (only where required) Use harness/fall arrest only when collective protection is not reasonably practicable. Connect only to rated anchor points/lifelines. Pre-use checks each shift; defective equipment tagged out. B10 MEWP controls (where used) MEWP operated only by authorised operators. Pre-use checks completed. Gate closed. No over-reaching. Confirm ground bearing and keep within OEM limits. Use harness with restraint lanyard attached to designated anchor. B11 Weather stop-work triggers Stop for lightning. Stop/hold work at height if wind, rain, or poor visibility makes footing or handling unsafe. Resume only after Supervisor verifies conditions are safe. B12 Rescue readiness (HOLD POINT) Where fall arrest is used: task-specific rescue arrangement must be in place and briefed before work starts (roles assigned, anchor confirmed, rescue kit available where required, communications confirmed). If rescue readiness is not in place → STOP. | 5 | 1 | 5 | Project Manager Site Supervisor Foreman QHSE Officer | R1 Work at Height procedure R2 Rescue arrangements / task rescue plan (where fall arrest is used) R3 Competence records (WAH / MEWP where applicable) R4 Toolbox talk / daily briefing attendance R5 Harness and anchor inspection records (where used) R6 Scafftag / inspection register (where applicable) R7 Weather/wind hold record (where applicable) R8 Incident/near-miss reporting and CAPA register |
| 12 | Scaffold Erection & Structural Stability | H1 Scaffold erected by untrained/unauthorised personnel or without competent supervision. H2 Erection without approved drawing/sketch or without defined duty/load class/SWL (where required). H3 Unstable foundation / inadequate bearing (soft ground, uneven slab, voids, edges) causing settlement or collapse. H4 Design calculations missing or inadequate for weight-bearing/non-standard/load-bearing scaffold structures (e.g., loading bays, tanks, cantilevers, birdcages, sheeted scaffolds, suspended sections) H4 Incorrect base set-up (missing/incorrect sole boards, base plates, poor levelling, incorrect bay spacing). H5 Incorrect erection sequence / incomplete lifts creating instability during build-up. H6 Missing/incorrect bracing, ties, locking devices or loose connections (couplers/cups/wedges). H7 Incomplete or unsafe platforms (gaps, uneven decking, missing transoms/supports, unsecured boards). H8 Missing/insufficient edge protection (guardrails, midrails, toe boards) during erection/use. H9 Unsafe access/egress (incorrect ladders, unsecured access, poor landing platform, no stair control where required). H10 Dropped objects / line-of-fire injuries during erection (materials/tools falling to areas below). H11 Unauthorised access or use of incomplete scaffold (no RED tag, barriers not maintained). H12 Adverse weather affecting stability/handling (wind/rain/lightning/poor visibility) during erection or before handover. | 1 Minor/major injuries (MSI, cuts, bruising, fractures). 2 Serious injury/fatality due to fall from height or scaffold collapse. 3 Dropped object injuries to people below. 4 Property/asset damage and operational disruption. | Scaffolders Supervisor / Foreman Competent scaffold inspector Other authorised yard personnel nearby Visitors (if present) | 5 | 4 | 20 | B1 Competence and supervision Scaffold erection only by competent scaffolders under Scaffold Foreman/Supervisor control. Competence verified before start. B2 Design / duty clarity Erect to approved drawing/sketch where required. Confirm duty/load class/SWL and any tie pattern requirements before erection starts. B3 HOLD POINT – Pre-erection verification Before starting: materials inspected, work area segregated, exclusion zone set, ground/bearing verified, access method confirmed, and workforce briefed. B4 Design Mandatory – All weight-bearing/non-standard scaffolds All scaffold structures used for load-bearing, storage, loading bays, tank scaffold support, sheeted/covered arrangements, cantilevers, birdcages, or configurations outside standard access scaffolds shall be erected ONLY against approved scaffold design with calculations issued by Design Department (MS 7.1). Approved drawings and design calculations must be available at the workface. B5 Ground/Slab Bearing Verification (MS 6.11.2, 7.1) Before erection above base lift, obtain documented ground/slab bearing capacity confirmation. Bearing check must verify that base loads transferred through sole boards/base plates are within allowable ground/slab bearing capacity. Document required: “Ground/Slab capacity confirmation” from Design Dept or Facilities must be filed in scaffold pack. B6 SWL/Load Class Governance & Loading Controls (MS 6.11.3, 7.7) SWL/load class shall ONLY be assigned by Design Department or competent scaffold designer and must be referenced on design drawings, displayed on handover certificate, and shown on SWL signage. FOR WEIGHT-BEARING/STORAGE PLATFORM SCAFFOLDS: Develop loading plan including maximum distributed load limits, point load prohibitions, material stacking rules, and maximum persons permitted. Supervision must enforce loading controls. B7 Tie Testing – Mandatory Triggers (MS 7.4) Tie testing (calibrated proof load testing per tie design) is mandatory when: (1) new substrate type, (2) post-impact/anchor damage suspected, (3) sheeted/high duty class, (4) unusual geometry or non-standard config, (5) unknown wall strength. Minimum sampling per design/ITP. Record all pass/fail results in tie register. B4 HOLD POINT – Foundations and base lift Erect only on firm, level surface. Use base plates and sole boards as required. Verify no rocking/settlement and correct B8 Design Evidence Package (Audit Requirement – Design Checklist) Design Evidence Package must be compiled and available at workface before scaffold erection. Package contents (Design Evidence Checklist): - Design drawing (revision-controlled, e.g., ALS-FA-6020) - • Scaffold structure calculations (reference: ALS-FAB-0095-01-Rev00 or equivalent, checked by Design Dept, dated) - • Imposed load class and SWL justification (calculated load capacity vs. design loads) - • Tie pattern diagram and anchor requirements (tabulated/drawing) - • Base bearing pressures (kPa or equivalent, certified safe for ground/slab type) - • Ground/slab capacity confirmation (Design Dept / Facilities certification) - • Handover certificate (signed by competent inspector, GREEN Scafftag issued) - All documents must be filed in scaffold pack and available for inspection/audit verification. bay spacing before proceeding above base lift. B5 Lift-by-lift erection sequence Erect progressively maintaining stability at all times. Install ledgers/transoms/bracing per lift. Do not leave partially built unstable bays. B6 Stability controls – ties/bracing/connections Install bracing and ties as required by design/guidance. Ensure couplers/cups/wedges are correctly locked/tightened. No unauthorised removal of ties/bracing. B7 Platform integrity Platforms fully decked and properly supported. Boards/panels secured. Control gaps and protect openings immediately. B8 Edge protection Install guardrails/midrails and toe boards as soon as practicable and before releasing any area for use. B9 Safe access/egress Provide safe access early (stairs preferred where available). If ladders used: correct type, secured, correct angle, extend above landing and protected landing. B10 Dropped object and line-of-fire control Maintain exclusion zone beneath active workface. Do not throw materials. Use controlled raising/lowering methods. Tool lanyards where practicable. B11 Tagging and access control Incomplete scaffold RED TAG and physically blocked. Scaffold not to be used until inspected and GREEN tagged. B12 Inspection regime Competent person inspections: before first use, every 7 days, after alteration, after severe weather or impact. Records maintained in scaffold register. B13 HOLD POINT – Handover No use until final inspection completed and GREEN Scafftag issued with duty/SWL displayed where required. B14 Weather controls Stop erection during lightning and when wind/visibility makes handling unsafe. Apply yard wind trigger where used (e.g., 9.72 m/s) or strictest site/design limit. Secure incomplete scaffold before leaving. B15 Stop-work / MoC Any deviation, damage, missing components, unexpected movement, change in layout/loads/access/weather → STOP, make safe, review MS/RA, re-brief, re-inspect, re-authorise. | 5 | 1 | 5 | Project Manager Site Supervisor Scaffold Foreman Competent Scaffold Inspector QHSE Officer | R1 Approved drawing/sketch / duty-load class-SWL confirmation (where required) R2 Scaffold inspection / Scafftag register (incl. 7-day, post-alteration, post-weather) R3 Pre-use material inspection / quarantine log R4 Toolbox talk / daily briefing attendance R5 Daily residual risk checklist R6 Weather/wind hold record (where applicable) R7 Work at Height procedure + rescue arrangements (where fall arrest is used) R8 Incident/near-miss reporting and CAPA register |
| 13 | Scaffold Alteration & Dismantling | H1 Alteration/dismantling carried out by untrained/unauthorised persons or without competent supervision. H2 Scaffold accessed/used by others during alteration (no RED tag / barriers not maintained). H3 Incorrect alteration/dismantling sequence causing loss of stability (not controlled top-down / not reverse of erection). H4 Premature removal or relocation of ties, braces or key components leading to instability/collapse. H5 Dropped objects during alteration/dismantling (tools, fittings, boards) striking persons below. H6 Uncontrolled lowering of materials (no system, no landing zone, throwing materials). H7 Falls from height during alteration/dismantling due to missing edge protection or unsafe access. H8 Slips/trips on platforms due to debris, loose components and poor housekeeping. H9 Unstable incomplete bays/lifts during progressive strip-back. H10 Adverse weather affecting safe control (wind/rain/lightning/poor visibility). H11 Plant/vehicle interface during removal/loading of scaffold materials (where applicable). H12 Poor communication / unclear roles during dismantling and lowering operations. | 1 Falls from height causing serious injury/fatality. 2 Partial/full collapse causing serious injury/fatality and property damage. 3 Dropped objects causing major injury to people below. 4 Minor/major injuries (MSI, cuts, bruises, impact injuries). | Scaffolders Foreman / Supervisor Banksman (if used) Other authorised yard personnel nearby Visitors (if present) | 5 | 4 | 20 | B1 Competence and supervision Alteration/dismantling only by competent scaffolders under Scaffold Foreman/Supervisor control. B2 Permit/authorisation (where required) Obtain and brief any required permits/authorisations prior to alteration/dismantling where required by yard rules. B3 Pre-task briefing and roles Brief: exact scope, sequence, ties/bracing to remain, lowering method, landing zone, exclusion zones, weather limits and stop-work triggers. B4 RED tag and physical barriers (HOLD POINT) Before starting alteration/dismantling, change scaffold status to RED – DO NOT USE and physically barrier access points. Prevent use until completion and re-handover. B5 Exclusion/drop zone control Establish and maintain a barricaded exclusion/drop zone below and around the workface. Appoint a zone controller/banksman where needed. B6 Housekeeping before and during works Clear platforms of debris/off-cuts/loose fittings before and during alteration/dismantling. “Clean as you go”. B7 Controlled sequence / maintain stability Follow authorised sequence: dismantle top-down; alteration only as planned. Maintain a safe platform and stability at all times. B8 Ties/bracing and key components (critical control) Maintain minimum ties/bracing as required during strip-back. Remove/relocate ties and key components only in the authorised sequence and only when stability is assured. B9 Controlled lowering system Lower materials in a controlled manner (rope/gin wheel/lifting bag or mechanical means as approved). No throwing materials. B10 Landing zone and stacking Use a stable designated landing area. Stack materials safely. Quarantine damaged components and tag “NOT FOR USE”. B11 Dropped object controls Tool lanyards where practicable, toe boards/containment where applicable, keep loose items away from edges, and maintain exclusion zone at all times. B12 Work at height controls Collective protection first. Where fall arrest is required, use harness and connect only to rated anchors/lifelines. Rescue arrangements must be in place where fall arrest is used. B13 Weather stop-work Stop for lightning. Stop/hold external alteration/dismantling when wind/rain/visibility affects safe control. Secure incomplete works before leaving. Resume only after Supervisor verifies safe. B14 Stop-work / MoC Any deviation, unexpected movement, damage, missing ties, breach of barriers, or change in conditions → STOP, make safe, escalate, review controls, re-brief and re-authorise before restart. B15 Plant/traffic interface (where applicable) Coordinate loading/collection of materials. Use banksman and maintain segregation from pedestrians and operating plant. | 5 | 1 | 5 | Project Manager Site Supervisor Scaffold Foreman QHSE Officer | R1 Permit/authorisation procedure + register (where required) R2 Toolbox talk / briefing record + attendance R3 Tagging & inspection register (alteration/dismantling record) R4 Controlled lowering / dropped object controls procedure R5 Work at Height procedure + rescue arrangements (where fall arrest is used) R6 Weather/wind hold record (where applicable) R7 Housekeeping inspection log R8 Material quarantine / NCR log R9 Incident/near-miss reporting and CAPA register |
| 14 | Dropped Objects & Line-of-Fire | H1 Tools, fittings or materials falling from height during scaffold works, fabrication or WAH tasks. H2 Uncontrolled lowering of materials (no system / no landing zone). H3 Throwing materials during erection, alteration or dismantling. H4 Persons standing within line-of-fire during cutting, lifting, loading or release of materials. H5 Failure to maintain exclusion zone beneath overhead works. H6 Loose materials stored near edges of platforms. H7 Plant movement within drop zone (forklift/telehandler interface). H8 Poor housekeeping causing loose objects to become airborne or dislodged. H9 Unauthorised entry into restricted drop zone. H10 Adverse weather (wind/gusts) causing materials to shift or fall. | 1 Minor/major injuries (MSI, bruises, lacerations). 2 Serious injury/fatality from falling object impact. 3 Property/asset damage. 4 Secondary incidents (panic movement, plant collision). | All personnel Scaffolders Operatives Plant operators Supervisors Visitors (where applicable) | 5 | 4 | 20 | B1 Exclusion zone (primary control) Establish and maintain a clearly barricaded exclusion/drop zone beneath any overhead work. No unauthorised entry permitted. B2 Line-of-fire positioning Personnel must not stand directly below, behind or in the direction of potential movement or release of materials. Maintain safe positioning at all times. B3 No throwing materials Materials, fittings and tools shall never be thrown. All items must be lowered in a controlled manner. B4 Controlled lowering system Use rope/gin wheel/lifting bag or mechanical means where required. Designate a stable landing zone. B5 Tool control Use tool lanyards where practicable during work at height. Keep tools secured and not placed near edges. B6 Edge controls Install toe boards and edge protection where required. Keep loose materials away from platform edges. B7 Housekeeping Maintain “clean as you go” standard. Remove loose fittings, off-cuts and debris promptly. B8 Plant interface control No plant to operate within drop zone. Coordinate lifting/loading operations and maintain segregation. B9 Weather controls Stop overhead work during lightning or when wind/gusting affects control of materials. Secure incomplete works before leaving. B10 Supervision and stop-work Supervisor to monitor drop zones and line-of-fire risks continuously. If exclusion zone fails or unsafe positioning observed → STOP and restore controls before resuming. | 5 | 1 | 5 | Project Manager Site Supervisor Foreman | R1 Toolbox talk record (dropped objects / line-of-fire control) R2 Work at Height procedure R3 Controlled lowering / dropped object controls SOP R4 Daily residual risk checklist R5 Housekeeping inspection record R6 Incident/near-miss reporting + CAPA register |
| 15 | Rescue & Emergency | H1 No/unclear rescue plan, roles, or emergency arrangements. H2 Insufficient trained First Aiders / rescuers on shift. H3 Communication failure during an incident (no agreed method/channel). H4 Unsuitable anchors or incorrect rescue equipment setup (where fall arrest is used). H5 Rescue kit not available / not accessible at the workface (where required). H6 Plant/traffic interfaces during rescue creating secondary risk. H7 Untrained rescue attempts increasing risk to casualty/rescuers. H8 Delayed response / poor access for emergency services. | 1 Serious injury/fatality due to delayed or unsafe rescue. 2 Secondary incidents (struck-by/crush, falls) during rescue activity. 3 Property/asset damage and reputation impact. | Personnel Operatives Visitors (if present) Pedestrians (if present) | 5 | 4 | 20 | B1 Emergency arrangements briefed daily Daily briefing/TBT confirms: alarm method, emergency contact number(s), muster point(s), nearest first aid provision, and appointed First Aider(s) on shift. B2 Rescue roles and communications (where fall arrest is used) Before any WAH task using fall arrest, brief the task rescue plan and assign roles (Rescue Lead, rescuer(s), spotter/exclusion-zone controller). Confirm communication method (radio channel/backup). B3 Rescue readiness check before high-risk work (HOLD POINT) Before starting tasks where rescue may be required, confirm: suitable anchor points (rated/verified), clear casualty route to a safe location, and rescue kit available where required. If not in place → STOP. B4 Rescue equipment available and inspected Rescue kit to be accessible where required and inspected/recorded as per company/site system. Replace/repair any defective items. B5 Trained rescuers only Only trained/briefed personnel to carry out rescue using the provided equipment. Maintain first aid and rescue awareness records; drills completed where required. B6 Scene control / secondary risk prevention Establish and maintain an exclusion zone during rescue. Stop nearby activities and control interfaces (plant/traffic/lifting) to prevent secondary incidents. B7 Make plant/loads safe (where involved) If plant/lifting is involved, stop operations, park safely, apply brake, isolate as required, and make the area/load safe as per yard rules. B8 No lifting persons with plant unless authorised Do not use plant to lift people unless a purpose-designed, approved man-riding/rescue system is authorised under site/company rules. B9 Raise alarm and call for help Raise the alarm immediately and notify Site Control/HSE as applicable. Call emergency services as required by procedure. B10 Start rescue only if safe and competent If safe and within competence/equipment capability, commence recovery without delay. Do not attempt untrained/unsafe rescue. B11 First aid and handover First Aider treats as a medical emergency and follows site medical procedure and emergency services instructions until handover. B12 Preserve scene and quarantine equipment Preserve the scene where safe. Quarantine any rescue equipment/anchors/components involved and arrange competent inspection before re-use. B13 Reporting and corrective actions Report incidents/near misses as required. Record actions and close out through CAPA; brief learnings at TBT. B14 Staffing / no lone working No lone working for higher-risk tasks where rescue may be required (e.g., WAH, lifting, plant interface). Ensure adequate crew and supervision for a safe response. | 5 | 1 | 5 | Project Manager Supervisor Foreman QHSE Officer | R1 Site Emergency Plan / Medical Emergency Response Procedure R2 Rescue arrangements / task rescue plan (where fall arrest is used) R3 First aid training records and appointed First Aider list (shift coverage) R4 Toolbox talk / daily briefing sheets (emergency + rescue roles) R5 Rescue kit inspection records / equipment register R6 Permits/authorisations (where applicable: WAH / lifting) R7 Incident/near-miss reporting + CAPA register R8 Fire safety plan (where applicable) |
| 17 | Adverse Weather (Wind, Rain, Lightning, Heat) | H1 Heat stress from high temperature/humidity and PPE thermal burden. H2 Dehydration / electrolyte imbalance due to insufficient hydration. H3 Reduced concentration/poor judgement due to heat fatigue (unsafe acts/errors). H4 High winds/gusts affecting handling and control of materials (scaffold components, sheets, loads). H5 Rain/wet surfaces causing slips/trips and unsafe footing (ramps, platforms, yard routes). H6 Lightning risk during outdoor work and elevated works. H7 Poor visibility (dust/rain/night interface) increasing vehicle/plant and WAH risk. H8 Restart after weather without re-check of access routes, platforms, ground bearing, and structures. | 1 Heat exhaustion/heat stroke; dehydration; loss of consciousness. 2 Slips/trips/falls (including falls from height where applicable). 3 Struck-by/dropped object incidents due to loss of control in wind. 4 Serious injury/fatality in severe cases or secondary incidents. 5 Property/asset damage and operational disruption. | All yard personnel Scaffolders Plant operators / banksman Workshop operatives Visitors (if present) | 4 | 3 | 12 | B1 Work planning Plan work to reduce heat exposure: schedule heavy tasks for cooler periods and reduce non-essential outdoor work during peak heat where practicable. B2 Midday break / heat restrictions Apply mandatory midday break and follow UAE requirements and Al Laith yard rules for heat stress controls. B3 Rest and shade Provide shaded/ventilated rest areas (fans/AC where available). Encourage frequent rest breaks. B4 Hydration and electrolytes Provide cool potable drinking water at the workface. Encourage frequent drinking. Provide electrolytes where appropriate. B5 Acclimatisation and buddy checks New/returning workers to ramp up exposure gradually. Supervisors to apply buddy checks and monitor closely. B6 Heat illness response If symptoms occur (cramps, dizziness, confusion, fainting): STOP, move to shade/cool area, cool down, hydrate if conscious, call First Aider/medical support, and record. B7 Weather monitoring and daily brief Check forecast before shift. Brief weather warnings and stop-work triggers at TBT. Increase supervision when conditions change. B8 Wind stop-work STOP/hold wind-sensitive tasks when wind becomes unsafe. Apply the strictest limit from site/design/manufacturer rules (yard trigger may be used where applicable, e.g., 9.72 m/s). Secure materials and incomplete works before leaving. B9 Rain / poor visibility stop-work Stop work when footing or visibility becomes unsafe. Reassess routes, ramps, platforms, ladders, and housekeeping before restarting. B10 Lightning stop-work Stop outdoor work and work at height during lightning. Make area safe and wait for Supervisor authorisation to resume. B11 Slip/trip controls during adverse weather Maintain housekeeping and safe underfoot conditions. Remove standing water where practicable. Keep access routes clear and stable. B12 HOLD POINT – Post-weather re-check before restart After any weather hold (wind/rain/lightning), Supervisor to confirm workfaces are safe before resuming: - access/egress routes clear and stable - platforms/edge protection intact - ties/bracing and structure condition acceptable (where applicable) - ground bearing/laydown areas safe Record on Daily Residual Risk Checklist and/or permit sheet (where used). | 4 | 1 | 4 | Project Manager Site Supervisor Foreman QHSE Officer | R1 Toolbox talk / daily briefing record (heat + weather triggers) R2 Heat stress protocol / midday break compliance record (where used) R3 Weather/wind hold & restart record (where applicable) R4 Daily residual risk checklist R5 Housekeeping inspection log R6 Incident/near-miss reporting + CAPA register |
| 17 | Fuel Refilling | H.1 Fire, spillage H.2 Static/ignition sources; overfill; incorrect containers; fueling near watercourses | 1. Fatal and property damage | Crew Facilities | 5 | 5 | 25 | B.1 Do fueling of vehicles and heavy equipment after switching off the engine. B.2 Drip trays shall be used to avoid spillage of oil on the ground. B.3 Periodic maintenance should be done and documented. B.4 No smoking policy must be maintained. B.5 Ensure & maintain fire extinguisher in the vehicle. B.6 All dispensing or transferring of fuel will be attended for the duration of the operation. B.7 The attendant must be aware of proper fuel handling procedures to minimise the risk of a spill and shall continuously scan the area adjacent to the fuelling operation for possible leaks or spills. B.8 The transferring and dispensing of fuel will be done with pumping equipment, an approved hose, and top-fill nozzle. B.9 Fuel in designated bunded area only; keep spill kit & absorbents at point of use. B.10 Use approved containers; bond/ground where required; max fill limits; close caps/vents properly. B.11 Any spill: stop source, contain, notify Supervisor, log actions. B.12 No fueling within buffer distance of open water; if unavoidable, additional booms/absorbents staged. | 5 | 1 | 5 | Project Manager Site Supervisor Foreman | R.1 TBT on fuel refilling. R.2 Use drip trays during fueling and maintenance, keep spill kits at fueling points, inspect kits daily, and log spills or near misses with actions. Store fuels and oils in bunded areas to prevent soil and water contamination. |
| 18 | Heat Stress & Environmental Exposure | H1 High temperature/humidity causing heat stress and heat illness. H2 Dehydration / electrolyte imbalance due to insufficient fluids or long exposure. H3 PPE thermal burden and radiant heat increasing fatigue. H4 Reduced concentration/poor judgement due to heat (unsafe acts/errors). H5 Sun/UV exposure causing sunburn and long-term skin damage. H6 Dust exposure (yard dust; cutting/grinding dust where applicable) causing respiratory irritation. H7 Noise exposure from yard operations (tools, plant, blasting where applicable). H8 Fumes/vapours from paints/solvents/welding (where applicable) causing irritation. H9 Slips/trips due to sweat/condensation and wet/slippery surfaces (including water use for dust suppression). H10 Restart after heat/weather stress without re-check of controls (hydration, breaks, conditions). | 1 Heat exhaustion/heat stroke; dehydration; fainting. 2 Slips/trips/falls and secondary injuries due to fatigue or poor footing. 3 Respiratory irritation/illness from dust/fumes; hearing loss from noise exposure. 4 Serious injury/fatality in severe heat illness or secondary incidents (plant/WAH). | All yard personnel Supervisors / foremen Plant operators / banksman Workshop operatives Visitors (if present) | 4 | 4 | 16 | B1 Work planning Plan tasks to reduce heat exposure. Schedule heavy outdoor work for cooler periods where practicable and reduce non-essential outdoor work during peak heat. B2 Midday break / heat restrictions Apply mandatory UAE midday break and follow Al Laith yard heat-stress rules. Suspend or reduce work when heat conditions make safe execution difficult. B3 Rest and shade Provide shaded/ventilated rest areas (fans/AC where available). Encourage frequent rest breaks. B4 Hydration and electrolytes Provide cool potable drinking water at the workface. Encourage frequent drinking. Provide electrolytes where appropriate. B5 Acclimatisation New/returning workers to increase exposure gradually and be monitored more closely for the first days back. B6 Supervision / buddy checks Supervisors to monitor heat stress indicators and apply buddy checks. Rotate tasks and increase breaks as needed. B7 Heat illness response If symptoms occur (cramps, dizziness, confusion, fainting, hot dry skin): STOP, move to shade/cool area, cool down, hydrate if conscious, call First Aider/medical support, and record. B8 Sun/UV protection Use long sleeves where practicable, hat/helmet shade where permitted, and apply SPF-rated protection to exposed skin. B9 Dust/fume/noise controls (where applicable) Use low-dust methods and extraction where practicable. Wear RPE and hearing protection where required by task/RA. Stop if exposure becomes uncontrolled. B10 Slip/trip prevention Maintain housekeeping and safe underfoot conditions. Manage water use for dust suppression to avoid creating slip hazards. Keep access routes clear. B11 Record and re-brief Record heat/conditions controls in the Daily Brief/TBT and/or Heat Stress log (where used). If conditions change or controls drift → re-brief and adjust the plan. | 4 | 1 | 4 | Project Manager Site Supervisor Foreman QHSE Officer | R1 Heat stress protocol / midday break compliance record (where used) R2 Daily toolbox talk / briefing record (heat + exposure controls) R3 Daily residual risk checklist R4 Housekeeping inspection log R5 PPE issuance/inspection record (RPE/hearing where applicable) R6 Incident/near-miss reporting + CAPA register |
| 20 | Environmental & Waste Management (Spills, Dust, Noise) | H1 Spill/leak of fuel, oils, paints, chemicals during storage, use, or refuelling. H2 Spill entering drains/ground causing environmental contamination and non-compliance. H3 Uncontrolled dust generation (yard dust; cutting/grinding/blasting dust where applicable). H4 Exposure to airborne dust/fibres causing respiratory/eye irritation (task dependent). H5 Dry sweeping creating airborne dust and spreading contamination. H6 Poor housekeeping causing debris build-up, slip/trip hazards, and waste mixing. H7 Noise exposure from tools/plant/blasting affecting workers and nearby operations. H8 Incorrect waste segregation, open skips, or unsecured waste causing windblown debris. H9 Handling/disposal of hazardous waste without correct containment/labels/SDS. H10 Unapproved waste contractor or missing waste transfer notes/manifests/tipping tickets. H11 Runoff from wet suppression/washdown causing slips or entering storm drains. | 1 Environmental contamination (soil/drain/water), regulatory non-compliance and fines. 2 Respiratory/eye irritation or illness from dust/fumes exposure. 3 Hearing damage from noise exposure. 4 Slips/trips/falls and minor/major injuries from poor housekeeping. 5 Property damage and business interruption. | All yard personnel Operators and technicians handling substances Waste handlers / labourers Visitors (if present) | 5 | 4 | 20 | B1 Waste planning and segregation Segregate waste streams (general/recyclable/hazardous) using labelled bins/skips. Do not mix waste types. B2 Covered skips and secure storage Use covered skips/tarpaulins where required. Prevent windblown debris. Keep waste in designated areas only. B3 Hazardous substances control (where applicable) Store hazardous substances sealed and labelled with SDS available. Use secondary containment (bund/drip trays). Keep incompatible materials separated. B4 Spill prevention Refuel only in designated attended areas where practicable. Engines OFF. Use drip trays and suitable containers. Keep spill kit available and checked. B5 Spill response (STOP–CONTAIN–PROTECT DRAINS–NOTIFY–CLEAN–DISPOSE–RECORD) If spill occurs: stop source, contain, protect drains immediately, notify Supervisor/QHSE, clean up using approved materials, dispose via approved route, and record in spill/environmental log. B6 Drain protection / runoff control Do not allow washdown or wet suppression runoff to enter storm drains. Use bunding, wet-vac, absorbents, or designated containment method. Maintain safe walking surfaces. B7 Dust control hierarchy Use low-dust methods where practicable. Apply on-tool extraction/LEV where available; otherwise use controlled wet suppression or containment/sheeting as suitable without creating slip hazards. B8 Housekeeping (no dry sweeping) Clean using HEPA vacuum or damp methods. No dry sweeping for fine dust. “Clean as you go” and remove debris daily. B9 Noise controls Use quieter equipment where practicable. Limit exposure time, rotate tasks, and apply barriers/screens where practical. Hearing protection as required by site rules/RA. B10 Waste contractor and documentation Use approved/licensed waste contractor where required. Retain waste transfer notes/manifests/tipping tickets and file in the work pack. B11 Daily checks and records Supervisor to verify: skip covers, housekeeping standard, spill kit condition, and dust controls. Record checks in housekeeping/environmental log. B12 Stop-work triggers Stop work if spill cannot be contained, dust becomes uncontrolled, runoff threatens drains, or housekeeping degrades to unsafe/non-compliant condition. | 5 | 1 | 5 | Project Manager Site Supervisor Foreman QHSE Officer | R1 Environmental/spill response procedure + spill log R2 Housekeeping inspection log / HEPA vacuum log (where used) R3 Waste transfer notes / manifests / tipping tickets file R4 COSHH/SDS register (where applicable) R5 Toolbox talk / daily briefing record (dust/noise/spill controls) R6 Incident/near-miss reporting + CAPA register |
| 21 | Third Party / Yard Interface | H.1 Untrained/unauthorised operator, slinger/rigger, or banksman H.2 Pinch/crush points (outriggers, hook block, rigging, tail-lift, load placement) H.3 Sharps (banding, tubes, fittings, damaged pallets/edges) H.4 Uncontrolled operating area / pedestrian interface H.5 Poor ground bearing / underground services affecting stability H.6 Mechanical failure (HIAB/winch/hydraulics/lifting accessories) H.7 Load instability, falling load, or uncontrolled swing H.8 Slips on wet decks/ramps; tail-lift failure/entrapment (if used) H.9 Falls from trailer/flatbed during slinging/unhooking H.10 Adverse weather (wind, lightning, poor visibility) H.11 Vehicle movements/reversing during set-up and operations H.12 Fuel/oil leaks or spills causing environmental contamination | 1 Minor/major injuries (MSI, bruises, impact injuries) 2 Serious injury/fatality (crush, struck-by, falls) 3 Loss of limbs (amputation risk) 4 Property/asset damage and reputation impact | Operatives Drivers Visitors Pedestrians / third parties (where applicable) | 5 | 4 | 20 | B.1 Competence and authorisation Competent HIAB operator, slinger/rigger, and banksman only. Verify in-date licences/cards and lifting certification before operation. B.2 Lift planning / permit Lift Plan and Permit to Lift in place where required. Pre-lift briefing to confirm: roles, load weight and centre of gravity, crane configuration, load path, landing area, exclusion zone, wind/weather limits, emergency arrangements, and communication method. ### Set-up & stability B.3 Vehicle positioning and ground bearing Set up on firm, level ground. Assess bearing capacity and underground services where relevant. Apply parking brake and wheel chocks. B.4 Outriggers Fully deploy outriggers with suitable pads/mats. Confirm vehicle level within manufacturer tolerances before lifting. ### Exclusion zone & people control B.5 Segregation Establish barricaded exclusion zone covering swing radius and load path. Appoint a named Exclusion-Zone Controller. B.6 Prohibitions No persons under suspended loads. No man-riding. No conflicting instructions to operator. ### Lifting accessories & load control B.7 Rigging selection and inspection Select slings/shackles suitable for WLL/SWL including sling angles. Conduct pre-use checks. Defective gear → tag NOT FOR USE and quarantine. B.8 Load balance / test lift Confirm load security and centre of gravity. Conduct controlled test lift (100–200 mm) to confirm stability before full lift. B.9 Tag lines / pinch-point control Use tag lines for awkward loads where required. Keep hands clear of pinch/crush points. Never wrap tag lines around hands. ### Trailer / tail-lift / ramp controls (if applicable) B.10 Tail-lift and ramp checks Confirm SWL, locking pins and safety devices engaged. Maintain anti-slip surfaces. Only trained operator at controls. No riding the platform. B.11 Safe access to trailer Provide secured steps/ladder where required. Maintain three-point contact. Prevent falls from flatbed. ### Slips, housekeeping & manual handling B.12 Housekeeping / footing Keep decks/ramps clean and dry. Remove banding and debris promptly. B.13 Manual handling interface Use mechanical aids where practicable. Team-lift where required. Keep clear of line-of-fire. ### Weather controls B.14 Weather stop-work STOP for lightning. Stop/hold lifting operations when wind reaches site/Lift Plan limit (apply strictest limit). Secure load and boom. Resume only after Supervisor confirmation and re-brief. Record hold where required. ### Vehicle movement & traffic interface B.15 Traffic control Banksman to control reversing and tight manoeuvres. Maintain pedestrian segregation. Confirm mirrors/cameras/beacons/reverse alarms functional. ### Spill prevention & environmental protection B.16 Environmental controls Spill kit available on vehicle and work area where required. Any leak/spill → STOP, contain, protect drains, notify Supervisor/QHSE, clean and log as per Environmental Procedure. ### Supervision B.17 Supervisor oversight Supervisor to oversee set-up and lifting operations and STOP work if controls are not maintained. | 5 | 1 | 5 | Project Manager Site Supervisor Driver QHSE Officer | R.1 Lifting & Hoisting Procedure + Lift Plan / Permit to Lift R.2 HIAB/vehicle pre-use check record + maintenance certification R.3 Lifting gear register / inspection certificates R.4 Exclusion zone / banksman / radio briefing record R.5 Weather/wind hold & restart record R.6 Manual handling procedure R.7 Spill response procedure + spill log R.8 Incident/near-miss reporting + CAPA register |
| 22 | Night Work (If Applicable) | H1 Poor lighting/visibility at workfaces and along access/egress routes. H2 Worker fatigue / reduced alertness and decision-making. H3 Pedestrian–plant interface risk increases (blind spots, reversing, crossings). H4 Poor route definition/signage and changing yard layout causing confusion. H5 Slips/trips due to poor housekeeping, trailing cables, uneven ground. H6 Increased error risk during higher-risk tasks (WAH, lifting/loading/offloading, hot works). H7 Communication failure (unclear radio channel, delayed response). H8 Lone working during higher-risk tasks (WAH, plant movements, exclusion zones). | 1 Minor/major injuries (slips/trips, struck-by, manual handling injury). 2 Serious injury/fatality (WAH fall or vehicle/plant interface). 3 Property/asset damage and operational disruption. 4 Non-compliance and reputation impact. | All personnel on night shift Operators and banksman Supervisors/foremen Visitors (if present) | 4 | 3 | 12 | B1 Night work authorisation and planning Night work only with adequate supervision, lighting, segregation, and communication arrangements in place. B2 Fit-for-duty / fatigue checks Supervisor to check fitness at start and mid-shift. If fatigue indicators seen → reassign tasks, increase breaks, or stand down. Record actions. B3 Lighting provision (HOLD POINT) Provide adequate task and route lighting for workfaces, walkways, crossings, loading/offloading areas, and signage/barriers. If lighting is inadequate → STOP and rectify before continuing. B4 High-visibility and identification Reflective high-visibility PPE mandatory. Banksman/marshal clearly identifiable. B5 Traffic management at night Apply yard traffic rules: reduced speed where required, clear route definition, and banksman control for reversing/tight manoeuvres or where visibility is restricted. B6 Segregation and access control Maintain physical barriers and clear signage around work zones and exclusion zones. Increase segregation where visibility is reduced (additional barriers/marshalled crossings). B7 Housekeeping and route condition Keep routes clear, stable and free of trailing cables, debris and spill hazards. Repair/cover uneven ground where practicable. If safe access/egress cannot be maintained → STOP. B8 Work-at-height and lifting restrictions (where applicable) For WAH and lifting/loading: confirm exclusion zones, rescue readiness (where fall arrest used), and banksman control before starting. Stop if controls cannot be maintained. B9 Communications and emergency readiness Confirm radio channel/backup method, emergency numbers, muster point, emergency access routes, and First Aider coverage before starting night shift. B10 No lone working for high-risk tasks No lone working for WAH, plant movements, loading/offloading, hot works, or any task requiring an exclusion zone. B11 Night residual risk check Supervisor completes the Daily Residual Risk Checklist for the night shift and updates toolbox talk if risk increases or controls drift. | 4 | 1 | 4 | Project Manager Site Supervisor Foreman QHSE Officer | R1 Night work supervision record (where used) R2 Daily residual risk checklist (night shift) R3 Toolbox talk / daily briefing record (night controls) R4 Site traffic rules / TMP + banksman record (where used) R5 Lighting inspection/check record (where used) R6 Emergency response plan / incident reporting + CAPA register |